### CENWP-OP-B                                                                                                                          4 June 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 22 of 2007, which covers the period from 27 May through 02 June 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 221.4 to 269.5 kcfs.  Daily average project tailwater elevation ranged from 18.2’ to 20.3’ msl.  Daily average powerhouse forebay elevation ranged from 73.1’ to 74.5’ msl.  Daily average river temperature ranged between 59F and 61F.  Secchi disk measurements averaged 4’ to 5.5’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 92.2 to 98.3 kcfs.

  - Unit Operation. Powerhouse 2 is the priority powerhouse.

    - Unit 8 remains out of service for turbine rehab.

    - Unit 10 was out of service from 0700 until 1354 on 31 May for brush inspection.

    - Unit 15 returned to service at 1525 on 31 May.

    - Fish Units 1 and 2 were run at reduced load from 1408 until 1455 on 27 May to test entrance differential for a lamprey study.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  No AWS valves were closed this week:

  - STS/VBS Inspections.  No STS/VBS inspections this week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 |  |  |  |
| 14 |  |  |  |
| 15 |  |  |  |
| 16 |  |  |  |
| 17 |  |  |  |
| 18 |  |  |  |

  - Dewatering and Fish Salvage.  No dewatering or salvage activities this week.

- RESEARCH

  - Battelle spillway survival.  Releasing juvenile yearling chinook in the spillway forebay for a spillway survival evaluation.

  - Fish Field Unit.  Sea lion observations concluded on 26 May.

  - NOAA.  Salt water rearing at the Smolt Monitoring Facility.

  - NOAA.  B2CC detection evaluation underway.

  - NOAA.  Tagging juvenile yearling chinook at the Smolt Monitoring Facility.

  - NOAA Lamprey.  Construction of the lamprey ramp in the WA shore AWS began in mid January.

  - PSMFC Smolt Monitoring.  No fallback observed crossing the separator bars.   A total of 298 clipped yearling Chinook were held for NOAA and Battelle research.

  - University of Idaho.  Spring chinook evaluations continue.

  - USACE TDG.  New TDG monitors were installed in the spillway tailrace.

  - USDA Wildlife Services.  Hazing sea lions from the powerhouses concluded on 26 May.

  - USGS.  Lamprey continue to be held in the FERL for tagging evaluation.

  - WDFW Fish Counting.  Visual counting continues Bradford Island and Washington Shore

  - WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats concluded on 26 May

- FISHWAYS

  - Project biologists inspected daily from 27 May through 2 June.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |  |
| 6/2 | B-branch | Staff gauge <1.3 | Unknown | Operator adjusted |  |  |
| 5/27, 5/30-6/1 | B-branch | FG3-23 open incorrectly for tailwater | Unknown | Operator adjusted |  |  |
| Once | Calibration | Check not done | Short-handed | New bio hired. She started on 14 May. |  |  |
| CASCADES ISLAND |  |  |  |  |  |  |
| All week | FG6-10 | Closed | Shaft and motor have parted ways. | TR submitted. See 4.2.6 |  | 07-37350 |
| All week | FG6-14 | ½ way open | Electricians need to do some rewiring. | Electricians and riggers working out scheduling. |  |  |
| WASHINGTON SHORE |  |  |  |  |  |  |
| All week | 11A-S | Closed | Broken shaft | Mech. working on it. |  | 07-40917 |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

    - FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.

    - FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.

    - FG6-10 closed.  Only out of criteria when tailwater is above 19.0’ msl.  Gear box work will begin the week of 11 June.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures.

| Date | WS average | BI average |
| --- | --- | --- |
| 5/27 | 59.0 | 58.8 |
| 5/28 | 59.0 | 58.8 |
| 5/29 | 59.1 | 58.9 |
| 5/30 | 59.7 | 59.6 |
| 5/31 | 60.7 | 60.4 |
| 6/1 | 61.4 | 61.3 |
| 6/2 | 61.8 | 61.7 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration will be checked on weekends.

    - B-Branch was put into manual on 2 June.  Cascades Island remains in auto.

    - FV3-7 is in manual until the A-Branch diffusers are repaired and operating correctly.

    - SLEDS and floating orifice gate stab plates were removed from Powerhouse 2 on 30 and 31 May respectively.

    - As of 10 May, the fish ladders are in shad passage mode.

  - Juvenile Fishways.

    - PH1 chaingates 1A and 1B remain closed at this time.

    - DSM1 continues to remain OOS.

    - DSM2 orifice lights are scheduled to be switched back to LEDs on 4 June.

    - Chaingates are still set at 70.

  - Lamprey Fishways.

    - The Bradford Island lamprey ramp remains in service.

    - The north downstream entrance (NDE) was closed from 1510 until 1535 on 29 May for installation of the lamprey ramp

- CONSTRUCTION

  - B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.

  - PH1 STS/VBS removal and TIES removal.  Six TIES and 23PH1 STSs/VBSs will be scrapped.

  - PH2 VBS mods.  Units 14 and 18 are scheduled for winter 07/08.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 06 April requests juvenile spill start at 0001 on 10 April.  Bonneville will spill 100K 24 hours a day.  The spill table may be found in the 1007 FPP page Bon-51, Table Bon-15.  At no time will the project voluntarily spill above the 120% gas cap.

  - A teletype sent on 21 May requests that the Washington shore adult ladders be reduced to 0.5 feet head from 2200 until 0400 hours for a lamprey test running from 2 June until 1 October.

  - A teletype sent on 25 May requests that all Lower Snake and Columbia River FCRPS projects operate all turbine units within the 1% best efficiency standard during the best efficiency operating period (April 1 through October 31).  Operate outside of the 1% best efficiency standard only under the declaration of a system emergency.

JAMES R. MAHAR, P.E.

Operations Manager