### CENWP-OP-B                                                                                                                        10 September 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 36 of 2007, which covers the period from 02 September through 08 September 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 85.5 to 116.8 kcfs.  Daily average project tailwater elevation ranged from 8.0’ to 10.7’ msl.  Daily average powerhouse forebay elevation ranged from 76.0’ to 76.3’ msl.  Daily average river temperature ranged from 70 to 71°F.  Secchi disk measurements averaged 6.0’ to 7.0’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill was 1.5 kcfs.  Bulk spill for fish passage concluded on 01 Sept.

  - Unit Operation. Powerhouse 2 is the priority powerhouse.

    - Unit 3 has remained out of service since 27 August for protective relaying and main unit packing.

    - Unit 7 has remained out of service for turbine rehab since 04 June 2007.

    - Unit 8 has remained out of service for turbine rehab since 12 September 2006.

    - Unit 14 has remained out of service since 13 August for rewedge.

    - Unit 16 has remained out of service since 20 August for exciter replacement.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - F1 and F2 had reduced loading for lamprey passage studies from:

2200 on 01 September until 0400 on 02 September

2200 on 03 September until 0400 on 04 September

2200 on 04 September until 0400 on 05 September

0010 on 08 September until 0400 on 08 September

2200 on 08 September until 0400 on 09 September

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  There were no AWS closures this weeks.

  - STS/VBS Inspections.  No STS/VBS inspections occurred this week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA |  | NA |

  - Dewatering and Fish Salvage.  .  No dewatering and fish salvage activities occurred this week.

- RESEARCH

  - CRITFC.  Due to temperatures exceeding 70F, sampling is restricted to once a week for steelhead only.

  - CTUIR- lamprey.  Lamprey collection from NOAA Fisheries continues.

  - NOAA-Gilbreath.  Salt water rearing at the Smolt Monitoring Facility.

  - NOAA Lamprey.  Lamprey sampling continues.

  - PSMFC Smolt Monitoring.  No fallbacks observed crossing the separator bars.  56 juvenile shad were collected for U of I.  Sort by code (SbyC) for NOAA continues.  Modified sampling due to temperatures exceeding 70F.

  - University of Idaho- lamprey.  Block study for lamprey entrance evaluations at WS continues.

  - USDA Wildlife Services.  Avian hazing concluded.

  - WDFW Fish Counting.  Daily visual counting continues at Bradford Island and Washington Shore.  Tule counting continues.

- FISHWAYS

  - Project biologists inspected daily from 02 September through 08 September.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 9/2, 9/7 | FG3-27 | Incorrect position for TW | System in manual | Operator adjusted. |  |
| 9/2, 9/3 | B-branch entrance | Entrance differential high | System in manual | Operator adjusted. |  |
| 9/4, 9/8 | B-branch Entrance | Entrance differential low | System in manual | Operator adjusted. |  |
| 9/7 | FG3-26 | Incorrect position for TW | System in manual | Operator adjusted. |  |
| CASCADES ISLAND |  |  |  |  |  |
| 9/2, 9/7 | CI entrance | Entrance differential high | System in manual | Operator adjusted. |  |
| 9/4, 9/8 | CI entrance | Entrance differential low | System in manual | Operator adjusted. |  |
| 9/8 | CI entrance | North entrance gate closed | System in manual | Operator adjusted. |  |
| All week | FG6-14 | ½ way open | Electricians need to do some rewiring. | Electricians and riggers working out scheduling. |  |
| WASHINGTON SHORE |  |  |  |  |  |
| 9/8 | PH2 PCC diffusers | B4 open | Manually adjusted. | Operator opened diffusers. |  |
| All week | DSM2 Airburst | Screens 3, 6 and 9 not consistent. | Bad controls. | Will replace during winter maintenance. |  |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

    - FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.

    - FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 07/08.  Only out of criteria when tailwater is above 19.0’ msl.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures.

| Date | WS average | BI average | AFF average | JMF average |
| --- | --- | --- | --- | --- |
| 9/2 | 71.4 | 71.1 | 71.4 | 70.4 |
| 9/3 | 71.6 | 71.3 | 71.7 | 70.5 |
| 9/4 | 71.6 | 71.2 | 71.5 | 70.5 |
| 9/5 | 70.9 | 70.7 | 70.9 | 69.8 |
| 9/6 | 70.5 | 70.3 | 70.6 | 69.5 |
| 9/7 | 70.2 | 69.8 | 70.2 | 69.1 |
| 9/8 | 69.7 | 69.4 | 69.7 | 68.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 07 September.

    - B-Branch and Cascades Island are in manual.

    - FV3-7 is in manual until the A-Branch diffusers are repaired and operating correctly.

  - Juvenile Fishways.

    - DSM1 remains OOS.

    - B2CC was closed on 31 August.

    - The SMF outfall avian cannons were shut off on 01 September.

  - Lamprey Fishways.

    - All lamprey passage structures remain in service.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.

  - Bradford Island fish exit dredging.  Sediment sampling and sounding occurred on 31 July.

  - Bradford Island clean-up.  Removal of contaminated sediment scheduled to occur in October 2008.  Pre-removal sampling will occur in September.

  - PH2 VBS mods.  Units 14 and 18 are scheduled for winter 07/08.  U17 VBS monitoring equipment will be installed at this time as well.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 21 May requests that the Washington shore adult ladders be reduced to 0.5 feet head from 2200 until 0400 hours for a lamprey test running from 2 June until 1 October.

  - A teletype sent on 07 June requests that all Lower Snake and Columbia River FCRPS projects operate all turbine units within the 1% best efficiency standard during the best efficiency operating period (April 1 through October 31).  Operation outside of 1% is allowed for maintenance, start up/shut down and other instances as outlined in the Fish Passage Plan.  Operation outside of 1% is NOT allowed for any other reason unless a power emergency has been declared.

  - A teletype sent on 22 August requests that the Bonneville forebay have a hard constraint of 75.0’ to 76.5’ msl and a soft constraint of 75.5’ to 76.5’ msl from 0600 on 28 August until 1800 on 31 August.  This is to reduce the wear and tear on tribal gill net equipment.

  - A teletype sent on 30 August extends the treaty fishing restrictions (see 7.4) from 0600 on 04 September until 1800 on 08 September.

  - A teletype sent on 22 August requests low (≤150 kcfs) and constant outflows from TDA and BON from 0700 until 1130 on 3 September.  This is to reduce river currents during the Columbia River cross channel swim at Hood River.

  - A teletype sent on 6 September extends the treaty fishing restrictions (see 7.4) from 0600 on 11 September until 1800 on 14 September.

JAMES R. MAHAR, P.E.

Operations Manager