### CENWP-OP-B                                                                                                                         October 9, 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 40 of 2007, which covers the period from 30 September through 06 October 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 72.0 to 108.0 kcfs.  Daily average project tailwater elevation ranged from 7.6’ to 10.0’ msl.  Daily average powerhouse forebay elevation ranged from 74.0’ to 76.3’ msl.  Daily average river temperature ranged from 62 to 64°F.  Secchi disk measurements averaged 7.0’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 1.2 to 1.6 kcfs.

  - Unit Operation. Powerhouse 2 is the priority powerhouse.

    - Unit 3 returned to service at 1223 on 01 October.  It had been OOS since 24 September for the bank 3/4 outage.

    - Unit 4 returned to service at 1223 on 01 October.  It had been OOS since 24 September for the bank 3/4 outage.

    - Unit 7 has remained out of service for turbine rehab since 04 June 2007.

    - Unit 8 has remained out of service for turbine rehab since 12 September 2006.

    - Unit 11 was out of service from 1457 until 1728 on 03 October due to a line trip.

    - Unit 12 has remained out of service since 17 September for exciter replacement.

    - Unit 13 was out of service from 0116 until 1049 on 01 October for oil piping repair work.  U13 was out of service from 1457 until 1728 on 03 October due to a line trip.

    - Unit 14 has remained out of service since 13 August for re-wedge and FGE modifications.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - F1 and F2 had reduced loading for lamprey passage studies from:

2200 on 30 September until 0420 on 01 October

    - F1 was out of service from 1515 until 1726 on 03 October due to a loss of governor pressure.

    - F2 was out of service from 1515 until 1544 on 03 October due to a loss of governor pressure.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  No AWS closures this week.

  - STS/VBS Inspections.  No STS/VBS inspections occurred this week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA | NA | NA |

  - Dewatering and Fish Salvage.  No dewatering and fish salvage activities occurred this week.

- RESEARCH

  - CRITFC.  Normal sampling operations continue.

  - NOAA-Gilbreath.  Salt water rearing at the Smolt Monitoring Facility continues.

  - PSMFC Smolt Monitoring.  Two steelhead and one chinook fell back over the separator bars.

  - University of Idaho- lamprey.  Block study for lamprey entrance evaluations concluded on 01 October.

  - USGS shad.  No shad collected this week.

  - WDFW Fish Counting.  Daily visual counting continues at Bradford Island and Washington Shore.  Tule counting concluded on 30 September.  Video monitoring at Cascades Island began on 1 October.

- FISHWAYS

  - Project biologists inspected daily from 30 September through 06 October.  NOAA inspected on 05 October.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 9/30-10/2, 10/5 | A-branch differential | Reading <1.0> | System in manual | Operator adjusted. |  |
| CASCADES ISLAND |  |  |  |  |  |
| All week | FG6-14 | ½ way open | Electricians need to do some rewiring. | Winter maintenance item. |  |
| WASHINGTON SHORE |  |  |  |  |  |
| All week | DSM2 Airburst | Screens 3, 6 and 9 not consistent. | Bad controls. | Will replace during winter maintenance. |  |
| All week | DSM2 PLC | Not functioning at DSM2 |  | Repairs won’t be made until Dec/Jan |  |
| 10/3-5 | Weirs 37 &38 | Reading >1.1’ | UMT OOS | AFF valve 2 adjusted |  |
| 10/3-5 | Weir 67 | Reading >1.0’ | UMT OOS | No action taken |  |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

    - FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.

    - FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 07/08.  Only out of criteria when tailwater is above 19.0’ msl.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures

| Date | WS average | BI average |
| --- | --- | --- |
| 9/30 | 64.3 | 64.0 |
| 10/1 | 64.0 | 63.6 |
| 10/2 | 63.9 | 63.2 |
| 10/3 | 63.7 | 63.2 |
| 10/4 | 63.2 | 62.7 |
| 10/5 | 62.7 | 62.5 |
| 10/6 | 62.3 | 61.9 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 04 October.

    - B-Branch and Cascades Island are in manual.

    - FV3-7 is in manual until the A-Branch diffusers are repaired and operating correctly.

    - DSM1 continues to run south.  All available orifices are open.

    - The UMT was closed at 0812 on 01 October.  CI exit was opened and the picket leads raised at 0740 on 01 October.

  - Juvenile Fishways.

    - DSM1 is watered up and running south.  See 4.5.4

    - DSM2 PLC is OOS.  Estimated time of repair is December or January.

    - A loss of the fish units on 03 October resulted in the SMF going to bypass.from 1515 until 1544.

    - SMF hydrocannons resumed operation on 04 October.  This is to help deter gulls from perching on the outfalls.

  - Lamprey Fishways.

    - All lamprey passage structures remain in service.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Plans to install three automated gates.  Optional items include sloping the floor and removing the DSM wall.

  - Bradford Island clean-up.  Sediment sampling occurred on 01 October.  Removal of contaminated sediment currently underway.

  - PH2 VBS mods.  Unit 14 is currently OOS.  U18 will follow.  U17 VBS monitoring equipment will be installed at this time as well.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 21 May requests that the Washington shore adult ladders be reduced to 0.5 feet head from 2200 until 0400 hours for a lamprey test running from 2 June until 1 October.

  - A teletype sent on 07 June requests that all Lower Snake and Columbia River FCRPS projects operate all turbine units within the 1% best efficiency standard during the best efficiency operating period (April 1 through October 31).  Operation outside of 1% is allowed for maintenance, start up/shut down and other instances as outlined in the Fish Passage Plan.  Operation outside of 1% is NOT allowed for any other reason unless a power emergency has been declared.

  - A teletype sent on 28 September extends treaty fishing restrictions.  It requests that the Bonneville forebay have a hard constraint of 75.0’ to 76.5’ msl and a soft constraint of 75.5’ to 76.5’ msl from 0600 on 03 October until 1800 on 06 October.  This is to reduce the wear and tear on tribal gill net equipment.

JAMES R. MAHAR, P.E.

Operations Manager