### CENWP-OP-B                                                                                                                         December 5, 2007

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 48 of 2007, which covers the period from 25 November through 1 December 2007.

- OPERATION SUMMARY

  - Daily average river flows ranged from 116.6 to 131.6 kcfs.  Daily average project tailwater ranged from 11.5’ to 12.2’ msl.  Daily average powerhouse forebay elevation ranged from 73.6’ to 74.9’ msl.  Daily average river temperature ranged from 47° to 48°F.  Secchi disk measurements ranged from 6.0’ to 7.0.’

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill was 1.1 kcfs.

  - Unit Operation.   Powerhouse 2 is the priority powerhouse.

    - Unit 1 has remained out of service since 23 October due to governor trouble.

    - Unit 3 was out of service from 2204 on 22 November until 0937 on 26 November due to a failed STS motor.

    - Unit 7 has remained out of service since 04 June for turbine rehab.

    - Unit 8 has remained out of service since 12 September 2006 for turbine rehab.

    - Unit 11 was out of service from 0205 to 0944 on 30 November due failed STS ground.

    - Unit 14 has remained out of service since 13 August for re-wedge and FGE modifications.

    - Unit 15 was forced out of service at 2317 on 25 November due to exciter and pole failure, followed by a head gate hydraulic line rupture.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting period:

    - FV1-1 was closed from 2320 on 24 November until 0420 on 25 November to float trash and from 1547 to 1635 on 26 November for cleaning.

    - FV 3-9 was closed from 1007 to 1040 on 26 November for cleaning.

  - STS/VBS Inspections.  No STS/VBS inspections occurred this week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 3 | 11/27 | 487 |  |
| 4 | 11/27 | 651 |  |
| 11 | 11/28 | 476 |  |
| 12 | 11/28 | 680 |  |
| 13 | 11/28 | 651 |  |
| 14 | NA | NA | OOS |
| 15 | NA | NA | OOS |
| 16 | 11/28 | 615 |  |
| 17 | 11/28 | 671 |  |
| 18 | 11/28 | 699 | To be removed 12/3 for FGE mods |

  - Dewatering and Fish Salvage.  No dewatering or salvage activities took place this reporting week.

- RESEARCH

  - NOAA-Gilbreath.  Salt water rearing at the Smolt Monitoring Facility continues.

  - WDFW Fish Counting.  Video counting continues at Bradford Island, Cascades Island and WA Shore.

- FISHWAYS

  - Project biologists inspected daily from 25 through 30 November.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 11/25 | B-branch weir | <1.0’ | Unknown | Operator adjusted |  |
| 11/27 | B-branch diffusers | FG 3-26 open | System in manual | Operator adjusted |  |
| All week | B-branch diffusers | FG 3-27 incorrect position for TW | A lot of water exiting FG3-28 | See 4.2.6 or 4.5.3 |  |
| CASCADES ISLAND |  |  |  |  |  |
| 11/28 | UMT weir | Reading <1.0’ | Unknown | Operator adjusted. |  |
| All week | FG6-14 | ½ way open | Electricians need to do some rewiring. | Winter maintenance item. |  |
| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| WASHINGTON SHORE |  |  |  |  |  |
| 11/27, 29 | Weir 67 | Reading > 1.0’ | UMT closed. | No action taken. |  |
| 11/29 | PH2 monoliths | Differential <1.0’ | Raking F1 and F2 trashracks | Differential >1.0’ by 1450 on 29 Nov |  |
| 11/30 | PH2 monoliths | Differential <1.0’ | F1/F2 at reduced load for stab plate installation | Differential >1.0’ by 1630 on 30 Nov |  |
| All week | DSM2 Airburst | Screens 3, 6 and 9 not consistent. | Bad controls. | Will replace during winter maintenance. |  |

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter often reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.

    - FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

    - FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.

    - FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.

    - FG3-27 is open regardless of TW.  This is to reduce water and pressure from FG3-28.  The excess water may be due to FV4-3 not operating as it should.  Maintenance will repair FV4-3 this winter.

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 07/08.  Only out of criteria when tailwater is above 19.0’ msl.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

    - The Bradford Island temperature sensor is broken and parts are on order.  Temperatures generally track those of WS.

Table 3.  Bonneville Fishway Temperatures

| Date | WS average | BI average |
| --- | --- | --- |
| 11/25 | 49.1 | See 4.3.1 |
| 11/26 | 48.1 |  |
| 11/27 | 48.8 |  |
| 11/28 | 48.8 |  |
| 11/29 | 48.5 |  |
| 11/30 | 48.4 |  |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 30 November.

    - B-Branch and Cascades Island are in manual.

    - FG3-27 is open regardless of TW.  This is to reduce water and pressure from FG3-28.  The excess water may be due to FV4-3 not operating as it should.  Maintenance will repair FV4-3 this winter.

    - FV3-7 is in manual until the A-Branch diffusers are repaired and operating correctly.

    - DSM1 continues to run south.  All available orifices are open.

    - The UMT remains closed.  Cascades Island exit remains open.

    - WS Monolith gates remain in due to TW elevation sensor issues.

    - Installation of the PH2 FOG stab plates commenced on 30 November, and is scheduled to be completed on 3 December in preparation for Fish Unit 2 outage.

  - Juvenile Fishways.

    - DSM2 PLC is OOS.  Estimated time of repair is December or January.

    - SMF hydrocannons will be winterized next week.

    - The SMF remains in bypass.

  - Lamprey Fishways.  Winterized.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Three automated gates will be installed.  Optional items include sloping the floor and removing the DSM wall.

  - PH2 VBS mods.  Unit 14 is currently OOS.  U18 will follow.  U17 VBS monitoring equipment will be installed at this time as well.

  - TIE crane.  The TIE crane boom repair is on hold until funding is secured.

  - TIE scrapping.  Six TIEs are currently being scrapped and removed from the project.

- HAZMAT, SPILLS AND CLEANUP.

  - On 26 November at 1410 hours, while lowering Unit 15 headgates for clearance activities (see 1.3.7), an operator reported a possible headgate hydraulic line rupture.  At 1430, there was a report of a visible indication from the 15A headgate slot.  At 1440, all U15 orifices were closed.  There was no indication of oil in the PH2 tailrace or at the SMF.  Once the oil is removed from headgate slot 15A and a refurbished gate is installed, the exciter investigation will continue.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 7 November requests a tailwater operation for chum spawning, effective 9 November until further notice.  The Project is asked to maintain a hard constraint TW elevation of 11.3’ to 11.7’ 24 hours a day, with a target of 11.5’.  The official Project TW gauge is located at river mile 144.5, on the Oregon side.  This request was coordinated with TMT.

  - A teletype sent on 29 November modifies the chum spawning teletype (see 7.1) by changing the following language in the event flows are too high to maintain desired TW elevations:

    - Hard constraint minimum TW elevation of 11.3’ during all hours.

    - Between 0600 and 1800, hard constraint TW elevation 11.3’ to 11.7’, with a target of 11.5’.

    - Between 1800 and 0600, soft constraint TW elevation 11.3’ to 11.7’, with a target of 11.5’.

    - If there is a need to pass water in excess of the tailwater constraint, the project is authorized between the hours of 1800 and 0600 to raise the tailwater up to 13.0’ for up to 8 hours.

    - If the above step is not enough to pass excess water, the project is authorized between the hours of 2200-0400 to raise the tailwater up to 15.0’ for 4 hours.

  - The draft 2008 FPP will be out by early December.

  - The winter maintenance schedule is available from Project Fisheries.

    - Bradford Island is scheduled to go to orifice flow on 14 December.  It will be dewatered on 17 December.

    - DSM2 and the two mile pipe will be dewatered on 19 December.

    - DSM1 will be dewatered on 20 December.

JAMES R. MAHAR, P.E.

Operations Manager