### CENWP-OP-B                                                                                                                         February 6, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 05 of 2008, which covers the period from 27 January through 02 February 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 111.4 to 163.6 kcfs.  Daily average project tailwater ranged from 11.6’ to 14.9’ msl.  Daily average powerhouse forebay elevation ranged from 73.3’ to 75.0’ msl.  Daily average river temperature ranged from 36° to 37°F.  Secchi disk measurements were 7.0.’

  - Spill Bay 1, which supplies attraction flow for Cascades Island, is included in the spill pattern.  Daily average spill were 0.6 kcfs.

  - Unit Operation.   Powerhouse 2 is the priority powerhouse.

    - Unit 1 has remained out of service since 23 October due to governor trouble.

    - Unit 2 went out of service on 28 January due to Unit 1 special field test drive generator.

    - Unit 7 has remained out of service since 04 June for turbine rehab.

    - Unit 8 has remained out of service since 12 September 2006 for turbine rehab.

    - Unit 15 has remained out of service since 25 November due to exciter and pole failure, followed by a head gate hydraulic line rupture.

    - Unit 18 has remained out of service since 03 December for FGE modifications and 4 year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - Unit F1 has remained out of service since 13 January for exciter replacement.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting period:

    - FV 1-1, 3-7, and 3-9 remain closed for winter maintenance in A-branch.

    - FV 4-3 and 4-4 remain closed for winter maintenance in B-branch.

  - STS/VBS Inspections.  No STS/VBS inspections occurred this week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 3-4, 11-18 | NA | NA | All screens were pulled for winter maintenance. |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - WDFW Fish Counting.  Video counting continues at Cascades Island and WA Shore.

  - Fish Field Unit.  Pinniped observations continue.

  - WDFW/ODFW Sea lion hazing.  Hazing from boats continues.

- FISHWAYS

  - Project biologists inspected on 29-31 January.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES- OOS for the winter season |  |  |  |  |  |
| CASCADES ISLAND |  |  |  |  |  |
| Once | ##### Calibration | ##### Check not done | ##### Problems with PLC | ##### TR written | ##### 08-26031 |
| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| WASHINGTON SHORE |  |  |  |  |  |
| 1/31 | Weir 37 | Differential >1.0’ | Bleed off valve closed due to winterization | No action taken |  |
| 1/31 | Weir 67 | Differential >1.0’ | UMT closed | No action taken. |  |
| 1/23, 24 | PH2 monoliths | Differential <1.0’ | F1 OOS | Operators adjusted. |  |
| All week | WS velocity | Meter reading <1.0 | Unknown- may be related to the low differential. | Fisheries will monitor. |  |

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 07/08.  It is only out of criteria when tailwater is above 19.0’ msl.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 24 January.

    - BI was dewatered on 17 December.

    - Cascades Island fishway PLC is not functioning correctly.  A TR had been written (#08-26031).

    - The UMT remains closed.  Cascades Island exit remains open.

    - PH2 FOG stab plates are installed as of 03 December due to Fish Unit outages.

    - The AFF was dewatered on 31 December.

    - SLEDS were installed at all PH2 fishway entrances on 28 January.

    - SLEDS were installed at Cascades Island on 31 January.

  - Juvenile Fishways.

    - DSM 1 and 2 remain dewatered.

  - Lamprey Fishways.  Winterized.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Three automated gates will be installed.  Optional items include sloping the floor and removing the DSM wall.

  - PH2 VBS mods.  Unit 18 is currently OOS.  U17 VBS monitoring equipment will be installed during this winter maintenance period.

  - TIE crane.  The TIE crane boom repair is on hold until funding is secured.

  - BGS.  Installation of the BGS continues.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 20 December replaces the tail water operation for chum spawning teletype on 12 December.  Effective 0600 hours on Monday, December 24, until further notice, operate Bonneville tail water as follows – minimum 11.5 ft all hours.   The minimum 11.5 ft is for chum incubation.

  - A teletype sent on 02 January requests a change in PH1 unit priority.  Unit 2 is to be operated on a last on, first off basis for approximately one month, due to an undiagnosed problem with the unit.

  - A teletype sent on 25 January requests the Bonneville forebay be maintained at a hard constraint of 71.5 to 72.5 feet, and a soft constraint of 71.5 to 72.0 feet from 1000 to 1300 on 30 January.  This is necessary for ranking politicians to observe the Rock Creek slide in Stevenson, WA.

  - A teletype sent on 28 January replaces the teletype sent on 23 January.  The dates of the spillway machinery test will be shifted to 04 February through 08 February, and will occur during the hours of 0700 to 1700.  This is necessary to evaluate the condition of the spillway hoists.  Each spillway gate will be opened approximately two feet for about 15 minutes each, producing a flow of about 4.4 kcfs.

  - A teletype sent on 29 January cancels the teletype sent on 02 January (see 7.2).  The 29 January teletype requests that the PH1 unit priority return to normal operation as defined in Table BON-5 in the 2007 Fish Passage plan.

  - A teletype sent on 01 February requests the following special operations while the behavioral guidance structure is being installed in the PH2 forebay:

    - This operation is to occur on Thursdays and Fridays from 1100-1500 each day, February 7 & 8, 14 & 15, 21 & 22

    - The project will operate only two main units plus the fish units at PH2, with the rest of the outflow going through PH1.  The location of the two operating units will be determined by the location of the dive.

    - PH2 will return to normal operations over the next two hours, once the dive is completed.  BPA will attempt to keep low flows even outside the four hour operational period.

    - The minimum tailwater of 11.5’ will be maintained (see 7.1).

    - The project is authorized outside the standard unit operating priority as described in the Fish Passage Plan during this operation, and when needed to setup and return to normal operation.

    - The dive coordinators for Advanced American diving will contact the project, who will contact BPA, if the dives are completed earlier than specified.

JAMES R. MAHAR, P.E.

Operations Manager