### CENWP-OP-B                                                                                                                         March 03, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 09 of 2008, which covers the period from 24 February through 01 March 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 119.1 to 147.3 kcfs.  Daily average project tailwater ranged from 11.6’ to 13.3’ msl.  Daily average powerhouse forebay elevation ranged from 73.6’ to 74.5’ msl.  Daily average river temperature ranged from 40° to 41°F.  Secchi disk measurements ranged from 5.0 to 7.0.’

  - Spill Bay 1, which supplies attraction flow for Cascades Island, is included in the spill pattern.  Daily average spill ranged from 0.6 to 1.0 kcfs.

  - Unit Operation.   Powerhouse 2 is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June for turbine rehab.

    - Unit 8 has remained out of service since 12 September 2006 for turbine rehab.

    - Unit 11 was out of service from 1100 until 1600 on 25 February; from 0542 until 1705 on 27 February; from 0541 until 1842 on 28 Feb; from 0518 until 1800 on 29 Feb to facilitate the BGS dive.  U11 was out of service from 1800 on 29 February until 0611 on 1 March for debris removal.

    - Unit 12 was out of service from 0542 until 1705 on 27 February to facilitate the BGS dive; from 1227 until 1427 on 28 February due to U18 exciter testing; from 1800 on 29 February until 0557 on 1 March for debris removal.

    - Unit 13 was out of service from 1100 until 1600 on 25 February; from 0548 until 1756 on 29 February; from 0710 until 1852 on 1 March to facilitate the BGS dive.

    - Unit 14 was out of service from 0545 until 1705 on 27 February; from 0518 until 1756 on 29 February; from 0607 until 1903 on 1 March to facilitate the BGS dive.  U14 was out of service from 1836 on 27 February until 1901 on 28 February due to a 14A STS ground.

    - Unit 15 has remained out of service since 25 November due to exciter and pole failure, followed by a head gate hydraulic line rupture.

    - Unit 16 was out of service from 1100 until 1600 on 25 February; from 0528 until 1901 on 28 February; from 0520 until 1902 on 1 March to facilitate the BGS dive.

    - Unit 17 has remained out of service since 23 February for FGE modifications.

    - Unit 18 returned to service at 1427 on 28 February.  Unit 18 was out of service from 0548 until 1756 on 29 February; from 0520 until 1906 on 1 March to facilitate the BGS dive.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - F1 was out of service from 0512 until 1936 to facilitate the installation of the hydrophones.

    - F2 was out of service from 0512 until 1936 to facilitate the installation of the hydrophones.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  The following AWS valves were closed this reporting period:

    - FV 1-1, 3-7, and 3-9 returned to service at 1500 on 29 February.

    - FV 4-3 and 4-4 returned to service at 1500 on 29 February.

  - STS/VBS Inspections.  STS/VBS installation occurred this week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 2/25 |  | Screens installed for the start of fish passage season. |
| 12 | 2/25 |  | Screens installed for the start of fish passage season. |
| 13 | 2/25 |  | Screens installed for the start of fish passage season. |
| 14 | 2/25 |  | Screens installed for the start of fish passage season. |
| 15 | 2/25 |  | Screens installed for the start of fish passage season. |
| 16 | 2/25 |  | Screens installed for the start of fish passage season. |
| 17 |  |  | OOS for FGE mods |
| 18 | 2/25 |  | Screens installed for the start of fish passage season. |

  - Dewatering and Fish Salvage.  No dewatering or salvage activities this week.

- RESEARCH

  - WDFW Fish Counting.  Video counting continues at the WA Shore.  Video counting concluded at Cascades Island and began at Bradford Island on 01 March.

  - Fish Field Unit.  Pinniped observations continue.  The pinniped trap is now in place.

  - Fish Field Unit. Sea lion scat collections continue.

  - WDFW/ODFW Sea lion hazing.  Hazing from boats continues.

- FISHWAYS

  - Project biologists inspected on 25, 26, 28, February and 1 March.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 3/1 | Spillbay 18 | Spillbay 18 closed | Change to fish passage season | Operator opened |  |
| CASCADES ISLAND—no out of criteria items |  |  |  |  |  |
| WASHINGTON SHORE |  |  |  |  |  |
| 2/25-2/26, 2/28 | Weir 67 | Differential > 1.0’ | UMT closed | No action taken |  |
| 3/1 | PH2 entrances | Entrance differential <1.0’ | Fish units down for trolley pipe installation | No action taken |  |
| 3/1 | Units 11 and 12 | Orifices closed | Closed for forebay dredging | No action taken |  |

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 07/08.  It is only out of criteria when tailwater is above 19.0’ msl.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 3/01 | 42.0 | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 1 March.

    - BI returned to service at 1500 on 29 February.

    - Cascades Island fishway PLC is in manual.

    - The UMT returned to service at 1010 on 1 March.

    - PH2 FOG stab plates are installed as of 03 December due to Fish Unit outages.  Stab plates will remain installed until new sea lion barriers are fabricated.

    - SLEDs are installed at all PH2 fishway entrances and Cascades Island.

    - SLEDs were installed in all Bradford Island entrances at 1100 on 28 February.

  - Juvenile Fishways.

    - DSM 1 remains dewatered.

    - DSM2 returned to service at 1641 on 25 February.

    - DSM2 ERG was tripped from 1625 until 1643 on 27 February.

    - DSM2 U11-12 orifices and the add-in water were closed from 2030 until 0600 on 1 March to facilitate debris removal in front of U11 and U12.

    - B2CC was opened from 1847 until 0600 on 29 February and from 2000 until 0600 on 1 March to facilitate debris removal in front of U11and U12.

  - Lamprey Fishways.  Winterized.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Three automated gates will be installed.  Optional items include sloping the floor and removing the DSM wall.

  - PH2 VBS mods.  Unit 17 is currently OOS for installation of monitoring equipment.

  - TIE crane.  The TIE crane boom repair is on hold until funding is secured.

  - BGS.  Installation of the BGS continues.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 20 December replaces the tail water operation for chum spawning teletype on 12 December.  Effective 0600 hours on Monday, December 24, until further notice, operate Bonneville tail water as follows – minimum 11.5 ft all hours.   The minimum 11.5 ft is for chum incubation.

  - A teletype sent on 06 February, requesting special operations while the behavioral guidance structure is being installed in the PH2 forebay, was modified as follows by a teletype sent on 27 February:

    - This operation is to occur on from 0600 until 1800 each day from 27 February through 5 March.

    - The project will operate only two main units plus the fish units at PH2, with the rest of the outflow going through PH1.  The two operating units will be determined by the location of the dive.

    - The minimum tailwater of 11.5’ will be maintained.

    - The project is authorized outside the standard unit operating priority as described in the Fish Passage Plan during this operation, and when needed to setup and return to normal operation.

    - The dive coordinators for Advanced American diving will contact the project, who will contact BPA, if the dives are completed earlier than specified.

  - A teletype sent on 11 February changes BON unit priority, beginning now until further notice.  PH1 is to be the priority powerhouse, which overrides Table BON-5 in the 2007 Fish Passage Plan, to facilitate the installation of the BGS in the PH2 forebay.  When low flows are not required at PH2, it is requested that a minimum of four units be operated at PH2.

  - A teletype sent on 11 February requests the lowest flows practical from PH2 on the following Tuesdays and Wednesday: February 12-13, February 19-20, and February 27-28.  The hours on Tuesdays are from 0800-1600, and on Wednesdays the hours are 0800-1100 and 1500-1600.  The minimum tailwater of 11.5’ (see7.1) will be maintained.  This request is a soft constraint.

  - A teletype sent on 19 February requests additional dates and times be added to the teletype sent on 06 February (see 7.2): 1100-1600 each day, on February 20, 21, 22, 23, 25, 27, 28, and 29.

  - A teletype sent on 26 February requests a soft forebay constraint of 71.5’ – 73.0’ from 0700 until 1200 on 27 and 28 February to accommodate work at Hood River City Park.

  - A teletype sent on 29 February requests the B2CC be opened from 1800 until 0600 from 29 February until 3 March to assist in the removal of debris in front of U11 and U12.

  - A teletype sent on 29 February requests the fish units be OOS from 0600 until 1800 on 1 March to facilitate the trolley pipe installation.

JAMES R. MAHAR, P.E.

Operations Manager