### CENWP-OP-B                                                                                                                         March 12, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 10 of 2008, which covers the period from 02 March through 08 March 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 129.5 to 175.2 kcfs.  Daily average project tailwater ranged from 11.6’ to 15.0’ msl.  Daily average powerhouse forebay elevation ranged from 73.8’ to 75.7’ msl.  Daily average river temperature was 42°F.  Secchi disk measurements ranged from 5.0 to 7.0.’

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-branch, respectively, are included in the spill pattern.  Daily average spill ranged from 1.3 to 37.0 kcfs (see 7.7).

  - Unit Operation.   Powerhouse 2 is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June for turbine rehab.

    - Unit 8 has remained out of service since 12 September 2006 for turbine rehab.

    - Unit 11 was out of service from 2045 on 01 March until 0526 on 02 March for debris removal.  U11 was out of service from 1320 until 2130 on 02 March to facilitate BGS and hydro acoustic installation dives.

    - Unit 12 was out of service from 2045 on 01 March until 0518 on 02 March for debris removal.  U12 was out of service from 1321 until 2130 on 02 March to facilitate BGS and hydro acoustic installation dives; and from 0521 until 1616 on 03 March to facilitate the BGS dive.

    - Unit 13 was out of service from 0532 until 1810 on 02 March; and from 0521 until 1616 on 03 March to facilitate the BGS dive.

    - Unit 14 was out of service from 0530 until 1822 on 02 March and from 0521 until 1616 on 03 March to facilitate the BGS dive.

    - Unit 15 has remained out of service since 25 November due to exciter and pole failure, followed by a head gate hydraulic line rupture.

    - Unit 16 was out of service from 0520 until 1312 on 02 March and from 0521 until 1534 on 03 March to facilitate the BGS dive.  U16 went out of service at 0702 on 05 March for FGE repairs in gatewell 16A.

    - Unit 17 has remained out of service since 23 February for FGE modifications.

    - Unit 18 was out of service from 0519 until 1316 on 02 March to facilitate the BGS dive.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - Fish unit 1 was placed on standby from 1446 until 1448 on 06 March for GDACS testing.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.

    - FV 1-1 was closed for cleaning from 1555 until 1624 on 06 March.

  - STS/VBS Inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA | NA | NA |

  - Dewatering and Fish Salvage.

    - Fish were salvaged from U16 draft tube on 06 March.  One 7-8’ sturgeon, two 5-6’ sturgeon, and 21 sturgeon approximately 4’ and less were removed.  The larger three sturgeon were released to the PH2 forebay, the rest were released at the Hamilton Island boat launch.  All were alive upon release, but some bruising was observed.

- RESEARCH

  - WDFW Fish Counting.  Video counting continues at Bradford Island and WA Shore.

  - Fish Field Unit.  Pinniped observations continue.  The pinniped trap is now in place.

  - Fish Field Unit. Sea lion scat collections continue.

  - WDFW/ODFW Sea lion hazing.  Hazing from boats continues.

  - USDA Wildlife Services. Pinniped hazing from the powerhouses continues.

  - Battelle.  Monitoring with hydroacoustics near B2CC entrance to study juvenile fish and kelt passage.

  - NOAA.  Released Spring Creek fish into PH2 gatewell on 04-05 March.

  - PSMFC Smolt Monitoring. Index and condition sampling began on 03 March.

- FISHWAYS

  - Project biologists inspected on 02-08 March.  NOAA inspected on 07 March.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 3/2-3 | PH1 Chain gates | Incorrect position | Change to fish passage season | Riggers adjusted on 04 March. |  |
| 3/4, 6 | B-branch diffusers | Incorrect position for TW | System in manual | Operators adjusted. |  |
| All week | WG 64 | Incorrect position | Brake seized | Repairs to occur next week. |  |
| CASCADES ISLAND |  |  |  |  |  |
| 3/4 | UMT staff gauge | <1.0’ | Unknown | Operators adjusted. |  |
| 3/8 | CI entrance | Differential > 2.0’ | System in manual. | Operators adjusted. |  |
| WASHINGTON SHORE |  |  |  |  |  |
| All week | DSM2 orifices | Incorrect configuration | Bad proximity switches | TR 08-28029, 08-28030 |  |
| 3/5, 7 | Weir 38 | Reading <1.0’ | Weir 50 add-in closed due to winterization. | No action taken |  |
| 3/2 | Units 11 and 12 | Orifices closed at night. | Closed for forebay dredging | No action taken |  |

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 08/09.  It is only out of criteria when tailwater is above 19.0’ msl.

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter often reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 3/02 | 41.0 | NA |
| 3/03 | 40.0 | NA |
| 3/04 | 39.8 | NA |
| 3/05 | 38.9 | NA |
| 3/06 | 40.7 | NA |
| 3/07 | 40.1 | NA |
| 3/08 | 41.2 | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 08 March.

    - Cascades Island fishway PLC is in manual.

    - PH2 FOG (floating orifice gate) stab plates are installed as of 03 December due to Fish Unit outages.  Stab plates will remain installed until recently completed floating orifice bars are installed.

    - SLEDs (sea lion exclusion devices) are installed at all fishway entrances.

  - Juvenile Fishways.

    - DSM 1 remains OOS.

    - DSM2 returned to service at 1641 on 25 February.

    - DSM2 U11-12 orifices and the add-in water were closed from 2030 on 01 March until 0600 on 02 March to facilitate debris removal in front of U11 and U12.

    - B2CC was opened from 2000 on 01 March until 0600 on 02 March to facilitate debris removal in front of U11and U12.

    - B2CC was opened for the season at 1525 on 05 March.

    - The Smolt Monitoring Facility was placed in sample mode at 0635 on 03 March.

    - DSM2 orifice lights were changed back to halogens on 28 February.

    - The hydrocannon was put in service for the season on 04 March.

  - Lamprey Fishways.  Remain winterized.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Three automated gates will be installed.  Optional items include sloping the floor and removing the DSM wall.

  - PH2 VBS mods.  Unit 17 is currently OOS for installation of monitoring equipment.  Unit 16 is currently OOS for FGE repairs in gatewell 16A.

  - TIE crane.  The TIE crane boom repair is on hold until funding is secured.

  - BGS.  Installation of the BGS concluded 04 March.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 20 December replaces the tail water operation for chum spawning teletype on 12 December.  Effective 0600 hours on Monday, December 24, until further notice, operate Bonneville tail water as follows – minimum 11.5 ft all hours.   The minimum 11.5 ft is for chum incubation.

  - A teletype sent on 06 February, requesting special operations while the behavioral guidance structure is being installed in the PH2 forebay, was modified as follows by a teletype sent on 27 February:

    - This operation is to occur on from 0600 until 1800 each day from 27 February through 5 March.

    - The project will operate only two main units plus the fish units at PH2, with the rest of the outflow going through PH1.  The two operating units will be determined by the location of the dive.

    - The minimum tailwater of 11.5’ will be maintained.

    - The project is authorized outside the standard unit operating priority as described in the Fish Passage Plan during this operation, and when needed to setup and return to normal operation.

    - The dive coordinators for Advanced American diving will contact the project, who will contact BPA, if the dives are completed earlier than specified.

  - A teletype sent on 11 February changes BON unit priority, beginning now until further notice.  PH1 is to be the priority powerhouse, which overrides Table BON-5 in the 2007 Fish Passage Plan, to facilitate the installation of the BGS in the PH2 forebay.  When low flows are not required at PH2, it is requested that a minimum of four units be operated at PH2.

  - A teletype sent on 29 February requests the B2CC be opened from 1800 until 0600 from 29 February until 03 March to assist in the removal of debris in front of U11 and U12.

  - A teletype sent on 04 March cancels the special operations teletypes for the installation of the BGS and trolley pipes (see 7.2 and 7.3) due to completion of the work.

  - A teletype sent on 05 March requests the PH2 Corner Collector be opened at 1600 on 05 March until further notice.  This is to support the Spring Creek hatchery release.

  - A teletype sent on 06 March requests special Project operations (see 7.6), in support of the Spring Creek hatchery release of 3.75 million tule fall Chinook on 05 March and 06 March, from 2359 on 06 March until 0600 on 10 March:

    - Spill approximately 36.4 kcfs.  Bays 1 and 18 will be open 0.5’.  Odd numbered bays 3-17 will be open 2’, even numbered bays will be on seal.

    - Spill operations will not exceed 120% total dissolved gas at Cascades Island and 115% at Camas/Washougal.

    - Maintain a minimum tailwater of 12.5’ to provide a TDG depth compensation level of 105% at Chum salmon redds (see 7.1).

  - A teletypes sent on 06 March requests outflows not to exceed 140 kcfs to allow a petroleum barge to transit Garrison Rapids.  This operation will occur for approximately 30 minutes, beginning at approximately 0001-0100 on 07 March, while maintaining a minimum tailwater of 12.5’ (see7.7.3).

  - A teletype sent on 07 March requests special operations in support of the Spring Creek hatchery release.  From 0600 on 06 March until 0600 on 10 March, PH2 units will operate at the low end of the lower quarter point of the 1% best efficiency range, which is approximately 50 MW’s, with a maximum hard constraint of 57 MW’s.  PH2 will be the priority powerhouse up to this loading criterion.  If it is necessary to operate PH1 units, MGR units will be operated as a last on first off basis, as outlined in the FPP, and within the 1% best efficiency range.

  - A teletype sent on 07 March modifies the teletype sent on 06 March (see 7.7).  The minimum tailwater, as measured at Tanner Creek, will be maintained at 13.0’.

JAMES R. MAHAR, P.E.

Operations Manager