### CENWP-OP-B                                                                                                                         April 9, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 14 of 2008, which covers the period from 30 March through 05 April 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 119.4 to 172.5 kcfs.  Daily average project tailwater ranged from 11.7’ to 14.8’ msl.  Daily average powerhouse forebay elevation ranged from 74.3’ to 75.3’ msl.  Daily average river temperature ranged from 44°F to 45°F.  Secchi disk measurements ranged from 4.0 to 5.0.’

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-branch, respectively, are included in the spill pattern.  Daily average spill ranged from 1.4 to 1.6 kcfs.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June for turbine rehab.

    - Unit 8 has remained out of service since 12 September 2006 for turbine rehab.

    - Unit 13 has remained out of service since 27 March for exciter replacement work.

    - Unit 14 was operated at the high end of 1% at 1135 on 02 April, and at the low end of 1% at 1155 on 04 April to facilitate gatewell testing.

    - Unit 15 has remained out of service since 25 November due to exciter and pole failure, followed by a head gate hydraulic line rupture.

    - Unit 16 returned to service at 1754 on 31 March.  It had been out of service since 05 March for FGE repairs in gatewell 16A.

    - Unit 17 was out of service from 0500 until 0806 on 01 April due to 17A STS failure.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - F1 was operated at reduced load from 1245 until 1422 on 03 April to pull SDE SLED in an attempt to flush debris stuck between the SLED and the SDE gate.

    - F2 was operated at reduced load from 1245 until 1422 on 03 April to pull SDE SLED in an attempt to flush debris stuck between the SLED and the SDE gate.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  No AWS closures occurred this reporting week.

  - STS/VBS Inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA | NA | NA |

  - Dewatering and Fish Salvage. The following fish salvage activities occurred this reporting week:

    - 15 Sturgeon and one catfish were recovered from the U16 tail logs on 31 March.

- RESEARCH

  - Battelle.  Monitoring with DIDSON acoustic camera and split-beam hydroacoustics near B2CC entrance to estimate the numbers and distribution of kelt-sized targets.

  - Battelle.  Equipment installation continues in the spillway for a spillway survival evaluation.

  - Fish Field Unit.  Pinniped observation and scat collection continue.

  - NOAA- Gilbreath.  Juvenile fish releases in gatewell 14A began on 02 April.

  - PSMFC Smolt Monitoring. Index and condition sampling continues.

  - USDA Wildlife Services. Sea lion hazing from the powerhouses continues.

  - WDFW Fish Counting.  Visual counting began 01 April at Bradford Island and WA Shore.

  - WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats continues.

- FISHWAYS

  - Project biologists inspected on 30 March – 05 April.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 3/30, 4/2,5 | B-branch diff. | Incorrect position | System in manual | Ops adjusted |  |
| 3/31, 4/2,5 | A-branch weir | >1.0’ | Unknown | No action taken |  |
| 4/5 | B-branch ent. | Differential >1.0’ | System in manual | Ops adjusted |  |
| All week | WG 64 | Incorrect position | Brake seized | Waiting on parts to come in. |  |
| CASCADES ISLAND |  |  |  |  |  |
| 4/5 | CI diffusers | Incorrect position | System in manual | Ops adjusted |  |
| WASHINGTON SHORE |  |  |  |  |  |
| 4/3 | WS entrances | Reading <1.5’ | Flush debris at SDE | In criteria by 1422. |  |

    - FG6-10 closed.  The motor was repaired on 15 August.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 08/09.  It is only out of criteria when tailwater is above 19.0’ msl.

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter often reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

    - Bradford Island temperatures are not available at this time.  The Project is waiting on a new temperature probe.  The new probe should be installed by mid-April.

Table 3.  Bonneville Fishway Temperatures

| Date | WS average | BI average |
| --- | --- | --- |
| 3/30 | 35.0 | NA |
| 3/31 | 36.8 | NA |
| 4/1 | 36.6 | NA |
| 4/2 | 38.3 | NA |
| 4/3 | 40.5 | NA |
| 4/4 | 39.8 | NA |
| 4/5 | 39.0 | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 05 April.

    - B-Branch and Cascades Island fishway PLC are in manual.

    - Installation of the FOG sea lion exclusion bars began on 21 March.  The four large orifices have bars (two at each end of the powerhouse). The stab plates remain installed in gates without exclusion bars.  The lifting beam was damaged during installation and repairs must be made prior to any more barriers being installed.

    - SLEDs (sea lion exclusion devices) and acoustic deterrents are installed at all fishway entrances.

    - The SDE gate is now functioning as it should.  On 03 April, the SLED was pulled in an attempt to flush the debris that was found to be preventing the gate from fully lowering.

  - Juvenile Fishways.

    - U14 orifices returned to auto at 1200 on 30 March.  They had been in manual to facilitate gatewell testing.

    - U14 orifices were put in manual from 02-05 April to facilitate the gatewell testing that occurred this week.

  - Lamprey Fishways.  Remain winterized.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Three automated gates will be installed.  Optional items include sloping the floor and removing the DSM wall.

  - PH2 VBS mods.  All gatewell modifications have been completed.  U16 returned to service on 31 March.

  - TIE crane.  The TIE crane boom repair is on hold until funding is secured.

  - BGS.  Re-alignment of the anchors was completed on 29 March.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 20 December replaces the tail water operation for chum spawning teletype on 12 December.  Effective 0600 hours on Monday, December 24, until further notice, operate Bonneville tailwater as follows – minimum 11.5 ft all hours.   The minimum 11.5 ft is for chum incubation.

JAMES R. MAHAR, P.E.

Operations Manager