### CENWP-OP-B                                                                                                                         April 28, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 17 of 2008, which covers the period from 20 April through 26 April 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 160.7 to 213.5 kcfs.  Daily average project tailwater ranged from 14.1’ to 17.5’ msl.  Daily average powerhouse forebay elevation ranged from 73.4’ to 76.2’ msl.  Daily average river temperature ranged from 47°F to 48°F.  Secchi disk measurements ranged from 3.0 to 5.0.’

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-branch, respectively, are included in the spill pattern.  Daily average spill ranged from 98.1 to 100.9 kcfs.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 3 went out of service at 0831 on 14 April for a five year overhaul.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 8 has remained out of service since 12 September 2006 for turbine rehab.

    - Unit 13 was out of service from 27 March until 1753 on 22 April for exciter replacement work.

    - Unit 14 was out of service from 0708 until 1048 on 22 April to replace a governor relay.

    - Unit 15 has remained out of service since 25 November 2007due to exciter and pole failure, followed by a head gate hydraulic line rupture.

    - Unit 17 was out of service from 0816 until 1207 on 23 April for exciter follow on work.

    - Unit 18 was out of service from 1028 until 1334 on 24 April for STS inspection.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.

- ACTIVITIES

  - AWS Closures.  No AWS closures occurred this reporting week.

  - STS/VBS Inspections.  The following STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | 4/23 | 650 |  |
| 12 | 4/23 | 516 |  |
| 13 | 4/23 | 19 |  |
| 14 | 4/24 | 502 | 14-B large branch in corner of slot. |
| 15 |  |  | OOS |
| 16 | 4/24 | 257 |  |
| 17 | 4/24 | 444 | 17-B minor repair motor. Replaced with spare STS. |
| 18 | 4/24 | 590 | 18-B screen pulled for inspection. |

  - Dewatering and Fish Salvage. No fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle.  BGS hydrohone installation began this reporting week.  See 6.1

  - Battelle.  Equipment installation completed for a spillway survival evaluation.

  - CRITFC.  Annual chinook, coho, sockeye and steelhead sampling in the AFF began 15 April.

  - Fish Field Unit.  Pinniped observation and scat collection continue.

  - NOAA- Gilbreath.  Juvenile Spring Creek fish releases in gatewell 14A continue. PIT tagged fish are being collected in SbyC and SMP samples.

  - NOAA – Jacobson.  PSMFC collected 25 chinook and 25 coho.  The fish were frozen for pathogen testing.

  - PSMFC Smolt Monitoring. Smolt monitoring activities ongoing.  Five steelhead fallbacks and one steelhead kelt dead on arrival were observed at the separator.  134 fish were collected for Univ. WA and NOAA researchers this week.

  - USDA Wildlife Services. Sea lion hazing from the powerhouses continues.

  - University of Washington.  Collection of barged fish, SbyC, and run of river fish for challenge trial began on 18 April.  84 fish received from PSMFC.

  - WDFW Fish Counting.  Visual counts at Bradford Island and WA Shore continues.

  - WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats continues.

- FISHWAYS

  - Project biologists inspected on 20 April – 26 April.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND AND BRANCHES |  |  |  |  |  |
| 4/20 | A-Branch weir | Reading >1.0’ | FB fluctuation. | No action taken. |  |
| 4/23-24, 26 | FG3-5 | Closed. | Diffuser broken. | See 4.2.4 | 08-38527 |
| 4/25 | BI diffusers | FG3-24 open | System in manual | Ops adjusted. |  |
| CASCADES ISLAND |  |  |  |  |  |
| 4/25 | CI diffusers | FG6-11 open | System in manual | Ops adjusted. |  |
| WASHINGTON SHORE |  |  |  |  |  |
| All week | DSM2 | Airburst not bursting all panels | PLC issues. | Electricians working on it. | 08-37503 |

    - FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have parted ways, however, so the diffuser will not be operational until repaired in winter 09/10.  It is OOC when TW is above 19.0’ msl.

    - PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter often reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - FG3-5 closed.  The valve stem separated from the gate, repairs are scheduled for winter 09/10.  It is OOC when TW is above 16.7’ msl.

    - FV 3-7 is in manual because it is not tracking with forebay elevations correctly, and is adjusted as necessary to maintain ladder criteria.  TR 08-39938.

    - PH2 velocity meter.  The velocity meter is believed to be broken.  Repairs will not be made until winter 08/09.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

    - Bradford Island temperatures are not available at this time.  The Project is waiting on a new temperature probe.  The new probe should be installed by mid-April.

    - The Washington Shore fishway temps are considerably lower than the AFF temps in the return pool.  Fisheries suspects the probe needs to be replaced.  Until such time, the AFF and WS temps will be reported.

Table 3.  Bonneville Fishway Temperatures

| Date | WS average | AFF | BI average |
| --- | --- | --- | --- |
| 4/20 | 37.5 | 46.6 | NA |
| 4/21 | 34.0 | 46.9 | NA |
| 4/22 | 34.5 | 46.7 | NA |
| 4/23 | 37.2 | 46.9 | NA |
| 4/24 | 37.2 | 46.7 | NA |
| 4/25 | 41.2 | 47.4 | NA |
| 4/26 | 42.9 | 49.0 | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 25 April.

    - B-Branch and Cascades Island fishway PLC are in manual.

    - Installation of the FOG sea lion exclusion bars has halted due to a broken lifting beam.  The four large orifices have bars (two at each end of the powerhouse). The stab plates remain installed in gates without exclusion bars.

    - SLEDs and acoustic deterrents remain installed at fishway entrances.

    - The AFF was put back in service on 14 April.

  - Juvenile Fishways.

    - Orifice 15C-N was closed on 14 April to support U15 head gate removal.

    - U14 orifices were placed in manual at 0705 on 23 April with 14A-S open and 14A-N closed.  Gatewell testing continues until 27 April.

    - F1 and F2 orifices were moved to manual throughout the week.  When FB elevation exceeded 75.5’ msl, the orifices would be closed to prevent the channel sensor from tripping the ERG.

      - F1 orifices were closed from 2145 on 21 April until 0816 on 22 April.

      - F2 orifices were closed from 2145 on 21 April until 0816 on 22 April.

  - Lamprey Fishways.  Remain winterized.

- CONSTRUCTION

  - PH1 ITS Wall Removal.  Three automated gates will be installed.  Optional items include sloping the floor and removing the DSM wall.

  - TIE crane.  The TIE crane boom repair is on hold until funding is secured.

  - BGS.  Installation of the acoustic nodes continues.

- HAZMAT, SPILLS AND CLEANUP.

  - Battelle work boat capsized while working on BGS.  The PNNL incident report is available from Derek Faber or from Project Fisheries.

    - Due to the release of about one gallon of boat fuel, gatewell orifices were closed from 13C through 11A and the DSM add-in was closed.  A sheen was seen in gatewells 12C and 13A.  No fuel sheen was seen at the JMF.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - The teletype sent on 20 December, which replaced the tail water operation for chum spawning teletype on 12 December, was replaced by a teletype sent on 11 April.  This teletype, effective immediately, requests 11.5’ minimum tailwater during all hours as a soft constraint.  This request is to protect emerging chum fry.

  - A teletype sent on 10 April requests the Project to begin juvenile fish spill beginning at 0001 on 10 April and continuing until further notice.  Spill will be 100Kcfs (or as close as the 100K pattern allows given forebay elevation) for 24 hours a day.  The current pattern can be found in the 2008 FPP at www.nwd-wc.usace.army.mil/tmt/documents/fpp/

  - A teletype sent on 22 April requests U14 be operated as first priority from 1200 on 23 April until 1200 on 27 April to accommodate the NOAA gatewell tests.

JAMES R. MAHAR, P.E.

Operations Manager