### CENWP-OP-B                                                                                                                         August 24, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 34 of 2008, which covers the period from 17 August through 23 August 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 117.0 to 138.8 kcfs.  Daily average project tailwater ranged from 11.0’ to 12.2’ msl.  Daily average powerhouse forebay elevation ranged from 74.9’ to 75.8’ msl.  Daily average river temperature ranged from 71°F to 72°F.  Secchi disk measurements ranged from 2.0’ to 7.0’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-branch, respectively, are included in the spill pattern.  Daily average spill ranged from 66.5 to 84.7 kcfs.

    - Bay 18 remains set at 2 dogs, or 2.9’, while the Bay 15 hoist is being repaired.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 1 returned to service at 1741 on 21 August.  It had been out of service since 07 July for five year overhaul.

    - Unit 2 returned to service at 1255 on 19 August.  It had been out of service since 13 August to facilitate U1 efficiency testing.  Unit 2 was out of service from 1008 on 20 August until 1643 on 21 August due to rotor inspection.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August for turbine rehab.

    - Unit 11 has remained out of service since 0238 on 12 June due to rotor cracks.

    - U17 returned to service at 1627 on 18 August.  It had been out of service since 15 August due to turbine bearing oil switch problems.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System (AWS) Closures.  The following AWS closures occurred this reporting week:

    - FV 6-9 was closed from 1549 until 1620 on 18 August for cleaning.

    - FV 3-7 was closed from 0652 until 0803 on 19 August for cleaning.

    - FV 3-9 was closed from 1608 until 1625 on 21 August for cleaning.

  - Submersible Traveling Screens (STS)/Vertical Barrier Screens (VBS) inspections.

No STS/VBS video inspections occurred this week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | NA | NA | NA |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle.  Behavioral Guidance System (BGS) hydrophones are installed.

  - Battelle.  Spillway survival evaluation continues.

  - CRITFC.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.  Water temps reached 70°F on 08 August.  High temperature sampling protocols remain in effect.  No sampling occurred on 19-20 August due to water temperatures >72.0°F

  - CTUIR.  Received 192 lamprey for broodstock and 30 lamprey for radio-tagging from NOAA Fisheries as part of their Umatilla Basin restoration project, this season.

  - NOAA – Gilbreath.  Saltwater tag effects study continues.

  - NOAA – Jacobson.  Pathogen testing of subyearling chinook has concluded.

  - NOAA – Moser.  Trapping and PIT tagging at the AFF continues.  1424 lamprey have been trapped at the AFF this season.  Of the 1424 lamprey; 222 went to CTUIR, 20 to Warm Springs, 20 to UI for proximity analysis, 140 to UI for the flume testing, 295 were radio-tagged, 523 were PIT tagged and 300 were double tagged.  Water temps reached 70°F on 08 August.  High temperature sampling protocols remain in effect. No sampling occurred on 19-20 August due to water temperatures >72.0°F

  - PSMFC Smolt Monitoring.  Condition, index, and GBT monitoring continue.  No adult salmonids were recorded crossing the separator bars.  Seven juvenile shad were collected for USGS.  Water temps reached 70°F on 15 August.  High temperature sampling protocols were in effect from 15 August until 23 August.

  - University of Idaho.  Lamprey evaluations have concluded for the season.  595 lamprey were radio tagged and 300 of those were double tagged.

  - USDA Wildlife Services. Avian hazing concluded on 31 July.

  - USGS. Seven juvenile shad were received for the Impacts of American Shad on the Columbia River study.

  - WDFW Fish Counting.  Visual counts at Bradford Island and WA Shore continue.  Tule counting began on 15 August.

  - Yakima Nation Fisheries.  Sort by code collection, fat metering, floy tagging, and measuring Wenatchee River coho is scheduled to commence the week of 25 August.

- FISHWAYS

  - Project biologists inspected on 17-23 August.  NOAA inspected on 22 August.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 8/20, 23 | FG3-27 | Incorrect position | System in manual | Ops adjusted |  |
| 8/21 | BI Weir | Reading <1.0’ | Unknown | Ops adjusted |  |
| 8/21 | B-branch weir | Reading <1.0’ | Unknown | Ops adjusted |  |
| 8/22-23 | B-branch entrance | Differential >2.0’ | System in manual | Ops adjusted |  |
| CASCADES ISLAND (CI) |  |  |  |  |  |
| 8/20, 23 | FG6-14 | Incorrect position | System in manual | Ops adjusted |  |
| 8/23 | CI entrance | Differential >2.0’ | System in manual | Ops adjusted |  |
| WASHINGTON SHORE (WS) – nothing to report |  |  |  |  |  |

    - FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated.  The diffuser will not be operational until repaired in winter 08/09.  It is Out of Criteria (OOC) when Tailwater (TW) is above 19.0’ msl.

    - PH1 elec. velocity meter is in an eddy.  Readings for FPP criteria are taken at the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - FG3-5 closed.  The shaft and gate have separated.  Repairs scheduled for winter 09/10.  It is OOC when TW is above 16.7’ msl.

    - PH2 velocity meter.  The velocity meter is believed to be broken.  Repairs will not be made until winter 08/09.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the Upstream Migrant Transportation (UMT) channel and the main ladder at Washington shore. Temperatures are in Fahrenheit.

4.3.1       Bradford Island temperatures are not available at this time.

Table 3.  Bonneville Fishway Temperatures

| Date | WS average | AFF | SMF |
| --- | --- | --- | --- |
| 8/17 | 72.1 | 72.2 | 71.1 |
| 8/18 | 71.9 | 71.9 | 70.8 |
| 8/19 | 71.5 | 71.6 | 70.7 |
| 8/20 | 71.2 | 71.3 | 70.2 |
| 8/21 | 70.7 | 70.9 | 69.9 |
| 8/22 | 70.2 | 70.2 | 69.2 |
| 8/23 | 70.4 | 70.4 | 69.4 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 23 August.

    - B-Branch and Cascades Island fishway Program Logic Controls (PLC) are in manual.

    - Removal of the Floating Orifice Gate (FOG) sea lion exclusion bars has not yet begun due to a broken lifting beam.  The four large orifices have bars (two at each end of the powerhouse). The stab plates remain installed in gates without exclusion bars.

    - Cascades Island exit remains open to provide an exit for fish upstream of the UMT picket leads.

  - Juvenile Fishways.

4.6.1	   B2CC is scheduled to close on 01 September.

  - Lamprey Fishways.

    - WS and BI AWS lamprey ramps remain in service.

    - The WS NDE lamprey ramp was shut off on 08 August due to high temperatures in the AFF and limitations on trapping at temperatures >70.0oF.  The trap will be operated four days a week until the daily average temperature in the AFF <69.5oF.  No sampling occurred on 19-20 August due to water temperatures >72.0°F

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway (ITS) Wall Removal.  Three automated gates to be installed.

  - TIE crane.  The TIE crane boom repair contract is currently out for bid.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 19 June details the start of summer spill for juvenile fish passage.  It supercedes the teletype sent on 10 April.

    - From 0001 on 21 July until 2400 on 31 August, Bonneville will spill 75K during daytime hours and to the 120% gas cap at night.

    - The spill pattern is found in section 2, Table BON-15 of the 2008 FPP, and daytime hours are defined in section 2, Table BON-5 of the 2008 FPP.  www.nwd-wc.usace.army.mil/tmt/documents/fpp/

    - At no time will the Project voluntarily spill above the 120% gas cap.  The 120% spill cap level for Bonneville can be found in the teletype “SPILL PRIORITY LIST”.

    - At all times, maintain a minimum generation of 30kcfs and minimum spill of 50kcfs.

  - A teletype sent on 23 June authorizes the Project to dog open Spillbays 11, 14, and 18 at 2 stops (approx. 2.9’).  Spill shall be maintained as specified in 7.1.  This is to facilitate repairs to the Spillbay 15 hoist.

  - A teletype sent on 06 August authorizes the Project to deviate from unit priority from 06 August until 22 August to accommodate efficiency and relay testing of units 1 and 2.

  - A teletype sent on 15 August requests the BON pool be operated with a 1.5’ range hard constraint and a 1.0’ range soft constraint from 0600 on 19 August until 1800 on 22 August.  This operation is intended to hold the pool as steady as possible to minimize debris in the tribal gill nets.

  - A teletype sent on 21 August requests the BON pool be operated with a 1.5’ range hard constraint and a 1.0’ range soft constraint from 0600 on 26 August until 1800 on 29 August.  This operation is intended to hold the pool as steady as possible to minimize debris in the tribal gill nets.

JAMES R. MAHAR, P.E.

Operations Manager