### CENWP-OP-B                                                                                                                         September 29, 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 39 of 2008, which covers the period from 21 through 27 September 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 72.2 to 106.5 kcfs.  Daily average project tailwater ranged from 7.1’ to 9.7’ msl.  Daily average powerhouse forebay elevation ranged from 75.8’ to 76.4’ msl.  Daily average river temperature ranged from 65 to 67°F.  Secchi disk measurements averaged 7.0’.

  - Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-branch, respectively, are included in the spill pattern.  Daily average spill was 1.4 kcfs.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August for turbine rehab.

    - Unit 11 has remained out of service since 12 June due to rotor cracks.

    - Unit 15 has remained out of service since 05 September due to rotor pole failure.

    - Unit 16 has remained out of service since 03 September due to 230kV River Crossing  Transmission Lines 3&4 330MW limit.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System (AWS) Closures.  No AWS closures occurred this reporting week:

  - Submersible Traveling Screens (STS)/Vertical Barrier Screens (VBS) inspections.

No STS/VBS video inspections occurred this week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11-18 | N/A | N/A | N/A |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle.  Behavioral Guidance System (BGS) hydrophones were removed from Powerhouse 2.

  - Battelle.  Spillway survival evaluation concluded.  Equipment was removed on 22 September.

  - CRITFC.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - NOAA – Gilbreath.  Saltwater tag effects study continues.

  - PSMFC Smolt Monitoring.  Condition, index, and GBT monitoring continue.  Two chinook were recorded crossing the separator bars.  No research fish collected during this report period.

  - USGS. No shad were received for the Impacts of American Shad on the Columbia River study.

  - WDFW Fish Counting.  Visual counts at Bradford Island and WA Shore continue.  Tule counting began on 15 August.

- FISHWAYS

  - Project biologists inspected on 21 - 27 September.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 9/21 | B-branch entrance | Differential > 2.0 | System in manual | Ops adjusted |  |
| 9/24, 26 | B-branch entrance | Differential < 1.0 | System in manual | Ops adjusted. |  |
| 9/26, 27 | Powerhouse 1 collection channel | Differential <1.0 | Collection channel sensors working improperly | Ops adjusted in manual |  |
| 9/26 | B-branch entrance gates | Incorrect position for TW | System in manual | Ops adjusted |  |
| CASCADES ISLAND (CI) |  |  |  |  |  |
| 9/21 | CI entrance | Differential > 2.0 | System in manual | Ops adjusted |  |
| 9/24, 9/26 | CI entrance | Differential < 1.0 | System in manual | Ops adjusted |  |
| 9/26 | CI entrance gates | Incorrect position for TW |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 9/27 | Powerhouse 2 collection channel | B diffusers improper position for tailwater |  | Ops adjusted |  |

    - FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated.  The diffuser will not be operational until repaired in winter 08/09.  It is Out of Criteria (OOC) when Tailwater (TW) is above 19.0’ msl.

    - PH1 elec. velocity meter is in an eddy.  Readings for FPP criteria are taken at the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - FG3-5 closed.  The shaft and gate have separated.  Repairs scheduled for winter 09/10.  It is OOC when TW is above 16.7’ msl.

    - PH2 velocity meter.  The velocity meter is believed to be broken.  Repairs will not be made until winter 08/09.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the Upstream Migrant Transportation (UMT) channel and the main ladder at Washington shore. Daily average temperatures are in Fahrenheit.

4.3.1       Bradford Island temperatures are not available at this time.

Table 3.  Bonneville Fishway Temperatures

| Date | WA shore | AFF | SMF |
| --- | --- | --- | --- |
| 9/21 | 66.5 | 66.4 | 65.5 |
| 9/22 | 66.0 | 66.0 | 65.1 |
| 9/23 | 65.5 | 65.6 | 64.4 |
| 9/24 | 65.1 | 65.2 | 64.0 |
| 9/25 | 65.2 | 65.3 | 64.2 |
| 9/26 | 65.1 | 65.1 | 64.1 |
| 9/27 | 65.0 | 65.1 | 64.0 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 26 September.

    - B-Branch and Cascades Island fishway Program Logic Controls (PLC) are in manual.

    - Removal of the Floating Orifice Gate (FOG) sea lion exclusion bars has not yet begun due to a broken lifting beam.  The four large orifices have bars (two at each end of the powerhouse). The stab plates remain installed in gates without exclusion bars.

    - Cascades Island exit remains open to provide an exit for fish upstream of the UMT picket leads.

    - STSs were installed on 15 September in Units 1 and 3 for adult fallback.  DSM1 is running south.

  - Juvenile Fishways.

    - Spill for juvenile fish passage concluded 01 September.

4.6.2	   B2CC was closed on 01 September.

  - Lamprey Fishways.

    - WS north downstream entrance lamprey ramp was winterized and removed from the water on 16 September.  BI and WA shore AWS lamprey ramps remain in service.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway (ITS) Wall Removal.  Three automated gates to be installed.

  - TIE crane.  The TIE crane boom repair contract is currently out for bid.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 19 September requests that Bonneville forebay be operated between 75’-76.5’ as a hard constraint and between 75.5’ and 76.5’ as a soft constraint from 0600 on 22 September until 1800 hours on 26 September to limit wear and tear on gill net equipment and to provide a steady pool for a White Salmon River fish salvage.

  - A teletypes sent on 26 September authorizes a temporary deviation from Bonneville unit priority, as listed in table BON-11 of the 2008 Fish Passage Plan, beginning at 0001 on 29 September until tail log installation is completed.  During this time, the PH2 unit priority will be 11, 18, 12, 13, 15, 17, 14, 16.  This is necessary to guide fish away from Unit 11, prior to unit dewatering.

  - A teletype sent on 26 September requests that Bonneville forebay be operated between 75’-76.5’ as a hard constraint and between 75.5’ and 76.5’ as a soft constraint from 0600 on 29 September until 1800 hours on 3 October.  This operation is requested to limit wear and tear on gill net equipment, and facilitate work on the Dalles spill wall, and to provide a steady pool for White Salmon River fish salvage.

JAMES R. MAHAR, P.E.

Operations Manager