### CENWP-OP-B                                                                                                                                  02 November 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 44 of 2008, which covers the period from 26 October through 1 November 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 83.9 to 113.2 kcfs.  Daily average project tailwater ranged from 7.4’ to 10.5’ msl.  Daily average powerhouse forebay elevation ranged from 76.0’ to 76.4’ msl.  Daily average river temperature ranged from 56ºF to 58ºF.  Secchi disk measurements ranged from 6.0’ to 7.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged between 1.1 to 1.2 kcfs.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August for turbine rehab.

    - Unit 11 has remained out of service since 12 June due to rotor cracks.

    - Unit 15 has remained out of service since 05 September due to rotor pole failure.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS closures occurred this reporting week:

  - STS/VBS inspections.  No STS/VBS video inspections occurred this week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1, 3, 11-18 | NA | NA | NA |

  - Dewatering and Fish Salvage.  No fish salvage activity occurred this reporting week.

- RESEARCH

  - NOAA – Gilbreath.  Saltwater tag effects study continues.

  - NOAA – Lamprey.  Bradford Island and Washington Shore lamprey ramps were dewatered this week.

  - PSMFC Smolt Monitoring.  One adult chinook was recorded crossing the separator bars.  Monitoring operations concluded at 0700 on 31 October.

  - WDFW Fish Counting.  Visual counts at Bradford Island and WA Shore concluded on 31 October.  Video counting 16 hours/day began on 1 November.

- FISHWAYS

  - Project biologists inspected on 26 October through 1 November.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion |  | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE |  | ##### TR# |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND |  |  |  |  |  |  |  |
| 10/30-31 | A-branch weir | Reading <1.0’ |  | FV3-9 not controlling properly |  | Ops adjusted. |  |
| 10/26, 10/29 | B-branch diffusers | Incorrect position |  | System in manual |  | Ops adjusted. |  |
| 10/27, 10/29, 11/1 | B-branch entr. | Differential <1.0’> |  | System in manual |  | Ops adjusted. |  |
| 10/29-30 | B-branch gates | Incorrect position |  | System in manual |  | Ops adjusted |  |
| CASCADES ISLAND |  |  |  |  |  |  |  |
| 10/30, 11/1 | CI entrance | Differential <1.0’ |  | System in manual |  | Ops adjusted |  |
| 10/30 | CI entr. gates | Incorrect position |  | System in manual |  | Ops adjusted |  |
| WASHINGTON SHORE – no out of criteria items |  |  |  |  |  |  |  |

    - FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated.  The diffuser will not be operational until repaired in winter 08/09.  It is OOC when TW is above 19.0’ msl.

    - PH1 elec. velocity meter is in an eddy.  Readings for FPP criteria are taken at the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - FG3-5 closed.  The shaft and gate have separated.  Repairs scheduled for winter 09/10.  It is OOC when TW is above 16.7’ msl.

    - PH2 velocity meter.  The velocity meter is believed to be broken.  Repairs will be made in winter 08/09.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT channel and the main ladder at Washington shore.  Temperatures are in ºF.

Table 3.  Bonneville Fishway Temperatures

| Date | WA shore | Bradford Is. |
| --- | --- | --- |
| 10/26 | 57.6 | NA |
| 10/27 | 56.7 | NA |
| 10/28 | 56.5 | NA |
| 10/29 | 56.8 | NA |
| 10/30 | 57.0 | NA |
| 10/31 | 56.7 | NA |
| 11/1 | 56.7 | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 30 October.

    - B-Branch and Cascades Island fishway PLCs are in manual.

    - Due to a broken lifting beam, sea lion exclusion bars remain installed in the four large FOGs.  Stab plates remain installed in the smaller FOGs.

    - Cascades Island exit remains open to provide an exit for fish upstream of the UMT picket leads.

    - STSs were installed on 15 September in Units 1 and 3 for adult fallback.  DSM1 is running south.

    - The PH1 electronic velocity meter was IN criteria for most of the week.

    - The Bradford Island fish exit was closed for cleaning from 1045 until 1335 on 28 October and from 1457 until 1550 on 30 October.

  - Juvenile Fishways.

    - The JMF went to bypass at 0710 on 31 October.

  - Lamprey Fishways.  BI and WA shore AWS lamprey ramps were dewatered this week.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Three automated gates to be installed this FY09.  Wall removal anticipated for FY10.

  - TIE crane.  The TIE crane boom repair contract has been awarded.  Completion expected in Spring ‘09.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 3 October, in place until further notice, requests the Bonneville forebay be maintained between 75.0’-76.5’ msl as a hard constraint and between 75.5’ and 76.5’ msl as a soft constraint.  This operation is requested to facilitate TDA spillway work and White Salmon river fish salvage.

JAMES R. MAHAR, P.E.

Operations Manager