### CENWP-OP-B                                                                                                                                  25 November 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 47 of 2008, which covers the period from 16 through 22 November 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 120.3to 140.5 kcfs.  Daily average project tailwater ranged from 11.4’ to 12.7’ msl.  Daily average powerhouse forebay elevation ranged from 74.7’ to 75.7’ msl.  Daily average river temperature ranged from 52ºF to 53ºF.  Secchi disk measurements ranged from 5.0’ to 7.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged between 1.1 to 1.2 kcfs.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 5 was out of service from 0054 on 17 November until 1753 on 18 November due to wiring modifications.

    - Unit 6 was out of service from 0054 on 17 November until 1753 on 18 November due to wiring modifications.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August for turbine rehab.

    - Unit 11 has remained out of service since 12 June due to rotor cracks.

    - Unit 15 has remained out of service since 05 September due to rotor pole failure.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week:

    - FV 1-1 was closed for cleaning from 1139 until 1318 on 18 November, and from 1458 until 1602 on 20 November.

    - FV 6-9 was closed for cleaning from 0725 until 0732 on 17 November.

  - STS/VBS inspections.  The following STS/VBS video inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1 | 11/19 | 422 | OK |
| 3 | 11/19 | 375 | OK |
| 11 | 11/19 | 0 | Out of service |
| 12 | 11/19 | 650 | OK |
| 13 | 11/19 | 643 | OK |
| 14 | 11/19 | 627 | OK |
| 15 | 11/19 | 0 | Out of service |
| 16 | 11/19 | 550 | OK |
| 17 | 11/19 | 657 | OK |
| 18 | 11/19 | 649 | OK |

  - Dewatering and Fish Salvage.  No fish salvage activity occurred this reporting week.

- RESEARCH

  - NOAA – Gilbreath.  Saltwater tag effects study concluded.

  - WDFW Fish Counting.  Daily video counting at Bradford Island and Washington Shore continues.

- FISHWAYS

  - Project biologists inspected on 16 through 22 November.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion |  | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE |  | ##### TR# |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND |  |  |  |  |  |  |  |
| 11/20 | A-branch weir | Reading >1.0’ |  | Unknown |  | No action taken |  |
| 11/17,21 | PH1 S. entrance | Differential <1.0 |  | FV1-1 in manual |  | Ops adjusted. |  |
| 11/21 | PH1 N. entrance | Differential <1.0 |  | FV1-1 in manual |  | Ops adjusted. |  |
| CASCADES ISLAND– no out of criteria items |  |  |  |  |  |  |  |
| WASHINGTON SHORE – no out of criteria items |  |  |  |  |  |  |  |

    - FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated.  The diffuser will not be operational until repaired in winter 08/09.  It is OOC when TW is above 19.0’ msl.

    - PH1 elec. velocity meter is in an eddy.  Readings for FPP criteria are taken at the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - FG3-5 closed.  The shaft and gate have separated.  Repairs scheduled for winter 09/10.  It is OOC when TW is above 16.7’ msl.

    - PH2 velocity meter.  The velocity meter is believed to be broken.  Plans are to replace with a mechanical meter.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT channel and the main ladder at Washington shore.  Daily average temperatures are in Fahrenheit.

    - Temperatures are not available this week due to program upgrades on the new ACE-IT refresh computers.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 21 November.

    - B-Branch and Cascades Island fishway PLCs are in manual.

    - Due to a broken lifting beam, sea lion exclusion bars remain installed in the four large FOGs.  Stab plates remain installed in the smaller FOGs.

    - Cascades Island exit remains open to provide an exit for fish upstream of the UMT picket leads.

    - STSs were installed on 15 September in Units 1 and 3 for adult fallback.  DSM1 is running south.

    - Bradford Island fishway exit was closed for cleaning from 1005 until 1129 on 18 November.

    - Washington Shore exit was closed for cleaning from 1027 until 1129 on 18 november.

    - Bradford Island fishway is scheduled to go to orifice flow on 01 December.

  - Juvenile Fishways.

    - The JMF went to bypass on 31 October.  DSM2 remains in service.

  - Lamprey Fishways.  BI and WA shore AWS lamprey ramps are winterized.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Three automated gates to be installed this FY09.  Wall removal anticipated for FY10.

  - TIE crane.  The TIE crane boom repair contract has been awarded.  Completion expected in Spring ‘09.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 06 November cancels the 03 Oct teletype for high forebay operation as of 0600 on 07 November 2008.  The upper Bonneville forebay limit is 76.5’ msl.  The lower limit is 71.5’ msl.

  - A teletype sent on 12 November replaces the teletype sent on 07 November regarding BON tailwater operation for chum spawning.  Hard constraint 1 – Operate Bonneville project tailwater elevation no lower than 11.3’ msl during all hours.  Hard constraint 2 – Maintain 11.3’ to 11.7’ msl Bonneville project tailwater elevation between 0600 and 1800 hours daily.  Target elevation is 11.5’ msl.  Please see CBT message:  Bonneville chum operations #3 sent on 11/12/08 at 1610 for more detail.

JAMES R. MAHAR, P.E.

Operations Manager