### CENWP-OD-B                                                                                                                                  7 December 2008

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 49 of 2008, which covers the period from 30 November through 06 December 2008.

- OPERATION SUMMARY

  - Daily average river flows ranged from 124.5 to 136.1 kcfs.  Daily average project tailwater ranged from 11.4’ to 12.1’ msl.  Daily average powerhouse forebay elevation ranged from 74.0’ to 75.6’ msl.  Daily average river temperature ranged from 48ºF to 51ºF.  Secchi disk measurements ranged from 4.5’ to 7.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged between 0.5 to 1.2 kcfs.

    - On 1 December, spill bay 18 was closed due to the Bradford Island fishway going to orifice flow and subsequent dewatering.

  - Unit Operation.  Powerhouse 2 is the priority powerhouse.

    - Unit 1 was out of service from 0733 until 0927 on 01 December for STS removal.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 8 was out of service from 0339 on 04 December until 1630 on 06 December for Current Transformer installation.

    - Unit 9 has remained out of service since 04 August for turbine rehab.

    - Unit 11 has remained out of service since 12 June due to rotor cracks.

    - Unit 12 was out of service from 0501 until 1442 on 02 December for turbine bearing o-ring repair.

    - Unit 15 has remained out of service since 05 September due to rotor pole failure.

    - Unit 18 was out of service from 0935 until 1145 on 01 December for trashrack cleaning.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

    - Unit F1 went out of service at 0012 on 01 December for annual maintenance.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed for cleaning this reporting week:

    - FV 1-1 was closed for cleaning from 0743 until 0805 on 01 December.  FV 1-1 was closed at 0851 on 01 December for winter maintenance.

    - FV 3-7 and FV 3-9 were closed at 0851 on 01 December for winter maintenance.

    - FV 4-3 and FV 4-4 were closed at 0851 on 01 December for winter maintenance.

  - STS/VBS inspections.  The following STS/VBS video inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1, 3 | NA | NA | Scheduled for removal for ITS modifications in late January |
| 11-18 | NA | NA | Scheduled for removal the week of 15 December. |

  - Dewatering and Fish Salvage.  The following fish salvage and dewatering activities occurred this week;

    - Bradford Island fishway.  Project Fisheries, with the assistance of WDFW, CRITFC, and Project Maintenance crews dewatered the Bradford Island fishway on 3 December.  From the main ladder, bios salvaged two adult coho, two adult steelhead, 14 juvenile steelhead, four juvenile chinook, one sculpin, 22 whitefish/peamouth/suckers (residents), one catfish, one crawdad, 100 juvenile shad.  From the Junction pool, bios salvaged two adult steelhead, 53 juvenile steelhead, 10 juvenile chinook, three lamprey, 12 sturgeon (3’-8’), one adult shad, twenty four juvenile shad, fifty four resident fish.

    - A-branch diffuser pools.  Project Fisheries moved sturgeon out of A-Branch diffuser pools on 3 December by moving them through the orifices, to the next diffuser pool, by hand and by leaving flushing water on in FG3-6 overnight.  By 4 December the sturgeon had moved to tailwater.

- RESEARCH

  - WDFW Fish Counting.  Daily video counting at Washington Shore continues.  Video counting at Bradford Island concluded on 1 December, when the fishway went to orifice flow.

- FISHWAYS

  - Project biologists inspected 30 November, and 2, 4 and 5 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND |  |  |  |  |  |
| 11/30 | A-branch weir | Reading >1.0’ | Unknown. | No action taken |  |
| CASCADES ISLAND– no out of criteria items |  |  |  |  |  |
| WASHINGTON SHORE |  |  |  |  |  |
| All week | SA-24 board | Not reading | OOS for repairs |  |  |

    - FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated.  The diffuser will not be operational until repaired in winter 08/09.  It is OOC when TW is above 19.0’ msl.

    - PH1 elec. velocity meter is in an eddy.  Readings for FPP criteria are taken at the south end mechanical meter.

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - FG3-5 closed.  The shaft and gate have separated.  Repairs scheduled for winter 09/10.  It is OOC when TW is above 16.7’ msl.

    - PH2 velocity meter.  The velocity meter is believed to be broken.  Plans are to replace with a mechanical meter.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT channel and the main ladder at Washington shore.  Daily average temperatures are in Fahrenheit.

Table 3.  Bonneville Fishway Temperatures

| Date | WA shore | Bradford Is. |
| --- | --- | --- |
| 11/30 |  |  |

    - Temperatures are not available this week due to program upgrades on the new ACE-IT refresh computers.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 05 December.

    - Cascades Island fishway PLC is in manual.

    - Due to a broken lifting beam, sea lion exclusion bars remain installed in the four large FOGs.  Stab plates remain installed in the smaller FOGs.

    - Cascades Island exit remains open to provide an exit for fish upstream of the UMT picket leads.

    - STSs were installed on 15 September in Units 1 and 3 for adult fallback.  DSM1 is running south.

    - FV6-9 was put in manual at 0343 on 04 December.  The valve was not properly controlling in auto.

    - Bradford Island fishway went to orifice flow at on 1 December.  The ladder was dewatered on 3 December.  Flow remains out of FG3-18 to move sturgeon and to prevent a need to unnecessarily handle large numbers of sturgeon.

    - The AFF went to orifice flow at 1442 on 01 December.

  - Juvenile Fishways.

    - The JMF remains in bypass.

    - DSM1 dewatering has been scheduled for late January.

  - Lamprey Fishways.  BI and WA shore AWS lamprey ramps are winterized.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Automated gate installation in FY09.  Wall removal in FY10.

  - TIE crane.  The TIE crane boom repair contract has been awarded.  Completion expected in Spring ‘09.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 06 November establishes the upper BON forebay limit as 76.5’ msl. and the lower limit as 71.5’ msl.

  - A teletype sent on 12 November requests BON tailwater operations for chum spawning.  It requests a tailwater elevation hard constraint of a minimum of 11.3’ msl during all hours and between 11.3’ to 11.7’ msl between 0600 and 1800 hours daily.  Target elevation is 11.5’ msl.

  - Winter maintenance is scheduled to begin 1 December.  For a complete list of dewatering activities, please contact Project Fisheries.

JAMES R. MAHAR, P.E.

Operations Manager