### CENWP-OD-B                                                                                                                                  29 March 2009

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 13 of 2009, which covers the period from 22 through 28 March 2009.

- OPERATION SUMMARY

  - Daily average river flows ranged from 122.8 to 175.2 kcfs.  Daily average project tailwater ranged from 11.7’ to 15.8’ msl.  Daily average powerhouse forebay elevation ranged from 74.9’ to 76.1’ msl.  Daily average river temperature ranged from 42 to 44ºF.  Secchi disk measurements ranged from 5.0’ to 6.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-Branch, respectively, are open 6” during daylight hours.  Daily average spill was 1.3 kcfs on each day during the reporting period.

  - Unit Operation.  Powerhouse 2 remains the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 8 went through a 72 hour heat run from 1345 on 24 March until 1436 on 27 March.  It has been out of service since 17 December 2008 due to generator lower guide bearing failure.

    - Unit 9 has remained out of service since 04 August for turbine rehab.

    - Unit 11 has remained out of service since 12 June due to rotor damage.

    - Unit 14 was out of service from 0738 until 1114 on 24 March for trashrack fish release pipe installation.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valve was closed this reporting week:

    - FV1-1 was closed for cleaning from 1600 until 1620 on 23 March.

    - FV6-9 was closed for cleaning from 1620 until 1628 on 26 March.

  - STS/VBS inspections.  No STS/VBS activities occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1, 2 | NA | NA | Removed until 3 September. |
| 11 | NA | NA | U11 remains OOS |
| 12-18 | NA |  | STSs installed. |

  - Dewatering and Fish Salvage.  The following fish salvage or dewatering activities occurred this week.

    - Fisheries removed one juvenile unclipped steelhead, two juvenile bass and one stickleback from DSM1 on 24 March.

- RESEARCH

  - CRITFC- JMF video.  The viewing window has been installed in the sample flume at the Juvenile Monitoring Facility (JMF).

  - FFU- sea lion. Night time sea lion observations occurred on 27 March.

  - NOAA- spill gate test.  Vibration sensors scheduled for installation on Spillbay 11 on 29 March.

  - NOAA- B2CC PIT.  Fish releases scheduled for 13 April.

  - NOAA-FGE.  Test releases of PIT-tagged Spring Creek NFH fish began on 25 March.  Fish were released in the intake for Unit 14 A-slot.  Fish were recovered at the JMF.

  - PNNL- BGS.  Trailers were delivered on 23 March.

  - PSMFC- Smolt monitoring.  Fallbacks recorded crossing the separator bars included; 15 steelhead and one Chinook.  A daily update is available at the following link: www.fpc.org/adultsalmon/bonkeltcounts.htm.

  - WDFW Fish Counting.  Video counting continues at Bradford Island and Washington Shore.  Visual counting is scheduled to begin on 01 April.

  - WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats and trapping continues.

  - USDA Wildlife Services. Hazing from shore continues.

- FISHWAYS

  - Project biologists inspected 22-28 March.  NOAA inspected on 18 March.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND |  |  |  |  |  |  |
| 3/25 | ITS | 1B, 1C, 3B closed | Trashrack raking | Opened after raking. |  |  |
| 3/25-27 | A-Branch weir | Reading >1.0’ | Unknown | No action taken. |  |  |
| 3/26 | A-branch | FG3-4 closed | Power out for contractor work. | Power back by end of the day. |  |  |
| 3/24 | B-branch | FG3-25 closed | System in manual | Switched to auto at 1023. |  |  |
| CASCADES ISLAND (CI) |  |  |  |  |  |  |
| 3/27 | CI diffusers | FG6-11 closed | Unknown | Operators adjusted. |  |  |
| WASHINGTON SHORE |  |  |  |  |  |  |
| 3/26 | F2 orifices | South orifices closed | System in manual | Operators opened the orifices. |  |  |

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - Fisheries is working with maintenance to build and install a mechanical velocity meeting at the PH2 south monolith.  For now, there is no velocity meter.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 3/22 | 41.5 | 41.1 |
| 3/23 | 41.6 | 41.2 |
| 3/24 | 41.8 | 41.5 |
| 3/25 | NA | 41.6 |
| 3/26 | NA | 41.8 |
| 3/27 | NA | 42.1 |
| 3/28 | NA | 42.3 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 26 March.

    - B-branch PLC was placed in auto at 1023 on 24 March.  New transducers have been installed.  Fisheries will continue to monitor accuracy.

    - Cascades Island PLC is in auto and functioning properly.

    - 11 FOGs are in service, one remains out of service until structural repairs can be made.

    - The AFF remains dewatered.

    - The Washington Shore fishway entrances are in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain a 13’ opening (when tailwater elevations allows).

  - Juvenile Fishways.

    - The PH1 ice and trash sluiceway was closed from 1259 until 1315 on 24 March to facilitate dewatering DSM1.

    - DSM1 was filled with water in an effort to re-water Unit 8.  Since Units 7 and 9 were OOS, the only way to bring U8 to forebay was to backfill water from the DSM through the orifices and into the unit.  Water to the DSM was provided through Unit 10 orifices.  Unit 8 and unit 10 orifices were open from 1030 on 21 March until 1327 on 23 March.

  - Lamprey Fishways.  BI and WS AWS lamprey ramps remain winterized.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal is scheduled for FY10.

  - TIE crane.  The TIE crane boom repair has been delayed.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A 06 November teletype set the BON upper forebay limit as 76.5’ msl and the lower limit as 71.5’ msl.

  - A 31 December teletype, regarding BON tailwater operations for chum egg incubation, requests a minimum tailwater elevation of 11.5’ msl during all hours from 0001 on 01 January 2009 until further notice.

  - A 24 March teletype requests Unit 8 be moved to the first priority unit while completing a 72 hour heat run.  The change in priority began at 1345 on 24 March and ended at 1436 on 27 March.

  - Unit 10 was operated from 1545 until 2254 on 24 March and at 0853 on 26 March.  During this time of operation, sea lion observers reported dead sturgeon in the PH1 tailrace.  The unit was not started using the slow roll process on either date.  Fisheries sent a memo to the region detailing the mortalities and what has been done to correct the problem.

JAMES R. MAHAR, P.E.

Operations Manager