### CENWP-OD-B																								12 July 2009

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 28 of 2009, which covers the period from 05 through 11 July 2009.

- OPERATION SUMMARY

  - Daily average river flows ranged from 148.7 to 205.3 kcfs.  Daily average project tailwater ranged from 13.2’ to 17.2’ msl.  Daily average powerhouse forebay elevation ranged from 75.4’ to 76.2’ msl.  Daily average river temperature ranged from 67ºF to 68ºF.  Secchi disk measurements ranged from 5.3’ to 6.0’.

  - Daily average spill ranged from 93.6 to 97.1 kcfs.

    - Spillbay 17 remains on two dogs (2.9’).  The bay should be at 3.5’ but due to a broken hoist it has to be dogged instead.  Flow is compensated through adjacent bays.

  - Unit Operation.  Powerhouse 2 remains the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 10 was out of service from 1326 on 05 July until 1756 on 09 July for the bank 9/10 outage.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 16 has remained out of service since 09 May due to exciter troubles and a grounding issue.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 5-9 was closed for cleaning from 1504 until 1515 on 06 July.

    - FV 6-9 was closed for cleaning from 1520 until 1530 on 06 July.

  - STS/VBS inspections.  The following STS/VBS inspections occurred 30 June:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1-2 | NA | NA | Remains out of service until 03 September. |
| 11 | NA | NA | Out of service |
| 12 | 6/30 | 679 | OK |
| 13 | 6/30 | 676 | OK |
| 14 | 6/30 | 679 | OK |
| 15 | 6/30 | 675 | OK |
| 16 | NA | NA | Out of service |
| 17 | 6/30 | 678 | OK |
| 18 | 6/30 | 678 | OK |

2.2.1	Dewatering and Fish Salvage.  No fish salvage or dewatering activities occurred this week.

- RESEARCH

  - CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC- JMF video.  Waiting on new equipment for monitoring juvenile fish passage.

  - CTUIR.  10 lamprey collected for radio-tagging from the AFF this week.  Total of 50 lamprey collected for 2009.

  - NOAA-FGE.  Test releases of PIT-tagged run of the river fish continued this week to assess passage conditions at two turbine operating ranges (middle and upper end of the 1% efficiency range).  2886 sub-yearling chinook were collected at the JMF.

  - NOAA Lamprey.  Trapping lamprey at the AFF for passage evaluations.

  - NOAA- pathogens. Determination of pathogen prevalence in juvenile salmonids.  25 sub-yearling chinook were collected at the JMF.

  - NOAA- PIT tag detectors.  Vibration sensors remain installed on Spill bay 6.

  - PNNL- BGS.  All hydrophones are installed and collecting data for the BGS evaluation.

  - PSMFC- Smolt monitoring.  Daily condition and index monitoring continues.  Fallbacks recorded crossing the separator bars included; one steelhead and one salmon.  Gas Bubble Trauma (GBT) exams occur twice a week; see www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  2886 sub-yearling chinook were collected for the FGE testing and 25 sub-yearling chinook were collected for pathogen testing.

  - WDFW Fish Counting.  Visual counting continues at Bradford Island and Washington Shore fishways. Nighttime video counts began 15 June.

  - USDA Wildlife Services. Avian hazing continues.

  - University of Idaho.  Tagging lamprey for evaluation of the Cascades Is. entrance weir.

- FISHWAYS

  - Project biologists inspected 05 through 11 July.  NOAA inspected on 10 July.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) --Nothing to report |  |  |  |  |  |
| CASCADES ISLAND (CI)--Nothing to report |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 7/7, 10-11 | Entrances | <13.0’ opening | In manual | Operators adjusted. |  |

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

    - CI south entrance gate remains broken and closed.  It is been OOS since 10 April.  It will remain in the closed position for all tailwater elevations during the modified entrance evaluation.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington Shore.

    - The WS and BI probes have been sent to the manufacturer to be calibrated.  The AFF probe is being used as a surrogate.

Table 3.  Bonneville Fishway Temperatures

| Date | AFF average |
| --- | --- |
| 7/05 | 66.0 |
| 7/06 | 65.7 |
| 7/07 | 65.4 |
| 7/08 | 64.9 |
| 7/09 | 65.0 |
| 7/10 | 65.9 |
| 7/11 | 66.4 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 10 July.

    - The Washington Shore fishway entrances are in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening.

    - There are currently 11 of 12 floating orifice gates (FOG) installed at Powerhouse 2.  The last gate was removed for maintenance and replaced with a bulkhead.

    - WA shore and Bradford Is. fishways are out of shad passage criteria and returned to normal operation.

  - Juvenile Fishways.

    - The PH1 ITS was closed for debris removal on 17 June.  To provide the best flow possible until the new gates can be automated, the chaingates were set as follows:  1B- 71’msl, 3B- 70’ msl, 6C- 70’msl, and 9A- 72’.

  - Lamprey Fishways.

    - The CI entrance weir and bulkhead slot filler are installed.

    - All lamprey passage structures remain watered-up.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal is scheduled for FY10.  Old DSM bypass equipment in the tailrace is scheduled for removal in FY10 as well.

    - The Program Logic Control (PLC) for the automated gates has not been completed.

    - The keeper pin for gate 3B needs to be replaced.  The Project plans to do this on 14 July when flows have dropped enough to no longer require units to operate at PH1.

  - TIE crane.  The new boom is to arrive in mid-July.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 17 May and revised on 18 May provides guidance for voluntary and involuntary spill by project as well as spill caps per project for various total dissolved gas (TDG) levels.

  - A teletype sent on 19 June requests the summer spill operations starting at 0001 on 21 June.  From 21 June until 2400 on 20 July, the Project will spill approximately 85 kcfs during daytime hours and to the 120% gas cap at night.  From 0001 on 21 July until 2400 on 31 August, the Project will spill approximately 75 kcfs during the day and up to the 120% gas cap at night.

  - A teletype sent on 06 July requests that from 0600 on 08 July until 1800 on 11 July, the forebay elevation be maintained at 75.0’ – 76.5’ msl. as a hard constraint and 75.5’ – 76.5’ msl. as a soft constraint.  This operation is requested to reduce debris in tribal fishing nets, reduce entanglement of fishing nets, minimize boat access problem, and reduce the chances of nets being torn from their anchors by limiting pool fluctuation.

  - A teletype sent on 09 July temporarily lifts the normal full pool and the 06 July tribal fishing pool operation.  A 0.5’ forebay exceedence to 77.0’, effective 1800 on 09 July until 1700 on 10 July, has been approved to help free a stuck fuel barge in the Bonneville pool near Hood River.

  - A teletype sent on 09 July cancels the high forebay exceedence to free the stuck fuel barge and resumes tribal fishing pool operations.

JAMES R. MAHAR, P.E.

Operations Manager