### CENWP-OD-B																								27 July 2009

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 30 of 2009, which covers the period from 19 through 25 July 2009.

- OPERATION SUMMARY

  - Daily average river flows ranged from 122.8 to 156.5 kcfs.  Daily average project tailwater ranged from 11.7’ to 13.8’ msl.  Daily average powerhouse forebay elevation ranged from 75.0’ to 76.1’ msl.  Daily average river temperature was 69ºF to 70ºF.  Secchi disk measurements ranged from 4.7’ to 7.0’.

  - Daily average spill ranged from 95.8 to 101.0 kcfs.

    - Spillbay 17 remains on two dogs (2.9’).  The bay should be at 2.5’ during daytime hours, but due to a broken hoist it has to be dogged instead.  Spillbays are adjusted as necessary to maintain appropriate spill volume.

  - Unit Operation.  Powerhouse 2 remains the priority powerhouse.

    - Unit 3 was out of service from 1355 on 19 July until 1632 on 23 July for Annual Maintenance and Bank 3/4 bi-annual service.

    - Unit 4 was out of service from 1355 on 19 July until 1632 on 23 July for Bank 3/4 bi-annual service.

    - Unit 6 has remained out of service since 13 July for a five year overhaul.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 16 has remained out of service since 09 May due to exciter troubles and a grounding issue.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - Lamprey operations occurred on 20, 21 and 24 July.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed this reporting week:

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1-2 | NA | NA | Remains out of service until 03 September. |
| 11, 16 | NA | NA | Out of service |
| 12-15, 17-18 | NA | NA | Scheduled for the week of 27 July |

    - Dewatering and Fish Salvage.  No fish salvage/ dewatering activity occurred this week.

- RESEARCH

  - CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC- JMF video.  Waiting on new equipment for monitoring juvenile fish passage.

  - CTUIR.  Radio-tagging collections completed.  Total of 50 lamprey collected for 2009.

  - NOAA-FGE.  Gilbreath will present his data to the USACE PDT team on 29 July.

  - NOAA Lamprey.  Trapping lamprey at the AFF for passage evaluations.

  - NOAA- pathogens. Determination of pathogen prevalence in juvenile salmonids.  25 sub-yearling chinook were collected at the JMF.

  - NOAA- PIT tag detectors.  Vibration sensors remain installed on Spill bay 6.

  - PNNL- BGS.  All hydrophones are installed and collecting data for the BGS evaluation.

  - PSMFC- Smolt monitoring.  Daily condition and index monitoring continues.  No fallbacks recorded crossing the separator bars.  Gas Bubble Trauma (GBT) exams occur twice a week; see www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  25 sub-yearling chinook were collected for pathogen testing.

  - WDFW Fish Counting.  Visual counting continues at Bradford Island and Washington Shore fishways. Nighttime video counts began 15 June.

  - USDA Wildlife Services. Avian hazing continues.

  - University of Idaho.  Tagging lamprey for evaluation of the Cascades Is. entrance weir.

- FISHWAYS

  - Project biologists inspected 19 through 25 July.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 7/21 | ITS chaingates 3B & 6C | <2.5’ opening | Gates not automated | Chaingates were adjusted on 22 July. |  |
| CASCADES ISLAND (CI)--Nothing to report |  |  |  |  |  |
| 7/24 | FG 6-10 and 6-11 | Incorrect position for TW | PLC error | PLC reset |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 7/19-20, 25 | Weir 37 & 38 | Reading < 1.0’ | Unknown | Operators adjusted. |  |

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

    - CI south entrance gate remains broken and closed.  It is been OOS since 10 April.  It will remain in the closed position for all tailwater elevations during the modified entrance evaluation.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington Shore.

Table 3.  Bonneville Fishway Temperatures

| Date | WS Ave | BI Ave | AFF Ave | SMF Ave |
| --- | --- | --- | --- | --- |
| 7/19 | N/A | N/A | 68.3 | N/A |
| 7/20 | N/A | N/A | 68.3 | N/A |
| 7/21 | N/A | N/A | 68.8 | N/A |
| 7/22 | 68.3 | 69.0 | 68.7 | 68.9 |
| 7/23 | 68.2 | 68.8 | 68.7 | 68.9 |
| 7/24 | 68.4 | 68.9 | 68.9 | 69.0 |
| 7/25 | 69.1 | 69.6 | 69.5 | 69.6 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 25 July.

    - The Washington Shore fishway entrances are in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - There are currently 11 of 12 floating orifice gates (FOG) installed at Powerhouse 2.  The last gate was removed for maintenance and replaced with a bulkhead.

  - Juvenile Fishways.

    - To provide the best flow possible until the new gates can be automated, the PH1 ITS chaingates were set as follows:  1B- 72’msl, 3B- 72’ msl, 6C- 72’msl, and 9A- 72’.  The Project will monitor forebay elevations and adjust gates 3B and 6C to maintain >2.5’ depth.

  - Lamprey Fishways.

    - The CI entrance weir and bulkhead slot filler are installed.

    - All lamprey passage structures remain watered-up.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal is scheduled for FY10.  Old DSM bypass equipment in the tailrace is scheduled for removal in FY10 as well.

    - The Program Logic Control (PLC) for the automated gates has not been completed.

  - TIE crane.  The new boom is to arrive in late July.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 17 May and revised on 18 May provides guidance for voluntary and involuntary spill by project as well as spill caps per project for various total dissolved gas (TDG) levels.

  - A teletype sent on 19 June requests the summer spill operations starting at 0001 on 21 June.  From 21 June until 2400 on 20 July, the Project will spill approximately 85 kcfs during daytime hours and to the 120% gas cap at night.  From 0001 on 21 July until 2400 on 31 August, the Project will spill approximately 75 kcfs during the day and up to the 120% gas cap at night.

  - A teletype sent on 16 July changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July until U11 returns to service.

  - Starting on 20 July, Ben Hausmann will begin a 120-day detail in the NWP Planning Section and Jon Rerecich will begin a 120-day detail as Lead Project Biologist at Bonneville.

JAMES R. MAHAR, P.E.

Operations Manager