### CENWP-OD-B																							27 September 2009

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 39 of 2009, which covers the period from 20 through 26 September 2009.

- OPERATION SUMMARY

  - Daily average river flows ranged from 72.1 to 104.2 kcfs.  Daily average project tailwater ranged from 7.6’ to 9.6’ msl.  Daily average powerhouse forebay elevation ranged from 77.0’ to 77.4’ msl.  Daily average river temperature ranged from 67ºF to 70ºF.  Secchi disk measurements ranged from 6.5’ to 7.0’ all week.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged between 1.4 to 1.5 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 4 has remained out of service since 14 September 2009 for five-year overhaul.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 16 has remained out of service since 09 May due to exciter troubles and a grounding issue.

    - Unit 17 was out of service from 1216 until 1515 on 24 September for STS repairs.  Unit 17 went out of service at 1743 on 26 September due to exciter trouble.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 was on standby, or reduced load, for lamprey testing from 2246 on 23 September until 0424 on 24 September.

    - F2 was on standby, or reduced load, for lamprey testing from: 2203 on 19 September until 0356 on 20 September; 2155 on 22 September until 0358 on 23 September; 2357 on 25 September until 0410 on 26 September.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following  AWS valves were closed this reporting week:

    - FV3-9 was closed for cleaning from: 0705 until 0715 on 21 September; 1455 until 1510 on 26 September.

    - FV5-9 was closed for cleaning from: 1635 until 1645 on 24 September.

    - FV6-9 was closed for cleaning from: 1529 until 1538 on 24 September.

  - STS/VBS inspections.  The following STS/VBS inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1 | NA | NA |  |
| 2 | NA | NA |  |
| 11 | NA | NA | Out of service |
| 12 | 9/24 | 716 | Cleaned VBS |
| 13 | 9/24 | 137 |  |
| 14 | 9/24 | 285 |  |
| 15 | 9/24 | 565 |  |
| 16 | NA | NA | Out of service |
| 17 | 9/24 | 530 | STS-B replaced rivets |
| 18 | 9/24 | 697 | Cleaned VBS |

    - Dewatering and Fish Salvage.  The following dewatering/fish salvage activities occurred this week:

      - Fisheries removed 12 sculpin, two catfish and one crawdad from Unit 16 tail logs on 25 September.

- RESEARCH

  - CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC- JMF video.  New video equipment installed for monitoring juvenile fish passage.

  - NOAA- PIT tag detectors.  Vibration sensors remain installed on Spill bay 6.

  - NWW truck release site.  Trucking and release at BON began on 16 August; continues every other day.

  - PSMFC- Smolt monitoring.  Modified sampling protocols due to elevated river temperature occurred this week.  Two steelhead were observed crossing the separator bars.  Gas Bubble Trauma (GBT) exams occur twice a week; see www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - WDFW Fish Counting.  Visual counting continues at Bradford Island and Washington Shore fishways. Nighttime video counts began 15 June.

  - WDFW Tule counts.  Tule counting continues.

  - USGS Shad.   Shad collection concluded for the season.

- FISHWAYS

  - Project biologists inspected 20 through 26 September.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| One day | BI entrances | No calibration check |  |  |  |
| 9/23 | A-branch weir | Reading >1.0’ | High forebay | No action taken. |  |
| CASCADES ISLAND (CI) nothing to report |  |  |  |  |  |
| One day | CI entrance | No calibration check |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 9/25 | Weir 37 | Reading <1.0’> | Unknown | No action taken. |  |
| 9/22-23 | Weir 38 | Reading >1.0’ | Unknown | No action taken. |  |
| PH1 ITS – please see 4.5.6 |  |  |  |  |  |

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island; the junction of the UMT and the Washington Shore ladder; the return ladder at the Adult Fish Facility (AFF); and the general holding tank at the Juvenile Monitoring Facility (JMF).

    - Three out of four temperature probes are experiencing technical difficulties.  Please see www.nwd-wc.usace.army.mil/tmt/documents/ops/temp/200909.lcol.html for additional temp data.

Table 3.  Bonneville Fishway Temperatures

| Date | WS Avg | BI Avg | AFF Avg | JMF Avg |
| --- | --- | --- | --- | --- |
| 9/20 | NA | 67.4 | NA | NA |
| 9/21 | NA | 66.8 | NA | NA |
| 9/22 | NA | 66.2 | NA | NA |
| 9/23 | NA | 66.0 | NA | NA |
| 9/24 | NA | 66.3 | NA | NA |
| 9/25 | NA | 66.6 | NA | NA |
| 9/26 | NA | 66.4 | NA | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was not checked this week.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.

    - There are currently 11 of 12 floating orifice gates (FOG) installed at Powerhouse 2.  One gate was removed for maintenance and replaced with a bulkhead.

    - FV4-3 remains in manual.

    - DSM1 continues to head south.  STSs are in Units 1 and 2.  Orifice lights are on in those units as well.  Orifices are open for units 1-9.

      - STSs and VBS will be pulled, and DSM1 dewatered, starting on 19 October to facilitate the ITS wall removal contract.

    - Chaingates are not quite to FPP criteria.  Automated gates 3B and 10B are manually set to 2.5’.  Automated gate 6C still is not working properly.  Chain gate 6B has been opened to maintain at least 2.5’ opening.  With the high forebay, 1B has been closed and 1C opened to 72’ msl.

    - The Bradford Island fish exit was closed for cleaning from 1455 until 1510 on 26 September.

  - Juvenile Fishways.

    - The B2CC remains closed.

    - The B2CC avian wires were repaired on 22 September.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and bulkhead slot filler are installed.

    - The CI LPS and PH2 NDE ramp are dewatered for the season.

    - The BI and WS AWS LPSs remain watered-up.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Wall and DSM1 outfall scheduled for removal in FY10.

    - The Program Logic Control (PLC) for the automated gates has not been completed.

  - TIE crane.  The crane is scheduled to be available by the 2010 fish passage season.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July until U11 returns to service.

  - A teletype sent on 27 July changes the unit operation for PH2.  PH2 units will operate at the low end to mid point of the 1% efficiency range.

  - A teletype sent on 01 September requests BON forebay maintain 71.5’ to 78.0’ msl as a hard constraint and 71.5’ to 77.5’ as a soft constraint from 0001 on 08 September 2009 until 31 March 2010.

    - From 07 November through 31 December, 78.0’ msl will occur as needed to support chum spawning below the dam.

    - This requested operation is a deviation from the Bonneville Water Control Manual.  The deviation was coordinated with the Region on 21 August.

    - This operation is requested to facilitate TDA spillwall work.  If the spillwall construction is completed early, a return to normal forebay operating elevations will be coordinated.

  - A teletype sent on 08 September requests that BON operate the single highest priority unit at PH1 (currently Unit 1) first.  This is to reduce the risk of BON going outside the 1% efficiency range while supporting the high forebay request for TDA spillwall work.  This request is effective immediately until further notice.

  - A teletype sent on 18 September requests the forebay be operated at 76.0’ – 77.5’ msl as a hard constraint and 76.5’ – 77.5’ as a soft constraint from 0600 on 22 September until 1800 on 25 September.  This action is to limit pool fluctuation, during tribal fishing, to: reduce local currents that sweep debris into tribal fishing nets; reduce rapid water level changes that entangle nets; minimize boat access problems; avoid tearing nets from their anchors.

    - A teletype sent on 25 September extends the forebay restriction from 0600 on 29 September until 1800 on 01 October.

JERRY CARROLL

Acting Operations Manager