### CENWP-OD-B																							05 October 2009

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 40 of 2009, which covers the period from 27 September through 03 October 2009.

- OPERATION SUMMARY

  - Daily average river flows ranged from 76.7 to 93.8 kcfs.  Daily average project tailwater ranged from 6.8’ to 8.6’ msl.  Daily average powerhouse forebay elevation ranged from 77.2’ to 77.5’ msl.  Daily average river temperature ranged from 66ºF to 67ºF.  Secchi disk measurements ranged from 6.0’ to 7.0’ all week.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged between 1.4 to 1.5 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 4 has remained out of service since 14 September 2009 for five-year overhaul.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 16, which has been out of service since 09 May due to exciter troubles and a grounding issue, started a 24 hour heat run at 1415 on 29 September and returned to service at 1626 on 30 September.

    - Unit 17, which has been out of service since 26 September due to exciter trouble, returned to service at 0736 on 28 September.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F2 was on standby, or reduced load, for lamprey testing from: 2159 on 28 September until 0355 on 29 September; 2137 on 30 September until 0404 on 01 October.  Lamprey testing has concluded for the season.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following  AWS valves were closed this reporting week:

    - FV1-1 was closed from 0821 until 1030 on 30 September.

    - FV3-9 was closed for cleaning from: 1449 until 1506 on 01 October.

  - STS/VBS inspections.  The following STS/VBS inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1-2 | NA | NA | Scheduled for removal on 19 October. |
| 11, 16 | NA | NA | Out of service |
| 12-15, 17-18 | NA | NA | Scheduled for the week of 19 October. |

    - Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week:

- RESEARCH

  - CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC- JMF video.  New video equipment installed for monitoring juvenile fish passage.

  - NOAA- PIT tag detectors.  Vibration sensors remain installed on Spill bay 6.

  - NWW truck release site.  Trucking and release at BON began on 16 August; continues every other day.

  - PSMFC- Smolt monitoring.  Daily condition and index monitoring continues.  Three steelhead, 16 salmon and one unknown salmonid were observed crossing the separator bars.  Gas Bubble Trauma (GBT) exams occur twice a week; see www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - WDFW Fish Counting.  Visual counting continues at Bradford Island and Washington Shore fishways. Nighttime video count concluded on 30 September.

  - WDFW Tule counts.  Tule counting concluded on 30 September.

- FISHWAYS

  - Project biologists inspected 27 September through 03 October.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) nothing to report |  |  |  |  |  |
| CASCADES ISLAND (CI) nothing to report |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 9/28-29 | Weir 37 | Reading <1.0’> | Unknown | No action taken. |  |
| 9/29 | Weir 38 | Reading >1.0’ | Unknown | No action taken. |  |
| PH1 ITS – please see 4.5.6 |  |  |  |  |  |

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island; the junction of the UMT and the Washington Shore ladder.

    - Three out of four temperature probes are experiencing technical difficulties.

Table 3.  Bonneville Fishway Temperatures

| Date | WS Avg | BI Avg |
| --- | --- | --- |
| 9/27 | NA | 66.3 |
| 9/28 | NA | 65.6 |
| 9/29 | NA | 65.1 |
| 9/30 | NA | 64.7 |
| 10/01 | NA | 64.4 |
| 10/02 | NA | 64.3 |
| 10/03 | NA | 63.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 03 October.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.

    - There are currently 11 of 12 floating orifice gates (FOG) installed at Powerhouse 2.  One gate was removed for maintenance and replaced with a bulkhead.

    - FV4-3 remains in manual due to a defective conduit sensor.  Repairs scheduled for 09/10.

    - DSM1 continues to head south.  STSs are in Units 1 and 2.  Orifice lights are on in those units as well.  Orifices are open for units 1-9.

      - STSs and VBS will be pulled, and DSM1 dewatered, starting on 19 October to facilitate the ITS wall removal contract.

    - Chaingates are not quite to FPP criteria.  Automated gates 3B and 10B are manually set to 2.5’.  Automated gate 6C still is not working properly.  Chain gate 6B has been opened to maintain at least 2.5’ opening.  With the high forebay, 1B has been closed and 1C opened to 72’ msl.

    - The Bradford Island fish exit was closed for cleaning from: 1449 until 1506 on 01 October.

  - Juvenile Fishways.

    - The B2CC remains closed.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and bulkhead slot filler are installed.

    - The CI LPS and PH2 NDE ramp are dewatered for the season.

    - The BI and WS AWS LPSs remain watered-up.

- CONSTRUCTION

  - PH1 Ice and Trash Sluiceway Wall Removal.  Wall and DSM1 outfall scheduled for removal in FY10.

    - The Program Logic Control (PLC) for the automated gates has not been completed.

  - TIE crane.  The crane is scheduled to be available by the 2010 fish passage season.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July until U11 returns to service.

  - A teletype sent on 27 July changes the unit operation for PH2.  PH2 units will operate at the low end to mid point of the 1% efficiency range.

  - A teletype sent on 01 September requests BON forebay maintain 71.5’ to 78.0’ msl as a hard constraint and 71.5’ to 77.5’ as a soft constraint from 0001 on 08 September 2009 until 31 March 2010.

    - From 07 November through 31 December, 78.0’ msl will occur as needed to support chum spawning below the dam.

    - This requested operation is a deviation from the Bonneville Water Control Manual.  The deviation was coordinated with the Region on 21 August.

    - This operation is requested to facilitate TDA spillwall work.  If the spillwall construction is completed early, a return to normal forebay operating elevations will be coordinated.

  - A teletype sent on 08 September requests that BON operate the single highest priority unit at PH1 (currently Unit 1) first.  This is to reduce the risk of BON going outside the 1% efficiency range while supporting the high forebay request for TDA spillwall work.  This request is effective immediately until further notice.

  - A teletype sent on 25 September requests the forebay be operated at 76.0’ – 77.5’ msl as a hard constraint and 76.5’ – 77.5’ as a soft constraint from 0600 on 29 September until 1800 on 01 October.  This action is to limit pool fluctuation, during tribal fishing, to: reduce local currents that sweep debris into tribal fishing nets; reduce rapid water level changes that entangle nets; minimize boat access problems; avoid tearing nets from their anchors.

  - A teletype sent on 29 September requests Unit 16 be operated as first on/last off, and operated at the high end of 1%, from 1600 on 29 September until 1600 on 30 September.  This request is to facilitate testing Unit 16 after completion of the stator repairs.

JERRY CARROLL

Acting Operations Manager