### CENWP-OD-B																							06 December 2009

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 49 of 2009, which covers the period from 29 November through 05 December 2009.

- OPERATION SUMMARY

  - Daily average river flows ranged from 119.2 to 134.5 kcfs.  Daily average project tailwater ranged from 11.4’ to 12.3’ msl.  Daily average powerhouse forebay elevation ranged from 76.0’ to 77.0’ msl.  Daily average river temperature ranged between 46ºF to 49ºF.  Secchi disk measurements ranged from 4.0’ to 7.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 0.5 to 1.2 kcfs.

    - Bay 1 was on sill from 0626 on 01 December until 1000 on 03 December to facilitate Cascades Island remaining at orifice flow during the installation of the Smith-Root array in the UMT.

    - Bay 18 was closed at 1230 on 04 December to facilitate Bradford Island going to orifice flow; in preparation for dewatering on 07 December.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 3 has remained out of service since 19 November due to a loss of Bank 3/4.

    - Unit 4 has remained out of service since 19 November due to a loss of Bank 3/4.

    - Unit 5 went out of service at 1000 on 04 December for DSM1 orifice removal.

    - Unit 6 returned to service at 1309 on 30 November, after being out of service since 21 November for DSM1 orifice removal.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 10 has remained out of service since 16 November for DSM1 orifice removal.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 17 returned to service at 1212 on 30 November after being out of service since 05 October for a four year overhaul.  Unit 17 was out of service from 1649 on 30 November until 1007 on 01 December due to governor cycling issues.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 went out of service at 0003 on 01 December for 2-SJ1 work and annual maintenance.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed for cleaning this reporting week:

    - FV1-1- Closed so Bradford Island could go to orifice flow at 1245 on 04 December.

    - FV3-7- Closed so Bradford Island could go to orifice flow at 1245 on 04 December.

    - FV3-9- Closed for cleaning from 1624 until 1635 on 03 December.  Closed so Bradford Island could go to orifice flow at 1500 on 04 December.

    - FV4-3- Closed so Bradford Island could go to orifice flow at 1245 on 04 December.

    - FV4-4- Closed so Bradford Island could go to orifice flow at 1245 on 04 December.

    - FV5-3- Closed from 0039 on 01 December until 0950 on 03 December to facilitate Cascades Island going to orifice flow for the Smith-Root array installation in the UMT.

    - Fv5-4- Closed from 0039 on 01 December until 0950 on 03 December to facilitate Cascades Island going to orifice flow for the Smith-Root array installation in the UMT.

    - FV5-9- In manual from 0658 on 01 December until 0950 on 03 December to facilitate Cascades Island going to orifice flow for the Smith-Root array installation in the UMT.  Closed for cleaning from 1020 until 1046 on 03 December.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 1, 2 | NA |  | STSs and VBSs removed |
| 11 | NA |  | Out of service |
| 12-16 | NA |  | Scheduled to be pulled on 15 December. |
| 17 | NA |  | Out of service |
| 18 | NA |  | Scheduled to be pulled on 15 December. |

  - Dewatering and Fish Salvage.  The following dewatering/fish salvage activities occurred this week:

      - On 01 December 12 steelhead, one chinook, one bass and several juvenile shad were removed from the UMT by Project Fisheries, BPA and PSMFC biologists.

- RESEARCH

  - NOAA- PIT tag detectors.  Vibration sensors remain installed on Spill bay 6.

  - ODFW/WDFW sea lions.  Moved the forebay sea lion trap on 19 November.

  - Smith-Root sea lion array.  Installation in the UMT occurred on 02 December.

  - WDFW Fish Counting.  Video counting at Bradford Island and Washington Shore continues.

- FISHWAYS

  - Project biologists inspected 29 November through 04 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 12/4 | A-branch weir | Reading <1.0’> | Unknown | No action. |  |
| CASCADES ISLAND (CI) |  |  |  |  |  |
| 12/1-3 | UMT | No flow | Smith-Root array installation. | Retuned to service at 1000 on 12/3 |  |
| 12/1-3 | CI entrance | Ladder at orifice flow | Smith-Root array install in UMT | Returned to service at 1000 on 12/3 |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 12/1-5 | PCC/monoliths | Head differential, entrances closed | F1 OOS. No stab plates installed. | Stab plates to be installed the week of 12/6 |  |

    - PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 2009/2010 in water work period.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

  - Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island; the junction of the UMT and the Washington Shore ladder.

    - Three out of four temperature probes are experiencing technical difficulties.

    - Temperatures are not recorded during winter maintenance season.

Table 3.  Bonneville Fishway Temperatures

| Date | WS Avg | BI Avg |
| --- | --- | --- |
| 11/29 | NA | 47.9 |
| 11/30 | NA | 47.7 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 04 December.

    - FV4-3 was in manual due to a defective conduit sensor.  It was closed at 1245 on 04 December.

    - The BI exit was closed for cleaning from 1624 until 1635 on 03 December.  The exit was throttled at 1500 on 04 December to facilitate BI going to orifice flow.

    - The PH1 ice and trash sluiceway (ITS) remains dewatered.

      - The ITS concrete plate, located at the downstream edge of the ITS and covers access to an old wooden bulkhead, has been removed.  The bulkhead is scheduled for replacement this winter.

    - The Cascades Island fishway was at orifice flow from 0039 on 01 December until 1000 on 03 December to facilitate the Smith-Root array in the UMT.

      - The north UMT bulkhead was installed at 0729 on 01 December and removed at 0918 on 03 December.

      - The south UMT bulkhead was removed at 0830 on 03 December.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.

      - SDE and SUE were set to 3’ msl at 1337 on 03 December.

      - NUE and SUE were closed and NDE and SDE were full open at 1520 on 03 December.

    - 11 of 12 floating orifice gates (FOG) remain installed at PH2.  One gate was removed for maintenance and replaced with a bulkhead.

      - Stab plates have not yet been installed.  They are scheduled for installation the week of 06 December.

  - Juvenile Fishways.

    - The B2CC remains closed.

    - DSM1 wall and orifice removal continues.

  - Lamprey Fishways.  All LPSs have been dewatered.

    - The Cascades Is. entrance weir and slot filler remain installed.

    - The PH2 NDE ramp remains in the lowered position.

- CONSTRUCTION

  - PH1 Gantry 4.  Crane rails replacement continues.  New crane in FY11.

  - PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal continues.  DSM1 outfall removal may be delayed.

  - TIE crane.  Load testing occurred the week of 29 November.

- HAZMAT, SPILLS AND CLEANUP.   Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 01 September requests BON forebay maintain 71.5’ to 78.0’ msl as a hard constraint and 71.5’ to 77.5’ as a soft constraint from 0001 on 08 September 2009 until 31 March 2010 to support chum spawning below the dam.

  - A teletype sent on 08 September requests that BON operate the single highest priority unit at PH1 (currently Unit 1) as needed.  This is to reduce the risk of BON going outside the 1% efficiency range while supporting the high forebay request for TDA spillwall work.  This request is effective immediately until further notice.

  - A teletype sent on 05 November requests BON tailwater elevation hard constraint of no less than 11.3’ msl during all hours and a soft constraint of 11.3’ to 11.7’, with a target of 11.5’, from 0600 until 1700 daily.  This is effective from 0600 on 06 November until further notice.  This request is to provide necessary flows for chum spawning.

JERRY CARROLL

Acting Operations Manager