### CENWP-OD-B																							08 March 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 10 of 2010, which covers the period from 28 February through 06 March 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 110.2 to 120.6 kcfs.  Daily average project tailwater ranged from 11.4’ to 11.5’ msl.  Daily average powerhouse forebay elevation ranged from 76.5’ to 76.7’ msl.  Daily average river temperature ranged from 43 to 44ºF.  Secchi disk measurements ranged from 6.0’ to 7.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 1.3 to 1.5 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 was on standby from 1057 until 1138, 1358 until 1446, and 1544 until 1637 on 02 March due to issues with the governor.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed this reporting week.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 |  |  | STSs installed |

  - Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week.

- RESEARCH

  - CRITFC- sea lion. Deployed hydrophones on 01 March in BON tailraces to track sea lion activity.

  - FFU- sea lion.  Observations began on 08 January.  Weekly reports of sea lion activities are now available. If you would like to be included on the distribution list contact Robert Stansell-541-374-8801.

  - NOAA- lamprey.  Cascades Island lamprey ramp camera remains installed.

  - ODFW/WDFW- sea lions.  Sea lion traps remain in position.  Trapping began the week of 01 March.

  - PSMFC- smolt monitoring.  Collections began at 0700 on 01 March.  Fallbacks recorded crossing the separator bars included:  one steelhead.

  - Smith-Root- sea lion array.  Array remains installed in the UMT. Testing occurred on 02 March. Preliminary analysis shows no interference with PIT detectors.  DIDSON installation to occur in late March.

  - WDFW- Fish Counting.  Video counting continues at WA shore and Bradford Island.

  - WDFW, ODFW, CRITFC- Sea lion hazing.  Hazing of Steller sea lions has been occurring on a limited basis since mid-January.  Hazing of California sea lions began on 01 March.

  - USDA Wildlife Services- Sea lion hazing.  Hazing from shore began on 03 March, and will occur seven days per week.

- FISHWAYS

  - Project biologists inspected 01-06 March.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI)- nothing to report |  |  |  |  |  |
| PH1 ICE AND TRASH SLUICEWAY |  |  |  |  |  |
| 3/4-5 | ##### PH1 ITS | ##### Chaingate 6C closed | ##### Gate hung up. | ##### Repaired by 3/6 |  |
| CASCADES ISLAND (CI)- nothing to report |  |  |  |  |  |
| WASHINGTON SHORE (WS) – nothing to report |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 2/28 | N/A | N/A |
| 3/1 | 42.0 | N/A |
| 3/2 | 42.0 | N/A |
| 3/3 | 42.0 | N/A |
| 3/4 | 42.2 | N/A |
| 3/5 | 42.4 | N/A |
| 3/6 | 42.4 | N/A |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 03 March.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - Stab plates were removed and replaced with sea lion exclusion bars on 01 March, in 11 of 12 FOGs.  One gate remains out for maintenance, with a bulkhead installed.

    - Sea Lion Exclusion Devices (SLEDs) remain installed at all fishway entrances.

  - Juvenile Fishways.

    - The B2 Corner Collector remains closed. Concrete repairs are complete, and concrete is curing.

    - DSM2 remains in service.

    - The SMF switchgate was placed in sample mode at 0700 on 01 March.

    - The PH1 ice and trash sluiceway (ITS) remains watered up.  The automated chaingates are open and in manual due to issues with the PLC.  No manual chaingates are open at this time due to the inability to raise the bottom leaf of the ITS end gate.  Issues expected to be corrected the week of 08 March.

    - Hydrocannons returned to service on 01 March.

    - TIES were installed on 01 and 02 March.

  - Lamprey Fishways.  All LPSs have been dewatered.

    - The Cascades Is. entrance weir remains installed.

    - The PH2 NDE ramp remains in the lowered position.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in FY11.

  - PH1 Ice and Trash Sluiceway Wall Removal. Wall removal is complete.  Outfall removal has been postponed.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 08 January requests BON forebay maintain 71.5’ to 77.0’ msl as a soft constraint, and 71.5’ to 77.5’ msl as a hard constraint from 0001 on 09 January until 1800 on 31 March.  This is a modification of the 01 September teletype.  This request is to reduce the risk of flooding machinery and twisting bollards at BON locks, while still supporting spillwall construction at TDA.

  - A teletype sent on 08 September requests that BON operate the single highest priority unit at PH1 (currently Unit 1) as needed.  This is to reduce the risk of BON going outside the 1% efficiency range while supporting the high forebay request for TDA spillwall work.  This request is effective immediately until further notice.

  - A teletype sent on 30 December requests BON tailwater elevation be maintained at no less than 11.5’ msl during all hours.  This is effective from 1600 on 30 December until further notice.  This request is to provide necessary flows for chum egg incubation.

  - While conducting fishway inspections, Project biologists observed two kelts on 02 March and three kelts on 04 March in the vicinity of the B2CC bulkhead.

GLEN A. SMITH, P.E.

Operations Project Manager