### CENWP-OD-B																							29  March 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 13 of 2010, which covers the period from 21 through 27 March 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 90.5 to 115.7 kcfs.  Daily average project tailwater ranged from 9.0’ to 10.7’ msl.  Daily average powerhouse forebay elevation ranged from 72.9’ to 75.9’ msl.  Daily average river temperature ranged from 45 to 47ºF.  Secchi disk measurements ranged from 4.0’ to 6.5’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 0.0 to 0.6 kcfs.

    - Spill bays 1 and 18 remained on sill most of the week due to excessive TDG in the river while the PH2 corner collector was in service (see 7.4-7.7).

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed this reporting week.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 |  |  | STSs installed |

  - Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week.

- RESEARCH

  - Battelle – Gatewell velocity testing.  Concluded on 18 March.

  - CRITFC- sea lion. Deployed hydrophones on 01 March in BON tailraces to track sea lion activity.

  - FFU- sea lion.  Observations continue.  Weekly reports of sea lion activities are now available. If you would like to be included on the distribution list contact Robert Stansell-541-374-8801.

  - NOAA- lamprey.  Cascades Island lamprey ramp camera remains installed.

  - PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week:  17 steelhead.  Please see http://www.fpc.org/adultsalmon/bonkeltcounts.htm for updated kelt counts.

  - Smith-Root- sea lion array.  Array remains installed in the UMT.  DIDSON installation to occur in early April.

  - WDFW- Fish Counting.  Video counting continues at WA shore and Bradford Island.  Visual counting scheduled to begin on 01 April.

  - WDFW, ODFW, CRITFC- Sea lion hazing/trapping.  Hazing from boats continues.  Sea lion traps remain in position.  Trapping continues.

  - USDA Wildlife Services- Sea lion hazing.  Hazing from shore continues.

- FISHWAYS

  - Project biologists inspected 21-27 March.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 3/22-23 | ##### PH1 South entrance | ##### Differential <1.0’ | ##### Unknown | ##### Ops adjusted. |  |
| 3/21-27 | ##### Spillbays 1 & 18 | ##### No attraction flow | ##### Excessive gas (TDG) levels | ##### Attraction flow to remain closed until gas levels can be controlled or April 1 gas waiver is initiated |  |
| 3/26 | ##### B-branch entrance | ##### Differential >2.0’ | ##### FV4-4/4-3 in manual | ##### Ops adjusted. |  |
| PH1 ICE AND TRASH SLUICEWAY |  |  |  |  |  |
| 3/21-27 | ##### Sluiceway | ##### Closed | ##### Excessive gas (TDG) levels | ##### Attraction flow to remain closed until gas levels can be controlled or April 1 gas waiver is initiated |  |
| CASCADES ISLAND (CI)- nothing to report |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 3/23 | ##### Weir 38 | ##### Reading <1.0 | ##### Unknown | ##### Ops adjusted. |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 3/21 | 44.1 | 45.1 |
| 3/22 | 44.3 | 45.3 |
| 3/23 | 44.5 | 45.4 |
| 3/24 | 44.7 | 45.6 |
| 3/25 | 45.0 | 46.0 |
| 3/26 | 45.0 | 45.9 |
| 3/27 | 45.1 | 46.0 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 26 March.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - Sea lion exclusion bars remain in 11 of 12 FOGs.  One gate remains out for maintenance, with a bulkhead installed.

    - Sea Lion Exclusion Devices (SLEDs) remain installed at all fishway entrances.

    - The AFF was watered up and placed in bypass on 22 March.

  - Juvenile Fishways.

    - The B2 Corner Collector was open from 0815 on 17 March until 1309 on 23 March, and was reopened at 0711 on 25 March.  A regional agreement was reached to close the PH1 ice and trash sluiceway and the attraction flow from spillbays 1 and 18 in an attempt to lower TDG.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains watered up.  It was closed from 21-27 March in an attempt to lower TDG levels while the B2CC was in service (see 7.4-7.7).

    - Hydrocannon remains in service.

    - TIES remain installed.

  - Lamprey Fishways.  All LPSs are dewatered.

    - The Cascades Is. entrance weir remains installed.

    - The PH2 NDE ramp remains in the lowered position.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in FY11.

  - PH1 Ice and Trash Sluiceway Wall Removal. Wall removal is complete.  Outfall removal has been postponed.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 11 March requests BON forebay elevation be maintained at 71.5’ to 72.5’msl as a hard constraint, and 71.5’ to 72.0’ msl as a soft constraint from 0630 to 1700 on 02 and 03 April.  This request is to allow the Project to clean the latch pockets for the spill gate dogging mechanisms.

  - A teletype sent on 11 March replaces the teletype sent on 23 February.  It requests a gradual lowering of BON tailwater elevation:

    - Effective on 15 March at 0001, BON minimum tailwater elevation is 11.0’ msl.

    - Effective on 17 March at 0001, BON minimum tailwater elevation is 10.8’ msl.

    - Effective on 19 March at 0001, BON minimum tailwater elevation is 10.5’ msl.

    - Effective on 22 March at 0001, BON minimum tailwater limit is lifted.  The project will operate within its normal operating range.

  - A teletype sent on 17 March requests BON to open the corner collector (B2CC) and operate until further notice.  The PH1 sluiceway and attraction flow from spillbays 1 and 18 should be closed.

  - A teletype sent on 23 March requests BON operate the B2CC from 0700 until 1300 hours daily, until further notice.  The PH1 sluiceway and attraction flow from spillbays 1 and 18 should be closed.

  - A teletype sent on 24 March replaces the teletype sent on 23 March (see 7.5).  The Project is requested to operate the PH1 sluiceway and spillbays 1 and 18 as soon as possible.  The B2CC is to be opened as soon as weather conditions permit.  Once the B2CC is open, the sluiceway and spillbays are to be closed.

  - A teletype sent on 25 March replaces the teletype sent on 24 March (see 7.6).  BON project is requested to continue operating with B2CC open, and spillbays 1 and 18 and the PH1sluiceway closed.

GLEN A. SMITH, P.E.

Operations Project Manager