### CENWP-OD-B																							12 April 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 15 of 2010, which covers the period from 04 through 10 April 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 95.9 to 120.1 kcfs.  Daily average project tailwater ranged from 9.4’ to 11.9’ msl.  Daily average powerhouse forebay elevation ranged from 74.5’ to 76.1’ msl.  Daily average river temperature ranged from 46 to 47ºF.  Secchi disk measurements ranged from 5.0’ to 6.5’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 1.5 to 72.3 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse. Powerhouse 1 was priority on 10 April. (See 7. 3)

    - Unit 0 was out of service from 0142 on 06 April until 1620 on 07 April for semi-annual maintenance.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 8 was out of service from 0754 until 1140 on 05 April for oil cleanup.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12-18 was out of service from 0232 until 1630 on 10 April for BPA maintenance.  (See 7.3)

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 was out of service from 0232 until 1652 on 10 April for BPA maintenance.

    - F2 was out of service from 0232 until 1706 on 10 April for BPA maintenance.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed this reporting week.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 26 April |

  - Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week.

- RESEARCH

  - Battelle – survival study.  Hydrophone installation in PH1, PH2, and spillway forebays continues.

  - CRITFC- sea lion. Deployed hydrophones on 01 March in BON tailraces to track sea lion activity.

  - FFU- sea lion.  Observations continue.  Weekly reports of sea lion activities are now available. If you would like to be included on the distribution list contact Robert Stansell-541-374-8801.

  - NOAA- lamprey.  Cascades Island lamprey ramp camera remains installed.

  - PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week:  29 steelhead.  Please see http://www.fpc.org/adultsalmon/bonkeltcounts.htm for updated kelt counts.

  - Smith-Root- sea lion array.  Array remains installed in the UMT.  DIDSON installation occurred on 09 April.  Telemetry receivers were installed at three locations in the UMT on 02 April.

  - WDFW- Fish Counting.  Visual counting continues.

  - WDFW, ODFW, CRITFC- Sea lion hazing/trapping.  Hazing from boats continues.  Sea lion traps remain in position.  Trapping continues.

  - USDA Wildlife Services- Sea lion hazing.  Hazing from shore continues.

  - USDA Wildlife Services- Avian hazing.  Hazing from shore began on 01 April.

  - University of Idaho – lamprey. Collected 14 juvenile lamprey morts from the JMF.

  - University of Idaho - Adult salmon sampling began on 10 April to monitor passage effects at the modified Cascades Island entrance.

- FISHWAYS

  - Project biologists inspected  April 4-10.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 4/8 | ##### A-branch weir | ##### Staff gage > 1.0’ | ##### Unknown | ##### No action. |  |
| CASCADES ISLAND (CI) - Nothing to report |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Nothing to report |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 4/4 | 44.6 | 45.5 |
| 4/5 | 44.8 | 45.7 |
| 4/6 | 45.2 | 46.1 |
| 4/7 | 45.4 | 46.3 |
| 4/8 | 45.5 | 46.5 |
| 4/9 | 45.6 | 46.4 |
| 4/10 | 45.8 | 46.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 09 April.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - Sea lion exclusion bars remain in 11 of 12 FOGs.  One gate remains out for maintenance, with a bulkhead installed.

    - Sea Lion Exclusion Devices (SLEDs) remain installed at all fishway entrances.

    - Sampling at the AFF began this week.

  - Juvenile Fishways.

    - The B2 Corner Collector remains open.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service.

    - Hydrocannon remains in service.

    - TIES are installed.

  - Lamprey Fishways.  All LPSs are dewatered.

    - The Cascades Is. entrance weir remains installed.

    - The PH2 NDE ramp remains in the lowered position.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in FY11.

  - PH1 Ice and Trash Sluiceway Wall Removal. Wall removal is complete.  Outfall removal has been postponed.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is in progress.  Structural modifications scheduled to occur in late summer.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 31 March requests BON operate the B2CC, PH1 sluiceway, and spillbays 1 and 18, beginning at 0001 on 01 April and continuing until further notice.

  - A teletype sent on 09 April requests an outage of PH2 and shifts powerhouse priority to PH1from 0001 until 1800 hours on Saturday, 10 April to facilitate BPA maintenance.

  - A teletype sent on 09 April requests spill for juvenile fish passage begin at 0001 hours on Saturday, 10 April and continue until further notice. Spill approximately 100 kcfs 24 hours a day using the spill pattern in the 2010 FPP.  If Bonneville inflows are too low to maintain the 100 kcfs spill and minimum turbine discharge of 30 kcfs, then operate to minimum discharge and spill to pass inflow.

  - A teletype sent on 09 April requests special operations for Spring Creek and Little White Salmon hatchery releases on 12 April and 13 April respectively.  Beginning 0600 on 13 April until 0559 on 17 April as a hard constraint and from 0600 on 17 April until 0600 on 21 April as a soft constraint, operate PH2 units at the low 1% best efficiency range.

GLEN A. SMITH, P.E.

Operations Project Manager