### CENWP-OD-B																							09 May 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 19 of 2010, which covers the period from 02 through 08 May 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 170.3 to 199.1 kcfs.  Daily average project tailwater ranged from 15.0’ to 17.0’ msl.  Daily average powerhouse forebay elevation ranged from 75.0’ to 76.2’ msl.  Daily average river temperature ranged from 52 to 53ºF.  Secchi disk measurements ranged from 4.0’ to 5.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 89.1 to 99.7 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 13 was out of service from 0300 on 03 May until 1709 on 05 May for head gate replacement.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed this reporting week.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA |  |

  - Dewatering and Fish Salvage.  The following dewatering/fish salvage activities occurred this week:  Unit 13 gatewells were dipped.  Approximately 40 yearling Chinook were released into DSM2 in good condition.  One yearling Chinook mortality was recovered from the dipping basket during the fish salvage.

- RESEARCH

  - Battelle – survival study.  Hydrophone installation in PH1, PH2, and spillway forebays complete. Tagged fish releases upstream have begun for dam passage and survival. Sort by Code operations at the SMF commenced this week to evaluate acoustic tagging effects on juvenile salmonids.

  - CRITFC- sea lion. Deployed hydrophones on 01 March in BON tailraces to track sea lion activity.

  - CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - FFU- sea lion.  Observations continue.  Weekly reports of sea lion activities are now available. If you would like to be included on the distribution list contact Robert Stansell-541-374-8801.

  - NOAA- lamprey.  Cascades Island lamprey ramp camera remains installed.

  - PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week:  88 steelhead, 18 salmon, and 10 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  Two juvenile lamprey collected for University of Idaho researchers this week.

  - WDFW- Fish Counting.  Visual counting continues.

  - WDFW, ODFW, CRITFC- Sea lion hazing/trapping.  Hazing from boats continues.  Sea lion traps remain in position.  Trapping continues.

  - USDA Wildlife Services- Sea lion hazing.  Hazing from shore continues.

  - USDA Wildlife Services- Avian hazing.  Hazing from shore continues.

  - University of Idaho – lamprey.   Two juvenile lamprey collected from PSMFC SMP this reporting week.

  - University of Idaho - Adult salmon sampling continues for monitoring passage effects at the modified Cascades Island entrance.

- FISHWAYS

  - Project biologists inspected 02 – 08 May.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 5/7 | ##### A-Branch | ##### Staff gauge >1.0 | ##### Unknown | ##### No action taken |  |
| 5/8 | ##### Bradford Is. ladder exit | ##### Log on trashrack | ##### River debris | ##### Riggers will clean |  |
| CASCADES ISLAND (CI) – Nothing to report |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 5/6-8 | ##### PH2 entrances | ##### Gates <13.0 ft open | ##### System in manual | ##### Operators adjusted |  |
| 5/8 | ##### WA Shore ladder | ##### Weir 37 < 1.0 | ##### Unknown | ##### No action taken |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - Design of a mechanical velocity meter at the PH2 south monolith is in progress.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 5/2 | 51.1 | 51.7 |
| 5/3 | 51.1 | 51.7 |
| 5/4 | 50.9 | 51.5 |
| 5/5 | 50.9 | 51.6 |
| 5/6 | 50.7 | 51.5 |
| 5/7 | 50.9 | 51.6 |
| 5/8 | 51.2 | 52.1 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 06 May.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - Sea lion exclusion bars remain in 11 of 12 floating orifice gates (FOGs).  The repairs on one FOG have been completed.  Another FOG was removed for maintenance, with a bulkhead installed in its place.

    - Sea Lion Exclusion Devices (SLEDs) remain installed at all fishway entrances.

    - Sampling at the AFF continues.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service.

    - Hydrocannon remains in service.

    - TIES are installed.

  - Lamprey Fishways.  All LPSs are dewatered.

    - The Cascades Is. entrance weir remains installed.

    - The PH2 NDE ramp remains in the lowered position.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in FY11.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is in progress.  Shoring of the walls is scheduled to occur in late summer.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 09 April requests spill for juvenile fish passage begin at 0001 hours on Saturday, 10 April and continue until further notice. Spill approximately 100 kcfs 24 hours a day using the spill pattern in the 2010 FPP.  If Bonneville inflows are too low to maintain the 100 kcfs spill and minimum turbine discharge of 30 kcfs, then operate to minimum discharge and spill to pass inflow.

  - A teletype sent on 07 May requests special operations for the 4.65 million Spring Creek hatchery Chinook releases on 10 May.  Beginning 0600 on 11 May until 0559 on 15 May as a hard constraint, and 0600 on 15 May until 0559 on 19 May as a soft constraint - Operate PH2 as priority with units operating within 5 MW of the low 1% limit as a hard constraint.  PH1 units shall operate within the 1% best efficiency range.

GLEN A. SMITH, P.E.

Operations Project Manager