### CENWP-OD-B																							12 July 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 28 of 2010, which covers the period from 04 through 10 July 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 193.2 to 225.5 kcfs.  Daily average project tailwater ranged from 16.7’ to 18.4’ msl.  Daily average powerhouse forebay elevation ranged from 73.9’ to 76.0’ msl.  Daily average river temperature ranged from 63 to 66ºF.  Secchi disk measurements ranged from 4.0’ to 6.5’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 89.4 to 100.3 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 3 was out of service from 0001 on 06 July until 1757 on 08 July for annual maintenance.

    - Unit 9 has remained out of service since 04 August 2008 for turbine rehab.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 13 went out of service at 0733 on 14 June for four-year overhaul.

    - Unit 16 was out of service from 0915 until 1102 on 08 July for governor oil pressure regulator repair.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed this reporting week.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week:

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 26 July. |

  - Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week.

- RESEARCH

  - Battelle – survival study.  Hydrophone installation in PH1, PH2, and spillway forebays complete. Tagged fish releases upstream continue for dam passage and survival evaluations. Sort by Code operations at the SMF continued this week to evaluate acoustic tagging effects on juvenile salmonids.

  - CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - NOAA- lamprey.  PIT tagging adult lamprey in the AFF continues.

  - PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week:  3 steelhead, 95 salmon, and 53 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - WDFW- Fish Counting.  Visual counting continues.  Night counts continue.

  - USDA Wildlife Services- Avian hazing.  Hazing from shore continues.

  - University of Idaho – lamprey.   Radio and PIT tagging adult lamprey in the AFF.

  - University of Idaho - Adult salmon sampling concluded on 30 June.

- FISHWAYS

  - Project biologists inspected 04-10 July.  NOAA inspected on 09 July.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 7/4 | ##### PH1 SCC entrance | ##### Differential < 1.0’ | ##### Unknown | ##### Operators adjusted |  |
| 7/7 | ##### B-branch entrance | ##### Differential < 1.0’ | ##### System in manual | ##### Operators adjusted |  |
| CASCADES ISLAND (CI) Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 7/4, 6, 8 | ##### PH2 entrance gates | ##### Submerged <13’ | ##### In manual | ##### Operators adjusted |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

    - Temperatures are unavailable this week but are expected to return next week.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 7/4 | N/A | N/A |
| 7/5 | N/A | N/A |
| 7/6 | N/A | N/A |
| 7/7 | N/A | N/A |
| 7/8 | N/A | N/A |
| 7/9 | N/A | N/A |
| 7/10 | N/A | N/A |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 09 July.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - Sea lion exclusion bars remain in 11 of 12 floating orifice gates (FOGs).  One FOG remains out for maintenance, with a bulkhead installed in its place.

    - Sampling at the AFF continues.  The downstream exit bulkhead remained in the lowered position after sampling this week to keep the ladder in criteria and shad off the weir 37 bleed off valve.  The automatic blow down function for the weir 37 bleed off valve was programmed on 19 May to operate every six hours.

    - WA shore and Cascades Is. ladders returned to salmon passage mode on 07 July and Bradford Is. on 08 July.

    - Operators continue to maintain 0.5’ of head at the PH2 fishway entrances for lamprey passage during night hours.  Table Bon-5 of the FPP is being utilized as a guideline.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Hydrocannon remains in service.

    - TIES remain installed.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains in service. Scheduled to operate Sunday night through Friday.

    - The PH2 NDE ramp remains in the lowered position and in service.

    - Bradford Island and WA Shore LPS’s remain in service.

    - B2CC was closed from 0710 until 1110 on 08 July to support transducer repair in the PH2 forebay (see 7.7).

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in FY11.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is in progress.  Shoring of the walls is scheduled to begin on 19 July.

- HAZMAT, SPILLS AND CLEANUP.  Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 22 June requests Bonneville operate PH2 units within 5 MW of the mid to lower end of the 1% efficiency range (approx. 14 kcfs or less), hard constraint, effective immediately.  This operation was requested to reduce high levels of fish descaling observed at the Juvenile Monitoring Facility.  This operation originally began on 16 June.

  - A teletype sent on 28 June requests that Bonneville forebay be operated for the treaty fishery as follows:  0600 Tuesday, 06 July until 1800 Thursday, 08 July – operate within 1.5’ band, hard constraint.  This is to limit pool fluctuation and reduce wear and tear on equipment.

  - A teletype sent on 29 June requests Bonneville forebay be operated from 1000-1800 each day on 9-11 July as follows: maintain 73.0-74.0’ msl, soft constraint.  This operation was requested to support a kite board event at the Port of Hood River.

  - A teletype sent on 01 July requests Bonneville forebay be operated from 2000-2300 on 04 July as follows: 75.8-76.5’ msl, hard constraint, 76.0-76.5’ msl, soft constraint.  This operation was requested to support the Hood River 4th of July fireworks show. At all other times from 03 – 05 July during daylight hours, it is requested Bonneville maintain a lower pool to benefit recreational users.

  - A teletype sent on 02 July requests BON project begin the summer spill test effective immediately until 0430 on 20 July.  Two treatments will be used in a block test: 85kcfs day/121 kcfs night and 95 kcfs for 24 hours.  The teletype also requests a continuation of the current PH2 operation (see 7.2) until Tuesday 06 July at 0900.

  - A teletype sent on 02 July requests BON Project close the PH2 corner collector (B2CC) on Wednesday 07 July from 0700-1100.  If the B2CC is not closed on Wednesday, the project can make another attempt to close it on 08 July from 0700-1100.  Operate MU 15 at the low end of the 1% efficiency range.  This operation is to support the repair of PH2 forebay transducers.

  - A teletype sent on 06 July requests BON maintain discharge about 220 kcfs, soft constraint, from 0400 to 0700 on 07 July.  This operation was requested to assist in recovery of a grounded boat at Rooster Rock.

  - A teletype sent on 08 July requests BON maintain discharge about 220 kcfs, soft constraint, from 0400 to 0700 on 09 July.  This operation was requested to assist in recovery of a grounded boat at Rooster Rock.

  - A teletype sent on 09 July requests that Bonneville forebay be operated for the treaty fishery as follows:  0600 Tuesday, 13 July until 1800 Thursday, 15 July – operate within 1.5’ band, hard constraint.  This is to limit pool fluctuation and reduce wear and tear on equipment.

  - A teletype sent on 09 July requests Bonneville forebay be operated from 1200-1800 each day on 16-18 July as follows: maintain 73.0-74.0’ msl, soft constraint.  This operation was requested to support a kite board event at the Port of Hood River.

GLEN A. SMITH, P.E.

Operations Project Manager