### CENWP-OD-B																						06 December 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 49 of 2010, which covers the period from 28 November through 04 December 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 132.6 to 149.4 kcfs.  Daily average project tailwater ranged from 12.4’ to 13.7’ msl.  Daily average powerhouse forebay elevation ranged from 73.3’ to 74.4’ msl.  Daily average river temperature ranged from 46 to 48ºF.  Secchi disk measurements ranged from 5.0 to 7.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 0.5 to 1.1 kcfs.

    - Bay 1 was placed on seal at 1701 on 30 November to facilitate Cascades Island fishway going to orifice flow on 01 December, in preparation for dewatering on 03 December.  It will remain on seal while the CI fishway is out of service for maintenance.

  - Unit Operation.  Powerhouse One became the priority powerhouse on 01 December to facilitate the Washington Shore fishway going to orifice flow on 01 December.

    - Unit 1 has remained out of service since 17 September due to sticking wicket gates.

    - Unit 2 has remained out of service since 05 October for five year overhaul and U1/U2 efficiency testing.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 has remained out of service since 13 September for four year overhaul.

    - Unit 13 was out of service from 0005 until 1650 on 01 December due to T11 outage.

    - Unit 14 was out of service from 0006 until 1650 on 01 December due to T11 outage.

    - Unit 15 was out of service from 0132 until 1400 on 02 December due to T12 outage.

    - Unit 16 was out of service from 0132 until 1400 on 02 December due to T12 outage.

    - Unit 17 was out of service from 0132 until 1400 on 02 December due to T12 outage.

    - Unit 18 was out of service from 0132 until 1400 on 02 December due to T12 outage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 went out of service at 2025 on 30 November to facilitate the Washington Shore fishway going to orifice flow on 01 December, in preparation for dewatering on 03 December.

    - F2 has remained out of service since 15 November for annual maintenance and headgate rehab.  PH2 fishway entrances and PH2 collection channel are configured for single fish unit operation.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 1-1 was closed for cleaning from 0737 until 0750 on 30 November.  FV 1-1 was closed to float debris from 1800 on 27 November until 0645 on 28 November, from 1830 on 28 November until 0737 on 29 November, and from 2210 on 04 December until 0205 on 05 December.

    - FV 3-7 was closed for cleaning from 0700 until 0734 and from 0830 until 0929 on 24 November.  FV 3-7 was closed to float debris from 1830 on 28 November until 0737 on 29 November, and from 2210 on 04 December until 0205 on 05 December.

    - FV 3-9 was closed for cleaning from 0824 until 0842 on 30 November, from 0758 until 0807 on 01 December, from 0730 until 0745 on 02 December, and from 0714 until 0734 on 03 December.  FV 3-9 was closed to float debris from 1800 on 27 November until 0645 on 28 November, from 1830 on 28 November until 0737 on 29 November, and from 2210 on 04 December until 0205 on 05 December.

    - FV 5-3 and FV 5-4 were closed at 1920 on 30 November for winter maintenance.

    - FV 5-9 was placed in manual at 1920 on 30 November, and was closed at 0955 on 03 December for winter maintenance.

    - FV 6-9 was closed at 0611 on 01 December for winter maintenance.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Screens scheduled to be removed after 15 December. |

  - Dewatering and Fish Salvage.

    - On 03 December, project biologists and volunteers dewatered the WA Shore ladder, Cascades Island ladder, and Upstream Migration Tunnel (UMT).   33 coho, 30 adult steelhead, 10 adult chinook, 115 suckers, 92 peamouth, 15 northern pikeminnow, two sculpin, and one sturgeon were removed from the WA shore ladder.  10 adult steelhead, two adult coho, five juvenile chinook, 100 peamouth, 35 northern pikeminnow, and three suckers were removed from the Cascades Island ladder.     20 adult steelhead, 10 adult chinook, eight juvenile chinook, four juvenile steelhead, 115 suckers, 37 peamouth, 15 northern pikeminnow, one sculpin, one bass, and one brown trout were removed form the UMT.  All were released at the Hamilton Island boat launch in good condition with the exception of a few salmonids from WA shore ladder.  One chinook, one steelhead, and an estimated six to eight coho did not survive.  Please see Bon Fisheries MFR 06 Dec 2010.

- RESEARCH

  - WDFW- Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 28 November through 02 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion |  | ##### FPP itEMS |  | ##### CAUSE | ##### RESPONSE |  | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Once | ##### Bonneville |  | ##### Calibration not checked |  | ##### Scheduling conflict | ##### Will be checked week of 05 December |  |  |  |
| BRADFORD ISLAND (BI) |  |  |  |  |  |  |  |  |  |
| 11/30 |  | FV 3-9 trashrack |  | Drawdown > 0.5’ | Debris |  | Cleaned |  |  |
| CASCADES ISLAND (CI) |  |  |  |  |  |  |  |  |  |
| 11/28 | ##### UMT weir |  | ##### Reading < 1.0’ |  | ##### Unknown | ##### No action taken. |  |  |  |
| WASHINGTON SHORE (WS) – Nothing to report |  |  |  |  |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.

    - The PH1 PCC mechanical velocity meter remains out for maintenance crews to use as a template while constructing a new one.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 11/28 | 46.9 | NA |
| 11/29 | 46.7 | NA |
| 11/30 | 45.9 | NA |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was not checked this week.

    - The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.

    - Stab plates were installed in 11 of 12 floating orifice gates (FOGs) on 15-17 November due to single fish unit operations.  One FOG remains out for maintenance, with a bulkhead installed in its place.

    - The Washington Shore and Cascades Island fish ladders were set at orifice flow on 01 December, and were dewatered to tailwater on 03 December for winter maintenance.

    - The Bradford Island fish exit was closed for cleaning from 0758 until 0807 on 01 December and from 0730 until 0745 on 02 December.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - B2CC remains closed for the season.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Chain gate 6C was closed and 7A opened on 04 August.  Chain gate 7A is set to 72’ msl.  After 01 December all chain gates will be closed except 1A, 1B, and 10B (see section BON 2.5.1.1.m. in the 2010 FPP).

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.

    - The PH2 NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service for the season.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in Spring, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls within 50’ of the Cascades Island fishway resumed on 08 November.

  - BGS removal. Scheduled to begin on 06 December.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 14 October requests BON project change PH1 unit priority as follows: 1, 10, 9, 3,7,2,6,4,5,8, effective immediately until further notice. This moves Unit 9 to third in priority, and replaces the teletype sent on 29 July that moved Unit 7 to third in priority. This operation is necessary to complete a 100 day operation per PH1 rehab contract.

  - A teletype sent on 19 November requests the following BON tailwater elevations for chum spawning:

    - Effective immediately until further notice, hard constraint of no less than 11.3’ msl during all hours, and a hard constraint of 11.3’ to 11.7’ with a target of 11.5’, from 0630 until 1700 daily.

    - If needed, BON is authorized to pass water in excess of what is necessary to meet the 11.3’ to 11.7’ hard constraint by increasing tailwater elevation to 18.5’ msl between 1700 and 0600 hours.  Normal ramp rates apply.  It is preferred to concentrate the highest tailwater around midnight without exceeding 16.5’ msl.

  - A teletype sent on 01 December requests limited generation at Bon 2nd powerhouse for BGS

removal.  Limit generation to two units from 0500 to 1800, Monday through Friday, beginning 06 December 2010 and ending 07 January 2011.  Dive safety will determine if it is safe to operate a 3rd unit and unit priority to achieve safe dive conditions.  Spill is authorized to maintain chum tailwater elevation requirements.

GLEN A. SMITH, P.E.

Operations Project Manager