### CENWP-OD-B																						13 December 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 50 of 2010, which covers the period from 05 December through 11 December 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 132.9 to 138.5 kcfs.  Daily average project tailwater ranged from 12.4’ to 13.8’ msl.  Daily average powerhouse forebay elevation ranged from 72.0’ to 74.5’ msl.  Daily average river temperature ranged from 43 to 45ºF.  Secchi disk measurements ranged from 5.0 to 7.0’.

  - Spill Bay 18, which provides attraction flow for B-branch, is open 6” during daylight hours.  Daily average spill ranged from 0.4 to 3.2 kcfs.

  - Unit Operation.  Powerhouse One is the priority powerhouse.

    - Unit 1 has remained out of service since 17 September due to sticking wicket gates.

    - Unit 2 returned to service at 1029 on 07 December.  It had been out of service 05 October for five year overhaul and U1/U2 efficiency testing.

    - Unit 7 was out of service from 0016 on 06 December until 0945 on 09 December due to contract work.

    - Unit 9 was out of service from 0102 until 1234 on 11 December for post rehab work.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 has remained out of service since 13 September for four year overhaul.

    - Unit 13 was out of service from 0500 until 1707 on 07 December, from 0500 until 1650 on 08 December, from 0400 until 1706 on 09 December, and from 0404 until 1711 on 10 December for BGS removal.

    - Unit 14 was out of service from 0500 until 1707 on 07 December, from 0500 until 1650 on 08 December, from 0400 until 1706 on 09 December, and from 0404 until 1714 on 10 December for BGS removal.

    - Unit 15 was out of service from 0500 until 0613 on 07 December, from 0500 until 1650 on 08 December, from 0400 until 1706 on 09 December, and from 0404 until 1730 on 10 December for BGS removal.

    - Unit 16 was out of service from 0500 until 1747 on 07 December, from 0744 until 1650 on 08 December, from 0400 until 1706 on 09 December, and from 0404 until 1733 on 10 December for BGS removal.

    - Unit 18 was out of service from 1920 until 1934 on 10 December due to a high VBS differential reading, sensor was found to be inaccurate.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 and F2 remain out of service while the WS fishway is dewatered.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 1-1 was closed for cleaning from 0752 until 0810 on 08 December, from 0736 until 0820 on 09 December, from 1014 until 1056 on 10 December, and from 0733 until 0750 on 11 December.  FV 1-1 was closed to float debris from 2210 on 04 December until 0205 on 05 December, from 0615 until 0758 on 06 December, from 0030 until 0220 on 07 December, from 2130 on 07 December until 0020 on 08 December, and from 2125 on 11 December until 0428 on 12 December.

    - FV 3-7 was closed for cleaning from 0800 until 1000 on 10 December.  FV 3-7 was closed to float debris from 2210 on 04 December until 0205 on 05 December, from 0615 until 1126 on 06 December, and from 2125 on 11 December until 0428 on 12 December.

    - FV 3-9 was closed for cleaning from 0713 until 0758 on 06 December, from 0740 until 0812 on 07 December, from 0715 until 0747 on 08 December, from 0707 until 0731 on 09 December, from 0759 until 0810 on 10 December, and from 0712 until 0730 on 11 December.  FV 3-9 was closed to float debris from 2210 on 04 December until 0205 on 05 December.

    - FV 5-3, 5-4, and 5-9 remain closed while the Cascades Island fishway is dewatered.

    - FV 6-9 remains closed while the WS fishway is dewatered.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Screens scheduled to be removed the week of 20 December. |

  - Dewatering and Fish Salvage.

    - On 06 December Project Biologists recovered four channel catfish, seven sculpin, and 25 sturgeon from Unit 12 tail logs.  All were released at the Hamilton Island boat launch in good condition.

    - On 09 December Project Biologists and maintenance crews recovered approximately 900-1,000 sturgeon, 12 coho, and one bass from the PH2 south monolith.  All were released to the tailrace near the SUE in good condition.

- RESEARCH

  - WDFW- Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 06 through 08 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS |  | ##### CAUSE | ##### RESPONSE |  | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12/6 | ##### Spillway | ##### Bay 18 closed |  | ##### Negligence | ##### Ops adjusted |  |  |  |
| BRADFORD ISLAND (BI) |  |  |  |  |  |  |  |  |
| 12/6 | FG 3-26 |  | Incorrect position | PLC error |  | Ops adjusted |  |  |
| 12/7-8 | FV 1-1 trashrack |  | Drawdown > 0.5’ | Debris |  | Riggers cleaned |  |  |
| CASCADES ISLAND (CI) – Nothing to report |  |  |  |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Nothing to report |  |  |  |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.

    - The PH1 PCC mechanical velocity meter remains out for maintenance crews to use as a template while constructing a new one.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was conducted on 10 December.

    - The Washington Shore and Cascades Island fish ladders remain dewatered.

    - Stab plates or bulkheads remain installed in the floating orifice gates (FOGs) while the WS fishway remains dewatered. One FOG remains out for maintenance.

    - The Bradford Island fish exit was closed for cleaning from 0713 until 0758 on 06 December, from 0740 until 0812 on 07 December, from 0707 until 0731 on 09 December, and from 0759 until 0810 on 10 December.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - B2CC remains closed for the season.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Chain gate 7A and 3B are scheduled to be closed the week of 13 December.  1A, 1B, and 10B will remain open (see section BON 2.5.1.1.m. in the 2010 FPP).

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.

    - The PH2 NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service for the season.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in Spring, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls continues.

  - BGS removal. Construction began on 07 December.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 14 October requests BON project change PH1 unit priority as follows: 1, 10, 9, 3,7,2,6,4,5,8, effective immediately until further notice. This moves Unit 9 to third in priority, and replaces the teletype sent on 29 July that moved Unit 7 to third in priority. This operation is necessary to complete a 100 day operation per PH1 rehab contract.

  - A teletype sent on 19 November requests the following BON tailwater elevations for chum spawning:

    - Effective immediately until further notice, hard constraint of no less than 11.3’ msl during all hours, and a hard constraint of 11.3’ to 11.7’ with a target of 11.5’, from 0630 until 1700 daily.

    - If needed, BON is authorized to pass water in excess of what is necessary to meet the 11.3’ to 11.7’ hard constraint by increasing tailwater elevation to 18.5’ msl between 1700 and 0600 hours.  Normal ramp rates apply.  It is preferred to concentrate the highest tailwater around midnight without exceeding 16.5’ msl.

  - A teletype sent on 07 December revises the teletype sent on 01 December and requests limited generation at PH2 for BGS removal.  Limit PH2 flow to a maximum of 32 kcfs using up to three units from 0500 to 1800, Monday through Friday, beginning 08 December 2010 and ending 07 January 2011.  Unit priority for these flows should be 17, 18, and 16.  Dive safety will determine if it is safe to operate a 3rd unit and unit priority to achieve safe dive conditions.  Spill is authorized to maintain chum tailwater elevation requirements.

  - A teletype sent on 10 December outlines a forebay exceedence above the normal 76.5’ msl.  From noon on 11 December until noon on 18 December, the maximum forebay elevation is 77’ msl.  This operation is needed to meet tailwater elevation for chum (see 7.3) with the forecasted high inflows and rising Willamette flows.

GLEN A. SMITH, P.E.

Operations Project Manager