### CENWP-OD-B																						27 December 2010

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 52 of 2010, which covers the period from 19 December through 25 December 2010.

- OPERATION SUMMARY

  - Daily average river flows ranged from 127.9 to 141.4 kcfs.  Daily average project tailwater ranged from 12.8’ to 14.5’ msl.  Daily average powerhouse forebay elevation ranged from 72.1’ to 74.8’ msl.  Daily average river temperature ranged from 41 to 43ºF.  Secchi disk measurements ranged from 5.0 to 7.0’.

  - Spill Bay 18, which provides attraction flow for B-branch, is open 6” during daylight hours.  Daily average spill ranged from 0.5 to 1.7 kcfs.

  - Unit Operation.  Powerhouse One is the priority powerhouse.

    - Unit 1 has remained out of service since 17 September due to sticking wicket gates.

    - Unit 2 returned to service at 1619 on 20 December.  It had been out of service since 12 December due to a Kaplan oil leak, contained in the powerhouse.

    - Unit 9 returned to service at 1700 on 21 December.  It had been out of service since 17 December due to a Kaplan oil leak, contained in the powerhouse.  Unit 9 went out of service at 0810 on 22 December due to a turbine bearing oil leak.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 was out of service from 0238 until 1754 on 20 December due to governor control issues.  Unit 12 was out of service from 0430 until 1500 on 21 December for BGS removal.

    - Unit 13 was out of service from 0430 on 20 December until 1716 on 21 December for BGS removal and a blown fuse.  Unit 13 was out of service from 0430 until 1003 on 22 December for breaker testing.

    - Unit 14 was out of service from 0430 until 1700 on 20 December, and from 0430 until 1515 on 21 December for BGS removal.

    - Unit 15 was out of service from 0430 until 1656 on 20 December, and from 0430 until 1515 on 21 December for BGS removal.

    - Unit 16 was out of service from 0430 until 1515 on 21 December for BGS removal.

    - Unit 17 was out of service from 0430 until 1658 on 20 December for BGS removal.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 and F2 remain out of service while the WS fishway is dewatered.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 1-1 was closed for cleaning from 0756 until 0808 on 20 December, from 0742 until 0752 on 21 December, and from 0720 until 0740 on 22 December.  FV 1-1 was closed to float debris from 2300 on 22 December until 0245 on 23 December.

    - FV 3-9 was closed for cleaning from 0721 until 0750 on 20 December, and from 0723 until 0735 on 21 December

    - FV 5-3, 5-4, and 5-9 remain closed while the Cascades Island fishway is dewatered.

    - FV 6-9 remains closed while the WS fishway is dewatered.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STSs scheduled to be reinstalled the week of 22 February. |

  - Dewatering and Fish Salvage.  The following dewatering or fish salvage activities occurred this reporting week:

    - DSM2 was dewatered on 21 December.  No fish were in the channel.

- RESEARCH

  - WDFW- Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 21 through 23 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS |  | ##### CAUSE | ##### RESPONSE |  | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |  |  |  |
| 12/22 | B-branch entrance |  | Reading >2.0’ | PLC malfunction |  | Ops adjusted |  | 11-51508 |
| CASCADES ISLAND (CI) – Dewatered |  |  |  |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Dewatered |  |  |  |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The PH1 PCC mechanical velocity meter remains out for maintenance crews to use as a template while constructing a new one.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 23 December.

    - The Washington Shore and Cascades Island fish ladders remain dewatered.

    - Stab plates or bulkheads remain installed in the floating orifice gates (FOGs) while the WS fishway remains dewatered. One FOG remains out for maintenance.

  - Juvenile Fishways.

    - DSM2 orifice lights were turned on 20 December.  The channel was dewatered on 21 December.

    - B2CC remains closed for the season.  It was open from 1120 until 1130 on 22 December to test the new boat exclusion boom in the PH2 forebay.  It is scheduled to open briefly on 27 December again to test the boat exclusion boom.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Chain gate 1A, 1B, and 10B remain open (see section BON 2.5.1.1.m. in the 2010 FPP).

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.

    - The PH2 NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service for the season.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in Spring, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls continues.

  - BGS removal. Completed the week of 20 December.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.

  - A teletype sent on 14 October requests BON project change PH1 unit priority as follows: 1,10,9,3,7,2,6,4,5,8, effective immediately until further notice. This moves Unit 9 to third in priority, and replaces the teletype sent on 29 July that moved Unit 7 to third in priority. This operation is necessary to complete a 100 day operation per PH1 rehab contract.

  - Teletypes sent on 15 December revise the teletype sent on 07 December and requests limited generation at PH2 for BGS removal.  Limit PH2 flow to 2 units from 0500 to 1700 daily on 16, 17, 20, and 21 December.  Unit priority for these flows should be 17, 18.  Diving may be terminated at 1500 on 16 and 17 December if high flow conditions continue.  Spill is authorized to maintain chum tailwater elevation requirements.

  - A teletype sent on 20 December replaces the previous chum tailwater teletypes:

    - Effective immediately until further notice, hard constraint of no less than 12.1’ msl during all hours, and a hard constraint of 12.1’ to 12.5’ with a target of 12.3’, from 0630 until 1700 daily.

    - If needed, BON is authorized to pass water in excess of what is necessary to meet the 12.1’ to 12.5’ hard constraint by increasing tailwater elevation to 18.5’ msl between 1700 and 0630 hours.  Normal ramp rates apply.  It is preferred to concentrate the highest tailwater around midnight without exceeding 16.5’ msl.

    - If the actions above are not sufficient, tailwater elevations may increase incrementally by 0.5’, as necessary, up to 13.5’ msl between 0630 and 1700 hours, daily.  The bottom of the range may become the new tailwater minimum.

    - From 0630 until 1300 on 21 December, BON is requested to maintain a tailwater of 11.3’ to 11.7’ with a target of 11.5’, and decrease outflow to 70 kcfs until 2400 on 21 December.  This operation is necessary to remove a fish trap in Duncan Creek.

  - A teletype sent on 21 December requests BON open the B2CC between 0900 and 1500 on 22 December for approximately two hours to test the new boat exclusion boom in front of the B2CC.

  - A teletype sent on 22 December outlines new tailwater operations for chum incubation.  Effective immediately, BON is requested to maintain a tailwater of 12.2’ or greater for all hours.

  - A teletype sent on 22 December notifies BON that slow roll start-up for units that have been idle for 12 hours or more is required from 01 December through 30 April (see FPP section BON 5.7).

  - A teletype sent on 22 December requests BON open the B2CC between 0900 and 1500 on 27 December for approximately two hours to test the new boat exclusion boom in front of the B2CC.

  - A teletype sent on 22 December cancels the BGS teletype (see 7.3), effective at 1400 on 22 December due to completion of the BGS removal work in the PH2 forebay.

GLEN A. SMITH, P.E.

Operations Project Manager