### CENWP-OD-B																						14 February 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 06 of 2011, which covers the period from 06 through 13 February 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 163.4 to 234.1 kcfs.  Daily average project tailwater ranged from 14.6’ to 18.9’ msl.  Daily average powerhouse forebay elevation ranged from 72.2’ to 73.0’ msl.  Daily average river temperature ranged from 38 to 40ºF.  Secchi disk measurements ranged from 2.0 to 5.5’.

  - Spill Bay 18, which provides attraction flow for B-branch, is open 6” during daylight hours.  Daily average spill ranged from 0.6 to 0.8 kcfs.

  - Unit Operation.  Powerhouse One is the priority powerhouse.

    - Unit 1 has remained out of service since 17 September 2010 due to sticking wicket gates.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 and F2 remain out of service while the WS fishway is dewatered.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 3-9 was closed for cleaning from 0721 until 0734 on 07 February and 0702 until 0735 on 11 February.

    - FV 5-3, 5-4, and 5-9 remain closed while the Cascades Island fishway is dewatered.

    - FV 6-9 remains closed while the WS fishway is dewatered.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STSs scheduled to be reinstalled the week of 22 February. |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this week.

- RESEARCH

  - FFU - Sea lion.  Observations began on 07 January.

  - ODFW - Sea lion trapping. Moved sea lion traps adjacent to the B2CC and downstream of the old Navlock on 27 January.  Trapping has not begun for the season.

  - WDFW - Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 08 through 11 February.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) – No items to report |  |  |  |  |  |
| CASCADES ISLAND (CI) – Dewatered |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Dewatered |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The PH1 PCC mechanical velocity meter remains out for maintenance crews to use as a template while constructing a new one.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 11 February.

    - The Washington Shore and Cascades Island fish ladders remain dewatered.

    - Stab plates or bulkheads remain installed in the floating orifice gates (FOGs) while the WS fishway remains dewatered.

    - SLEDs remain installed at PH1 and B-branch entrances.

  - Juvenile Fishways.

    - DSM2 returned to service from 0200 until 1330 on 11 February. It was shutdown due to water from DSM2 leaking into AWS through the emergency relief conduit.  Since maintenance activities are ongoing in the AWS, DSM2 was taken back out of service until repair work is complete.

    - B2CC remains closed for the season.  The B2CC was open from 0731 until 1547 on 09 February to flush debris (see 7.5).

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Chaingates 1A and 1B remain open. Chaingates 4A and 7A were opened on 22 January to assist in passing large quantities of debris that accumulated in the forebay following heavy rains. 10B was removed for maintenance on 10 February.  10A is open in its place.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.

    - The PH2 NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service for the season.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in June, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December outlines new tailwater operations for chum incubation.  Effective immediately, BON is requested to maintain a tailwater of 12.2’ or greater for all hours.

  - A teletype sent on 22 December notifies BON that slow roll start-up for units that have been idle for 12 hours or more is required from 01 December through 30 April (see FPP section BON 5.7).

  - A teletype sent on 03 February outlines BON unit priority.  Effective immediately until further notice, BON unit priority is: 1,10,3,6,2,4,5,8,7,9,11,18,12,17,13,14,15,16.

  - A teletype sent on 08 February outlines the operation of the B2CC on 09 February from 0700 to 1630.  If sufficient debris has passed prior to 1630, a call will be made to close the B2CC.  If necessary, the B2CC can be opened on 10 February from 0700 to 1630.  This operation is necessary to flush debris in the PH2 forebay.

  - Project Fisheries sent out two memorandums for the record on 07 and 08 February regarding a total of four sturgeon mortalities associated with Unit 1 testing and units 15 and 16 start up.

GLEN A. SMITH, P.E.

Operations Project Manager