### CENWP-OD-B																						22 February 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 07 of 2011, which covers the period from 13 through 19 February 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 190.3 to 244.0 kcfs.  Daily average project tailwater ranged from 16.6’ to 19.6’ msl.  Daily average powerhouse forebay elevation ranged from 71.7’ to 72.9’ msl.  Daily average river temperature ranged from 38 to 39ºF.  Secchi disk measurements ranged from 4.0 to 5.2’.

  - Spill Bay 18, which provides attraction flow for B-branch, is open 6” during daylight hours.  Daily average spill ranged from 0.7 to 42.5 kcfs.

  - Unit Operation.  Powerhouse One is the priority powerhouse.

    - Unit 1 has remained out of service since 17 September 2010 due to sticking wicket gates. It was returned to service from 1340 until 1705 on 14 February, from 0745 until 0858 on 15 February, from 1711 until 1754 on 16 February to test the wicket gates.

    - Unit 3 was out of service from 1216 until 1559 on 16 February for raking trash.

    - Unit 5 was out of service from 0836 until 1213 on 16 February for raking trash.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 was out of service from 0609 until 1744 on 17 February for SJ2 replace testing; from 0834 until 0944 for 20 MW load rejection test; from 1015 until 1021 and from 1026 until 1034 on 18 February for air gap testing; and finally from 1111 until 1617 on 18 February for T-11 and T-12 outages.

    - Unit 13 was out of service from 0602 until 1803 on 17 February for SJ2 replace testing, and from 1107 until 1617 on 18 February for T-11 and T-12 outages.

    - Unit 14 was out of service from 0556 until 1807 on 17 February for SJ2 replace testing, and from 1102 until 1617 on 18 February for T-11 and T-12 outages.

    - Unit 15 was out of service from 0854 until 1702 on 14 February and from 0656 until 1722 on 15 February for T12 outage; from 1356 until 1447 on February 16 for T-12 CT testing; from 0921 until 1803 on 17 February for SJ2 replace testing; and from 1055 until 1617 on 18 February for T-11 and T-12 outages.

    - Unit 16 was out of service from 0859 until 1703 on 14 February and from 0702 until 1735 on 15 February for T12 outage; from 1358 until 1457 on February 16 for T-12 CT testing; from 0926 until 1800 on 17 February for SJ2 replace testing; and from 1054 until 1617 on 18 February for T-11 and T-12 outages.

    - Unit 17 was out of service from 0903 until 1732 on 14 February and from 0706 until 1730 on 15 February for T12 outage; from 1407 until 1455 on February 16 for T-12 CT testing; from 0928 until 1757 on 17 February for SJ2 replace testing; and from 1109 until 1617 on 18 February for T-11 and T-12 outages.

    - Unit 18 was out of service from 0910 until 1705 on 14 February and from 0707 until 1731 on 15 February for T12 outage; from 1411 until 1459 on February 16 for T-12 CT testing; from 0936 until 1751 on 17 February for SJ2 replace testing; and from 1114 until 1617 on 18 February for T-11 and T-12 outages.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 and F2 remain out of service while the WS fishway is dewatered.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 3-7 was closed for cleaning from 0726 until 0805 on 14 February and 0706 until 0729 on 16 February.

    - FV 3-9 was closed for cleaning from 0713 until 0730 on 14 February and 0706 until 0729 on 16 February.

    - FV 5-3, 5-4, and 5-9 remain closed while the Cascades Island fishway is dewatered.

    - FV 6-9 remains closed while the WS fishway is dewatered.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STSs scheduled to be reinstalled the week of 22 February. |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this week.

- RESEARCH

  - FFU - Sea lion.  Observations began on 07 January.

  - NOAA – lamprey.

  - ODFW - Sea lion trapping. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Trapping has not begun for the season.

  - WDFW - Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 14 through 18 February.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE |  | ##### TR# |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |  |  |
| 02/17-18 | PH1CC | Differentials < 1.0’ | FV 1-1 needs recalibration |  | Valve in manual. |  | 11-92231 |
| CASCADES ISLAND (CI) – Dewatered |  |  |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Dewatered |  |  |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The PH1 PCC mechanical velocity meter remains out for maintenance crews to use as a template while constructing a new one.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 18 February.

    - The Washington Shore and Cascades Island fish ladders remain dewatered.

    - Stab plates or bulkheads remain installed in the floating orifice gates (FOGs) while the WS fishway remains dewatered.

    - SLEDs remain installed at PH1 and B-branch entrances.

  - Juvenile Fishways.

    - DSM2 returned to service at 0858 on 17 February, but remains in bypass at the JMF.

    - B2CC remains closed for the season.  The B2CC was open from 0731 until 1547 on 09 February to flush debris (see 7.5).

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Chaingates 1A and 1B remain open. Chaingates 4A and 7A were opened on 22 January to assist in passing large quantities of debris that accumulated in the forebay following heavy rains. 10B was removed for maintenance on 10 February.  10A is open in its place.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.

    - The I-beam for the new DIDSON camera at the PH2 NDE LPS was installed on 15 February. The NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service for the season.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in June, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December outlines new tailwater operations for chum incubation.  Effective immediately, BON is requested to maintain a tailwater of 12.2’ or greater for all hours.

  - A teletype sent on 22 December notifies BON that slow roll start-up for units that have been idle for 12 hours or more is required from 01 December through 30 April (see FPP section BON 5.7).

  - A teletype sent on 03 February outlines BON unit priority.  Effective immediately until further notice, BON unit priority is: 1,10,3,6,2,4,5,8,7,9,11,18,12,17,13,14,15,16.

  - A teletype sent on 08 February outlines the operation of the B2CC on 09 February from 0700 to 1630.  If sufficient debris has passed prior to 1630, a call will be made to close the B2CC.  If necessary, the B2CC can be opened on 10 February from 0700 to 1630.  This operation is necessary to flush debris in the PH2 forebay.

  - A teletype sent on 17 February requests that BON forebay be maintained between 71.5’-72.8’ as a hard constraint from 0700 HRS through 1700 HRS, daily, from 23 through 25 February. The purpose of the operation is to assist the Port of Hood River during a riprap construction/repair project.

GLEN A. SMITH, P.E.

Operations Project Manager