### CENWP-OD-B																						02 March 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 08 of 2011, which covers the period from 20 through 26 February 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 191.2 to 220.1 kcfs.  Daily average project tailwater ranged from 16.4’ to 18.5’ msl.  Daily average powerhouse forebay elevation ranged from 71.9’ to 73.5’ msl.  Daily average river temperature ranged from 38 to 39ºF.  Secchi disk measurements ranged from 5 to 6.3’.

  - Spill Bay 18, which provides attraction flow for B-branch, is open 6” during daylight hours.  Spill Bay 1, which provides attraction flow for Cascades Island, was opened 6” during daylight hours on 24 February. Daily average spill ranged from 0.6 to 0.7 kcfs.

  - Unit Operation.  Powerhouse One is the priority powerhouse.

    - Unit 1 has remained out of service since 17 September 2010 due to sticking wicket gates. It was returned to service at 1720 on 22 February.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 was out of service from 0701 until 1113 on 22 February for STS installation and from 1552 until 1656 on 22 February for north gate local repair.

    - Unit 13 was out of service from 1121 until 1548 on 22 February for STS installation.

    - Unit 14 was out of service from 0731 until 0946 on 23 February for STS installation.

    - Unit 15 was out of service from 0950 until 1146 on 23 February for STS installation.

    - Unit 16 was out of service from 1152 until 1353 on 23 February for STS installation.

    - Unit 17 was out of service from 0730 until 0921 on 24 February for STS installation.

    - Unit 18 was out of service from 0929 until 1121 on 24 February for STS installation.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 remains out of service while the WS fishway is dewatered.

    - F2 returned to service at 1911 on 26 February.  It had been out of service since 15 November for annual maintenance, headgate rehab, and while the WS fishway was dewatered.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 3-9 was closed for cleaning from 0713 until 0726 on 22 February.

    - FV 5-3, 5-4, and 5-9 remain closed while the Cascades Island fishway is dewatered.

    - FV 6-9 returned to service at 1519 on 24 February.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STSs installed 22 through 24 February. |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this week.

- RESEARCH

  - Battelle - survival study.  Acoustic tag training scheduled to begin the week of 07 March.  Fish for training currently held in raceways at the SMF.

  - CRITFC - Sea lion hazing.  Hazing from boats scheduled to begin on 28 February.

  - FFU - Sea lion.  Observations began on 07 January.

  - NOAA - lamprey. DIDSON mount installed on the NDE LPS on 15 February.  Divers armored the HDX PIT antennae conduit in the CI entrance on 21 February.

  - NOAA/PITAGIS - B2CC PIT testing.  Testing occurred from 22-24 March.

  - Normandeau - B2CC/ITS Kelt study.  Releases scheduled to begin on 28 February.

  - ODFW/WDFW - Sea lion trapping. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Trapping has not begun for the season.

  - PSMFC.  Smolt monitoring scheduled to begin on 01 March.

  - WDFW - Fish Counting.  Video counting continues.

  - USDA - Sea lion hazing. Hazing from the deck scheduled to begin on 28 February.

- FISHWAYS

  - Project biologists inspected 22 through 24 February.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 02/23 | FV 1-1 trashrack | Head > 0.5’ | Debris | Riggers cleaned. |  |
| 02/24 | FV 3-7 trashrack | Head > 0.5’ | Debris | Riggers cleaned. |  |
| CASCADES ISLAND (CI) – Dewatered |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Returned to service 24 February |  |  |  |  |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The PH1 PCC mechanical velocity meter remains out for maintenance crews to use as a template while constructing a new one.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 24 February.

    - The Washington Shore fishway exit was opened at 1519 on 24 February.  F2 returned to service on 26 February.  F1 remains OOS.

    - The Cascades Island ladder and UMT scheduled to return to service on 28 February.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs remain installed at all fishway entrances.

  - Juvenile Fishways.

    - DSM2 remains in service. The SMF is scheduled to return to sample mode on 01 March.

    - B2CC remains closed for the season.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Manual chaingates 1A and 1B remain open. Automated chaingates 3B, 6C, and 10B are open and functioning in manual.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.

    - The I-beam for a new DIDSON camera at the PH2 NDE LPS was installed on 15 February. The NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service for the season.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in June, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls complete.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December outlines new tailwater operations for chum incubation.  Effective immediately, BON is requested to maintain a tailwater of 12.2’ or greater for all hours.

  - A teletype sent on 22 December notifies BON that slow roll start-up for units that have been idle for 12 hours or more is required from 01 December through 30 April (see FPP section BON 5.7).

  - A teletype sent on 03 February outlines BON unit priority.  Effective immediately until further notice, BON unit priority is: 1,10,3,6,2,4,5,8,7,9,11,18,12,17,13,14,15,16.

  - A teletype sent on 17 February requests that BON forebay be maintained between 71.5’-72.8’ as a hard constraint from 0700 HRS through 1700 HRS, daily, from 23 through 25 February. The purpose of the operation is to assist the Port of Hood River during a riprap construction/repair project.

GLEN A. SMITH, P.E.

Operations Project Manager