### CENWP-OD-B																						03 April 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 13 of 2011, which covers the period from 27 March through 02 April 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 225.6 to 331.4 kcfs.  Daily average project tailwater ranged from 18.9’ to 25.0’ msl.  Daily average powerhouse forebay elevation ranged from 72.6’ to 75.0’ msl.  Daily average river temperature ranged from 42 to 44ºF.  Secchi disk measurements ranged from 3.0 to 4.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 1.4 to 99.8 kcfs.

    - Bay 1 was closed from 0600 until 0938 on 30 March for the trolley pipe dive.

    - Bay 18 was closed from 0935 until 1302 on 30 March for the trolley pipe dive.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 5 was out of service from 0926 until 1743 on 28 March for the trolley pipe dive and substation line #2 trip.

    - Unit 6 was out of service from 0929 until 1743 on 28 March for the trolley pipe dive and substation line #2 trip.

    - Unit 7 was out of service from 1611 until 1744 due to a trip of Transformer 5/6 which caused BPA's 230kV line #2 to trip.  Unit 7 was out of service from 0559 until 1220 on 29 March for the trolley pipe dive.

    - Unit 8 was out of service from 1611 until 1747 due to a trip of Transformer 5/6 which caused BPA's 230kV line #2 to trip. Unit 8 was out of service from 0559 until 1221 on 29 March for the trolley pipe dive.

    - Unit 9 was out of service from 1425 until 1708 on 29 March for the trolley pipe dive.

    - Unit 10 was out of service from 1422 until 1718 on 29 March for the trolley pipe dive.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 was out of service from 1228 until 1421 on 29 March to remove a trolley pipe.

    - Unit 13 was out of service from 1228 until 1419 on 29 March to remove a trolley pipe.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 1-1 was closed from cleaning from 0711 until 0800.

    - FV 4-3 and 4-4 were closed from 0948 until 1300 on 30 March for the trolley pipe dive.

    - FV 5-3 and 5-4 were closed from 0700 until 0940 on 30 March for the trolley pipe dive.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 04 April |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle - survival study.  Acoustic tag training began the week of 07 March.  Fish for training currently held in raceways at the SMF.  Hydrophone installation completed on 29 March.

  - CRITFC - Sea lion hazing.  Hazing from boats continues.

  - FFU - Sea lion.  Observations continue.

  - NOAA - lamprey. DIDSON mount installed on the NDE LPS on 15 February.

  - ODFW/WDFW - Sea lion trapping. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Trapping began this week.

  - PSMFC.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars:  19 steelhead.  Please see http://www.fpc.org/adultsalmon/bonkeltcounts.htm for updated kelt counts.

  - WDFW - Fish Counting.  Video counting continues.   Visual counting scheduled to begin 01 April.

  - USDA - Sea lion hazing. Hazing from the deck continues.

- FISHWAYS

  - Project biologists inspected 27 March through 02 April.  NOAA inspected on 01 April.

  - Fish Passage Plan Observations.

- Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 3/28-29 | ITS | Closed | Trolley pipe dive | Returned to service daily |  |
| 3/31 | B-branch entrance | Differential < 1.0’ | System in manual | Operations adjusted. |  |
| CASCADES ISLAND (CI) |  |  |  |  |  |
| 3/30 | Spill gate 1 & 18 | No attraction flow | Trolley pipe dive | Returned to service on 3/30 |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 4/1 | U15, U16, U17, U18 VBSs | Differential > 1.1’ | Debris | Riggers cleaned |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The PH1 PCC mechanical velocity meter has been repaired and a new mechanical velocity meter has been fabricated for PH2. Both are scheduled for installation.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. Work has begun on replacing the PLC to allow for fully automated operation of the PH2 adult fishway.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 3/27 | 41.9 | 42.3 |
| 3/28 | 42.2 | 42.7 |
| 3/29 | 42.7 | 43.2 |
| 3/30 | 43.2 | 43.7 |
| 3/31 | 43.8 | 44.4 |
| 4/1 | 44.2 | 44.6 |
| 4/2 | 44.4 | 44.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 02 April.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs remain installed at all fishway entrances.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Manual chaingates 1A and 1B remain open. Automated chaingates 6C and 10B are open and functioning in auto.  Chaingate 3B remains in manual.  The ITS end gate was closed from 0750 until 1816 on 28 March and from 0613 until 1500 on 29 March to facilitate the trolley pipe repositioning dive in the PH1 forebay.

    - The hydrocannon remains in service.

    - The B2CC remains in service.  The B2CC was on sill from 1117 until 1510 to facilitate the trolley pipe dive in the PH2 forebay.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.  Cascade Is. LPS remains out of service.

    - The NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in June, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material and shoring of the walls is complete.  Load test scheduled to occur spring 2011.

  - B2CC Hoist. Construction scheduled to begin fall of 2011.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December outlines new tailwater operations for chum incubation.  Effective immediately, BON is requested to maintain a tailwater of 12.2’ or greater for all hours.

  - A teletype sent on 22 December notifies BON that slow roll start-up for units that have been idle for 12 hours or more is required from 01 December through 30 April (see FPP section BON 5.7).Note: Though specified in the FPP and the teletype as only occurring when units have been idle for 12 or more hours, Bonneville has been starting all idle units with slow roll protocol.

  - A teletype sent on 15 March authorizes BON to open the corner collector (B2CC). If TDG at the Warrendale site exceeds 110% after 24 hours of operation, close Spillbays 1 and 18.  If the 110% TDG is still exceeded after 24 hours operating without spillbays, then close the B2CC and operate PH1 as the priority powerhouse.

  - A teletype sent on 24 March outlines a short term closure of the B2CC from 1200 until 1700 on 29 March to remove a damaged trolley pipe at the B2CC entrance.

  - A teletype sent on 24 March authorizes BON to close the PH1 ITS from 0700 until 1700 each day on 28 and 29 March.  This is operation is necessary to support relocation of three trolley pipes in the PH1 forebay.

  - A teletype sent on 28 March authorizes BON to close Spillbay 1 and the auxiliary water supply valves for the Cascades Island fishway from 0700 until 1200, and Spillbay 18 and the auxiliary water supply valves for the B-branch fishway from 1200 until 1700.  This is operation is necessary to support trolley pipe repair in the spillway forebay.

GLEN A. SMITH, P.E.

Operations Project Manager