### CENWP-OD-B																						22 May 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 20 of 2011, which covers the period from 15 through 21 May 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 377.0 to 468.9 kcfs.  Daily average project tailwater ranged from 27.3’ to 31.1’ msl.  Daily average powerhouse forebay elevation ranged from 72.2’ to 74.0’ msl.  Daily average river temperature ranged from 53 to 55ºF.  Secchi disk measurements ranged from 1.5’ to 3.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 156.0 to 258.6 kcfs.

  - Unit Operation.  Powerhouse Two was the priority powerhouse until 18 May. Powerhouse One became the priority powerhouse on 19 May.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 14 was out of service from 1201 until 1457 on 18 May for STS removal.

    - Unit 15 was out of service from 1202 until 1352 on 19 May for STS removal.

    - Unit 16 was out of service from 1205 until 1345 on 20 May for STS removal.

    - Unit 18 was out of service from 0814 until 1105 on 16 May and from 1532 until 1645 on 20 May for cooling water strainer work.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured as needed and on Wednesdays.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

    - FV 3-7 was closed for cleaning from 0955 until 1102 on 18 May.

    - FV 6-9 was closed for cleaning from 0828 until 0854 on 20 May and from 1352 until 1405 on 21 May.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 06 June. |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle - survival study.  Acoustic tagging and fish releases continue.

  - CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC - Sea lion hazing.  Hazing from boats has ended for the season.  Acoustic receivers for sea lion tags anchored in BRZ.  Video camera system for sea lion observation set up at PH2 south monolith.

  - FFU - Sea lion.  Observations continue.  Weekly pinniped reports can be found at

http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2011/sea_lion_hazing2011.html

  - Kintama Research.  Acoustic tagging yearling Chinook at the SMF continues.  175 yearling Chinook were collected from SMP sampling for research this week.

  - NOAA - COE Walla Walla District transportation evaluation.  Sort by Code collections at the SMF continue.

  - NOAA - lamprey. DIDSON mount installed on the NDE LPS on 15 February.  LPS count network remains installed.

  - ODFW/WDFW - Sea lion trapping. Sea lion traps were moved from B2CC to downstream of the old Navlock.  Acoustic tagging and branding continues.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars:  59 steelhead, 55 salmon and 26 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  A total of 25 yearling Chinook and 25 coho were collected for NOAA research this week.  175 yearling Chinook were collected for Kintama.

  - WDFW - Fish Counting.  Visual counting continues.

  - USDA - Sea lion and avian hazing. Hazing from the deck continues.

- FISHWAYS

  - Project biologists inspected 15 through 21 May.

  - Fish Passage Plan Observations.

- Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 5/17, 19, 21 | B-branch entrance | Differential < 1.0.’ | High tailwater, entrance elev. sensor | Operations adjusted. |  |
| 5/20 | PH1 South entrance | Differential < 1.0’ | High tailwater | None. |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 5/15-18 | VBSs | Differential >1.1 | Debris | Riggers cleaned. |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - The PH1 PCC mechanical velocity meter has been repaired and was installed on 12 May. A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

    - The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. Work has begun on replacing the PLC to allow for fully automated operation of the PH2 adult fishway.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 5/15 | 52.4 | 52.9 |
| 5/16 | 52.2 | 52.7 |
| 5/17 | 52.7 | 53.2 |
| 5/18 | 52.9 | 53.4 |
| 5/19 | 53.1 | 53.6 |
| 5/20 | 53.3 | 53.8 |
| 5/21 | 53.2 | 53.7 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 21 May.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs remain installed at all fishway entrances.

    - The AFF remains watered up.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - Due to high flow and high debris load, elevated descaling was observed at the JMF starting 13 May, leading to a teletype requesting a change in operations on 19 May (see section 8.4). Powerhouse One became priority and STSs were removed on units 14, 15, and 16.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains in service.

    - The B2CC remains in service.  Three avian lines are broken and five remain.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.  The CI LPS remains out of service.

    - The NDE ramp remains in the lowered position and is not currently operating.

    - Bradford Island and WA Shore LPS’s remain out of service.  Scheduled to water up the week of 22 May.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in August, 2011.

  - Spillway Gate Repair Pit. Clean-up of hazardous material and shoring of the walls is complete.  Load test scheduled to occur spring 2011.

  - B2CC Hoist. Construction scheduled to begin fall of 2011.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 22 December notifies BON that slow roll start-up for units that have been idle for 12 hours or more is required from 01 December through 30 April (see FPP section BON 5.7).  Note: Though specified in the FPP and the teletype as only occurring when units have been idle for 12 or more hours, Bonneville has been starting all idle units with slow roll protocol.

  - A teletype sent on 08 April requests spill for juvenile fish passage begin at 0001 hours on Sunday, 10 April and continue through 20 June at 2400 hours. Spill approximately 100 kcfs 24 hours a day using the spill pattern in the 2011 FPP.  If Bonneville inflows are too low to maintain the 100 kcfs spill and minimum turbine discharge of 30-40 kcfs, then operate to minimum discharge and spill to pass inflow.

  - A teletype sent on 19 May requests that BON operate Powerhouse One as the priority powerhouse. Continue this operation until further notice. The goal of this operation is to minimize debris loading that is contributing to increased descaling and mortality of juvenile fish at Powerhouse Two (PH2). Additionally, operate units 14, 15, and 16 without Submersible Traveling Screens. Vertical Barrier Screens (VBSs) will remain installed for these units until debris loading causes a drawdown of 1.5 ft. Once VBSs satisfy this criterion, remove VBSs in units 14, 15, and 16. When selecting operating range within the 1% range of the PH2 screened units (12, 13, 17, and 18) make efforts to the extent practicable to achieve the following two objectives: 1) Do not exceed the 1.5 ft head differential prior to the next cleaning and 2) Maximize discharge at each unit. Objective 1 takes priority over objective 2.

  - A teletype sent on 20 May requests the following operation at BON for the purposes of system flood control. Make the best effort to maintain outflows from BON near 465 kcfs, beginning at 0800 hrs, Friday, 20 May. Proceed to make the best effort to maintain flows near 470 kcfs on Saturday, 21 May, and 475 kcfs on Sunday, 22 May. On Monday, 23 May, ramp up outflow and maintain flows near 490 kcfs until further notice.

GLEN A. SMITH, P.E.

Operations Project Manager