### CENWP-OD-B																						17 July 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 28 of 2011, which covers the period from 10 through 16 July 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 304.0 to 382.4 kcfs.  Daily average project tailwater ranged from 23.1’ to 26.7’ msl.  Daily average powerhouse forebay elevation ranged from 73.5’ to 74.9’ msl.  Daily average river temperature ranged from 63 to 64ºF. Secchi disk measurements ranged from 2.5’ to 4.5’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 106.6 to 193.7 kcfs.

  - Unit Operation.  Powerhouse One remains the priority powerhouse.

    - Unit 5 went out of service at 0001 on 11 July for transformer bi-annual and unit annual overhaul.

    - Unit 6 went out of service at 0001 on 11 July for transformer bi-annual and unit annual overhaul.

    - Unit 7 was out of service from 0006 until 0042 on 11 July for bank 5/6 switching. It was out again from 1244 until 1315 on 14 July for a disabled recreational vessel that entered the Powerhouse 1 boat restricted zone (BRZ).

    - Unit 8 was out of service from 0009 until 0040 on 11 July for bank 5/6 switching. It was out again from 1243 until 1322 on 14 July for a disabled recreational vessel that entered the Powerhouse 1 BRZ.

    - Unit 9 was out of service from 1243 until 1320 on 14 July for a disabled recreational vessel that entered the Powerhouse 1 BRZ.

    - Unit 10 was out of service from 1242 until 1321 on 14 July for a disabled recreational vessel that entered the Powerhouse 1 BRZ.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 was out of service from 1118 on 11 July until 1737 on 14 July for annual maintenance/vibration evaluation.

    - Unit 13 was out of service from 1418 until 1538 on 12 July for STS installation.

    - Unit 15 went out of service at 0717 on 15 June for four year overhaul.

    - Unit 18 was out of service from 0803 until 1041 on 12 July for STS installation and from 0839 until 0910 on 14 July for trash raking of fish unit #1.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.  This week required fish units to be placed on standby to float debris during daylight hours due to excessive drawdown.

  - Gatewell drawdowns are measured on Wednesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS valves were closed for cleaning this reporting week.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

  - Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 25 July. |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle - survival study.  Monitoring University of Idaho acoustic tagged adult lamprey.

  - CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC - Sea lion.  Acoustic receivers for sea lion tags remain anchored in the BRZ.

  - NOAA/PSMFC - lamprey.  LPS count network improvements moving forward.  Traps deployed at PH2 monoliths in tailrace and AFF.

  - NOAA – Pathology Study. A total of 28 subyearling chinook were collected for NOAA pathology studies this week.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars:  11 steelhead, 28 salmon, and 30 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - University of Idaho.  Tagging lamprey in the AFF continues.

  - USDA - Avian hazing. Hazing continues.

  - WDFW - Fish Counting.  Visual counting continues.

- FISHWAYS

  - Project biologists inspected 10 through 16 July.

  - Fish Passage Plan Observations.

- Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 7/12 | Weir gate 65 | Submergence level | Tripped | Ops adjusted |  |
| 7/12 | B-branch entrance | Entrance diff > 2’ | System in local | Ops adjusted |  |
| 7/15 | Weir gate 64 | Bulkhead not open at TW 25.5 | Unknown | Ops adjusted |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 7/13 | PH2 entrance gates | Submergence <13’ | System in local | Ops adjusted |  |
| All week | DSM2 Screen cleaner #2 | Screen cleaner #2 OOS | Faulty prox switch | TR and adjusted air burst system to heavy debris | 11-196891 |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

    - The SA-24 board that controls the Powerhouse Two adult ladder components is in need of replacement. Work has begun on replacing the PLC to allow for fully automated operation of the PH2 adult fishway.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average |
| --- | --- | --- |
| 7/10 |  |  |
| 7/11 |  |  |
| 7/12 |  |  |
| 7/13 |  |  |
| 7/14 |  |  |
| 7/15 |  |  |
| 7/16 |  |  |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 15 July.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs were removed from all entrance gates as of 05 July.

    - The AFF remains watered up.

    - Cascades Is. ladder was returned to operation for salmon passage on 13 July. Bradford Is. ladder was returned to operation for salmon passage on 14 July. Both had been in shad passage mode.

    - One section of diffuser grating between weir 34 and 35 came loose during high tailwater and was removed on 14 June.  Please see 16 June MOC for more information.

    - The Bradford Is. AWS was dewatered and the ladder had reduced flow to remove 100 - 200 sockeye trapped in the AWS section downstream of the picket leads on 30 June.    Please see 30 June Bonneville Fisheries MFR for more information.

  - Juvenile Fishways.

    - DSM2 remains in service. Mechanical screen cleaner #2 developed a faulty prox switch on 09 July. It is currently OOS until repairs can be made. All other screen cleaners are operating without regard to this malfunction.

    - STSs for units 12, 13, and 18 were installed on 12 July.

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon was returned to service on 06 July.

    - The B2CC remains in service.  Five avian lines are broken due to high tailwater and three remain.

    - Operators manually adjusted spill gates 13 and 14 to flush trash on 23 June.  Both bays encountered problems and were dogged at the most appropriate position for the spill pattern. Bay 14 had problems with the brakes and was repaired on 02 July.  Bay 13 had problems with the overloads and was dogged at 4.9 ft on 05 July. It was repaired and returned to service 06 July.  Bay 17 was moved from the dogged position of 2.9 ft to 4.9 ft on 28 June to get closer to FPP spill pattern positions.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.  The CI LPS is in operation on weekdays.

    - The NDE ramp remains in the lowered position and is not currently operating.

    - WA Shore and Bradford Island LPSs remain watered up.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in August, 2011.

  - B2CC Hoist. Construction scheduled to begin fall of 2011.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 19 May requests that BON operate Powerhouse One as the priority powerhouse. Continue this operation until further notice. The goal of this operation is to minimize debris loading that is contributing to increased descaling and mortality of juvenile fish at Powerhouse Two. Additionally, operate units 14, 15, and 16 without Submersible Traveling Screens. Vertical Barrier Screens (VBSs) will remain installed for these units until debris loading causes a drawdown of 1.5 ft. Once VBSs satisfy this criterion, remove VBSs in units 14, 15, and 16. When selecting operating range within the 1% range of the PH2 screened units (12, 13, 17, and 18) make efforts to the extent practicable to achieve the following two objectives: 1) Do not exceed the 1.5 ft head differential prior to the next cleaning and 2) Maximize discharge at each unit. Objective 1 takes priority over objective 2.

  - A teletype sent on 15 June requests summer spill for juvenile fish passage with two treatments of 85 kcfs during the day and 121 kcfs at night or 95 kcfs for 24 hours.  Effective 16 June at 0430 hours and continuing until 31 August at 2400 hours, spill in accordance with the 2011 Fish Passage Plan and the 2011 Summer Fish Operations Plan, Appendix E of the Fish Passage Plan.

  - A teletype sent on 01 July requests that Bonneville pool be operated as follows:  Operate within a 1.5 foot band, soft constraint, from 0600 hours Monday, 11 July until 1800 hours Friday, 15 July.  The goal of this operation is to limit pool fluctuation during treaty fishing.

  - A teletype sent on 01 July requests that Bonneville pool be operated as follows:  Operate within a 1.5 foot band, soft constraint, from 0600 hours Monday, 18 July until 1800 hours Thursday, 21 July.  The goal of this operation is to limit pool fluctuation during treaty fishing.

  - A teletype sent on 14 July updated a former teletype with new forebay soft constraints for a kite boarding event. Request that as a soft constraint, the BON forebay be maintained as close practicable to the upper limit of the treaty fishing pool range of approximately 74.2’ from 1000 to 1500 hrs each day Thurs-Fri, 14-15 July.

  - A teletype sent on 07 July requests that BON forebay be maintained between elevation 75.5’ and 76.5’ as a hard constraint and 76.0’ and 76.5’ as a soft constraint from 1400 to 2000 hrs on Saturday, 06 August for a sail boat race. This request allows sail boats to be pulled from the water at the Dalles boat ramp after racing.

  - A teletype sent on 08 July requests that BON forebay be maintained between elevation 74.0’ and 75.5’ as a soft constraint, Saturday 16 July and Sunday 17 July, from 1000 to 1700 hrs. This operation will assure unobstructed access for large canoes to launching and landing sites during the Gore Outrigger races near Stevenson, WA.

  - A teletype sent on 13 July requests that on Thursday, 14 July from 0800-1000 hrs, shut down unit 18 in order to remove debris from the Washington shore adult fish units. Once the debris removal is complete, revert back to normal operations identified in the 2011 FPP.

JERRY CARROLL

Operations Project Manager