### CENWP-OD-B																						24 July 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 29 of 2011, which covers the period from 17 through 23 July 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 268.1 to 301.6 kcfs.  Daily average project tailwater ranged from 20.7’ to 22.8’ msl.  Daily average powerhouse forebay elevation ranged from 73.7’ to 74.9’ msl.  Daily average river temperature ranged from 64 to 65ºF. Secchi disk measurements ranged from 4.0’ to 5.0’.

  - Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 87.9 to 123.4 kcfs.

  - Unit Operation.  Powerhouse One remains the priority powerhouse.

    - Unit 5 went out of service at 0001 on 11 July for transformer bi-annual and unit annual overhaul. It was returned to service at 1628 on 20 July.

    - Unit 6 went out of service at 0001 on 11 July for transformer bi-annual and unit annual overhaul. It was returned to service at 1732 on 20 July.

    - Unit 7 was out of service from 0201 on 19 July until 1657 on 20 July for relay commissioning.

    - Unit 8 was out of service from 0201 on 19 July until 1702 on 20 July for relay commissioning.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 14 was out of service from 0707 until 0911 on 19 July for STS installation.

    - Unit 15 went out of service at 0717 on 15 June for four year overhaul.

    - Unit 16 was out of service from 0917 until 1106 on 19 July for STS installation.

    - Unit 17 was out of service from 1240 until 1420 on 19 July for STS installation.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.

    - FV3-7 was closed for cleaning from 0712 until 0745 on 21 July.

  - STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.

  - Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 25 July. |

  - Dewatering and Fish Salvage.  No fish salvage activities occurred this reporting week.

- RESEARCH

  - Battelle - survival study.  Monitoring University of Idaho acoustic tagged adult lamprey.

  - CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues. New sampling tank installation scheduled for the week of 25 July. CRITFC will postpone sampling in the AFF from 25 July to 28 July to facilitate installation.

  - CRITFC - Sea lion.  Acoustic receivers for sea lion tags remain anchored in the BRZ.

  - NOAA/PSMFC - lamprey.  LPS count network improvements moving forward.  Traps deployed at PH2 monoliths in tailrace and AFF.

  - NOAA – Pathology Study. A total of 27 subyearling chinook were collected for NOAA pathology studies this week.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars:  7 steelhead, 24 salmon, and 25 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - University of Idaho.  Tagging lamprey in the AFF continues.

  - USDA - Avian hazing. Hazing continues.

  - WDFW - Fish Counting.  Visual counting continues.

  - WDFW – AFF. Modifications to the sample flume and relocation of the old sample tank scheduled for the week of 25 July. Sampling for alternative gear study control group scheduled to begin the week of 22 August.

- FISHWAYS

  - Project biologists inspected 17 through 23 July.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) – Nothing to report. |  |  |  |  |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 7/23 | PH2 entrance gates | Submergence <13’ | System in local | Ops adjusted |  |
| 7/23 | South (B) Diffusers | Gate 6 not open at TW 21’ | System in local | Ops adjusted |  |
| All week | DSM2 Screen cleaner #2 | Screen cleaner #2 OOS | Faulty prox switch | TR and adjusted air burst system to compensate | 11-196891 |
| 7/23 | DSM2 Orifice 11A-S | Faulty prox switch | Unknown | TR | 11-209649 |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

    - The SA-24 board that controls the Powerhouse Two adult ladder components is in need of replacement. Work has begun on replacing the PLC to allow for fully automated operation of the PH2 adult fishway.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average | AFF average | SMF average |
| --- | --- | --- | --- | --- |
| 7/17 | 62.7 | 63.2 | 62.9 | 63.5 |
| 7/18 | 62.6 | 63.0 | 62.8 | 63.4 |
| 7/19 | 62.6 | 63.0 | 62.9 | 63.4 |
| 7/20 | 63.0 | 63.4 | 63.2 | 66.0* |
| 7/21 | 63.2 | 63.6 | 63.3 | 66.1* |
| 7/22 | 63.1 | 63.5 | 63.2 | 66.0* |
| 7/23 | 63.3 | 63.7 | 63.5 | 65.9* |

*The temperature probe in the SMF read inaccurately from 20 July to 23 July. It is being investigated.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 22 July.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs were removed from all entrance gates as of 05 July.

    - The AFF remains watered up.

    - Cascades Is. ladder was returned to operation for salmon passage on 13 July. Bradford Is. ladder was returned to operation for salmon passage on 14 July. Both had been in shad passage mode.

  - Juvenile Fishways.

    - DSM2 remains in service. Mechanical screen cleaner #2 developed a faulty prox switch on 09 July. It is currently OOS until repairs can be made. All other screen cleaners are operating without regard to this malfunction.

    - STSs for units 12 were installed on 11 July and on 12 July units 13 and 18 were installed. The remaining STSs for units 14, 16, and 17 were installed on 19 July. Unit 15 STSs will be installed when four year overhaul is complete (11 August).

    - The PH1 ice and trash sluiceway (ITS) remains in service.  Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon was returned to service on 06 July.

    - The B2CC remains in service.  Five avian lines are broken due to high tailwater and three remain.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.  The CI LPS is in operation on weekdays.

    - The NDE ramp remains in the lowered position and is not currently operating.

    - WA Shore and Bradford Island LPSs remain watered up.

    - One of the two water supply pumps that feed the Bradford Island LPS burnt out on 16 July. It was replaced on 18 July.

- CONSTRUCTION

  - PH1 Gantry 4.  New crane in August, 2011.

  - B2CC Hoist. Construction scheduled to begin fall of 2011.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 15 June requests summer spill for juvenile fish passage with two treatments of 85 kcfs during the day and 121 kcfs at night or 95 kcfs for 24 hours.  Effective 16 June at 0430 hours and continuing until 31 August at 2400 hours, spill in accordance with the 2011 Fish Passage Plan and the 2011 Summer Fish Operations Plan, Appendix E of the Fish Passage Plan.

  - A teletype sent on 01 July requests that Bonneville pool be operated as follows:  Operate within a 1.5 foot band, soft constraint, from 0600 hours Monday, 18 July until 1800 hours Thursday, 21 July.  The goal of this operation is to limit pool fluctuation during treaty fishing.

  - A teletype sent on 07 July requests that BON forebay be maintained between elevation 75.5’ and 76.5’ as a hard constraint and 76.0’ and 76.5’ as a soft constraint from 1400 to 2000 hrs on Saturday, 06 August for a sail boat race. This request allows sail boats to be pulled from the water at the Dalles boat ramp after racing.

  - A teletype sent on 08 July requests that BON forebay be maintained between elevation 74.0’ and 75.5’ as a soft constraint, Saturday 16 July and Sunday 17 July, from 1000 to 1700 hrs. This operation will assure unobstructed access for large canoes to launching and landing sites during the Gore Outrigger races near Stevenson, WA.

  - A teletype sent on 19 July states that on 20 July starting a 0830 hours use Gantry 6 to lower spillway gate 6 from the current 3.9’ to sill, and then begin raising the spillway gate 6 to a maximum of 15’. After reaching the maximum height of 15’, lower gate 6 back to sill. Repeat as necessary, up to a maximum of three times, to measure the friction contribution of the spillway gate rollers and the resulting hoist load of the spillway gate. Once this operation is complete revert back to summer spill operations.

  - A teletype sent on 20 July states that, effective immediately, resume operation of Powerhouse 2 as the priority powerhouse as defined in the Fish Passage Plan.

  - A teletype sent on 22 July requests that Bonneville pool be operated as follows:  Operate within a 1.5 foot band, soft constraint, from 0600 hours Monday, 25 July until 1800 hours Saturday, 30 July.  The goal of this operation is to limit pool fluctuation during treaty fishing.

JERRY CARROLL

Operations Project Manager