### CENWP-OD-B																						25 September 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 38 of 2011, which covers the period from 18 through 24 September 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 97.6 to 130.4 kcfs.  Daily average project tailwater ranged from 9.6’ to 11.6’ msl.  Daily average powerhouse forebay elevation ranged from 74.8’ to 75.6’ msl.  Daily average river temperature ranged from 68 to 69ºF. Secchi disk measurements remained at an unremitting 7.0’ all week.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Spill bay 1 is open 1’ during daylight hours per teletype in section 7.5.1.  Spill bay 18 remains on sill.  Daily average spill ranged from 0.8 to 1.3 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 9 went out of service at 0728 on 07 September for bearing replacement.

    - Unit 10 was out of service from 0652 until 0940 on 20 September for bank 9/10 relay testing.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 15 went out of service at 0719 on 05 September due to exciter failure.

    - Unit 16 went out of service at 0452 on 04 September due to exciter failure.

    - Unit 17 went out of service at 1010 on 09 August due to generator vibration.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS closures occurred this reporting week.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspections.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 26 September |

  - Dewatering and Fish Salvage. The following activity occurred this reporting week.

    - On 22 September Project Fisheries personnel salvaged 10 salmonids from the B-branch ladder diffuser pools that were not salvaged on 17 September due to limited visibility.  All fish were released to the forebay in good condition.

- RESEARCH

  - Battelle - survival study.  Monitoring University of Idaho acoustic tagged adult lamprey.

  - CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

  - CRITFC - Sea lion.  Acoustic receivers for sea lion tags remain anchored in the BRZ.

  - NOAA/PSMFC - lamprey.  LPS count network improvements moving forward. Cascades Island LPS trap and haul concluded on 15 September.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Fallbacks recorded crossing the separator bars: 9 salmon, and 5 unknown salmonids. Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - University of Idaho.  Tagging lamprey in the AFF concluded in early September.

  - University of Washington – JMF. 25 juvenile shad were collected this week.

  - WDFW - Fish Counting.  Visual counting continues.

  - WDFW – Tule Counting. Tule counting began 17 August.

  - WDFW – AFF. Sampling for alternative gear study control group began 24 August.

  - Yakama Fisheries – AFF. Coho energy study concluded 16 September.

- FISHWAYS

  - Project biologists inspected 18 through 24 September.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| All week | A-branch staff gauge | Water level > 1.1’ | B-branch OOS | N/A |  |
| All week | BI staff gauge | Water level < 0.9’ | B-branch OOS | N/A |  |
| All week | B-branch dewatered |  | Emergency repairs | N/A |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| All week | SDE, NDE | Will not lower past 3.5’ | Unknown, possible debris in gate slot | TR | 11-220488 |
| 9/21, 22 | Weir 37 bleed off | Weir 37 staff gauge out of criteria | No electrical operation | TR | 11-247377 |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

    - The SA-24 board that controls the Powerhouse Two adult ladder components is in need of replacement. Work replacing the PLC to allow for fully automated operation of the PH2 adult fishway will be complete by the end of the fiscal year.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

| Date | WA. Shore average | BI average* | AFF average | SMF Average |
| --- | --- | --- | --- | --- |
| 9/18 | 66.1 | 66.4 | 66.2 | 66.9 |
| 9/19 | 65.9 | 66.1 | 66.0 | 66.7 |
| 9/20 | 66.1 | 66.4 | 66.3 | 66.9 |
| 9/21 | 66.3 | 66.5 | 66.3 | 67.0 |
| 9/22 | 66.6 | 66.8 | 66.7 | 67.4 |
| 9/23 | 67.0 | 67.2 | 67.1 | 67.7 |
| 9/24 | 67.2 | 67.4 | 67.3 | 68.0 |

* Bradford Island B-branch was dewatered on 17 September.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 23 September.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs were removed from all entrance gates as of 05 July.

    - The AFF remains watered up.

    - B-branch was dewatered on 17 September. It remains out of service. See section 7.3 and MFR 110919 BON B-branch emergency dewater for more information.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon was returned to service on 06 July.

    - The B2CC was closed 01 September.  Five avian lines are broken due to high tailwater and three remain.

    - Spill ended on 01 September.

    - Truck releases of summer out-migrants originating from McNary began on 03 August. Truck releases from Lower Granite, Little Goose and Lower Monumental dams were inaugurated on 17 August.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - The CI LPS was taken out of service for the season on 15 September.

    - The NDE ramp is out of service for the season.

    - WA Shore and Bradford Island LPSs remain watered up.

- CONSTRUCTION

  - PH1 Gantry 4.  The new crane arrived on project 12 September. The old gantry was removed and the new gantry was put in place on 15 September. Testing of the new crane continues.

  - B2CC Hoist. Construction scheduled to begin fall of 2011.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 26 August requests that on Saturday, 17 September, 2011 from 0800 to 1600 hours, BON forebay be maintained in a 1.5 foot band of 72.0 feet - 73.5 feet as a hard constraint. It is also requested as a soft constraint to maintain the BON pool in a 1 foot band of 72.0 feet -73.0 feet for the same date and times. This operation will assist a clean-up event at the mouth of the White Salmon River.

    - This teletype was updated on 15 September to operate to a minimum elevation of 74.0 feet. Therefore, the hard constraint maximum is 75.5 feet and soft constraint maximum is 75.0 feet. This update of the operating range was requested by the Yakama Nation in response to new information regarding the clean-up event at the mouth of the White Salmon River on 17 September. Event personnel have identified that a pool lower than 74.0 feet would prevent boat access to certain areas of the White Salmon River.

  - A teletype sent 16 September states that emergency repair at the B branch Bradford Island fish ladder entrance is required immediately due to structural integrity issues and a high risk of failure related to erosion. The B branch needs to be dewatered and repaired. Effective immediately and until the fish biologists notify the control room operator operate Powerhouse 2 as priority and then use Powerhouse 1 if additional generation is requested by BPA. Continue with the same unit priority as listed in the 2011 Fish Passage Plan. It is recognized that due to the current high number of adult returning fish the project has been in a split flow unit priority, following the 2011 Fish Passage Plan guidance. Once notified by the biologists, return to this priority unless instructed differently by RCC. Effective immediately stop all attraction flow spill from spillbay 18 and add the same volume on to spill at spillbay 1. Continue this operation until further notice.

  - A teletype sent on 19 September requests that until Friday, September 23 at 1800 hours, operate within a 1.5’ band hard constraint of 74.0 – 75.5’ for fall treaty fishing. Until Friday, September 30, on Mondays, Wednesday, and Fridays from 0900 to 1400 hours each day, operation within a 1.0’ band hard constraint of 74.4 – 75.5 feet from fish trap and haul operations on the White Salmon River. Until further notice, operate the BON forebay to not exceed 75.5’ to facilitate repairs of the Bradford Island B-branch fishway entrance.

  - A teletype sent on 19 September states that effective immediately and until further notice from project biologists or RCC, operate Powerhouse 2 as the priority powerhouse in accordance with the 2011 Fish Passage Plan table BON-14. Fish passage numbers have decreased below 20,000 adult salmonids per day; therefore split flows are no longer required. Until further notice, close spillbay 18 and spill attraction flow only through spillbay 1. Until further notice, operate the BON forebay to not exceed an elevation of 75.5’, hard constraint.

    - This teletype was updated on 20 September to change spill from spillbay 1 from 6” to 12”.

  - A teletype sent on 23 September requests that the BON pool be operated for treaty fishing within a 1.5’ foot band (74.0’ – 75.5’), hard constraint for the period and times listed: From 0600 hours Monday 26 September to 1800 hours Thursday, 29 September 2011. (Note: A 1.0’ band (74.5 – 75.5’), hard constraint is required for fish salvage operations on the White Salmon river during period and times listed in below.) As per the teletype listed in 7.4, maximum BON pool elevation is 75.5’, hard constraint, required to accommodate emergency repairs at the Bradford Island B-branch fish ladder. Set BON pool floor at 74.5’, hard constraint, during times listed: From 0900 to 1400 hours, Monday September 26, 2011 from 0900 to 1400 hours, Wednesday September 28, 2011 From 0900 to 1400 hours, Friday, September 2011.

JERRY CARROLL

Operations Project Manager