### CENWP-OD-B																						16 October 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 41 of 2011, which covers the period from 09 through 15 October 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 104.2 to 145.3 kcfs.  Daily average project tailwater ranged from 10.0’ to 13.1’ msl.  Daily average powerhouse forebay elevation ranged from 73.7’ to 74.2’ msl.  Daily average river temperature ranged from 63 to 64ºF. Secchi disk measurements remained unremitting at 7.0’ all week.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Spill bay 1 is open 6” during daylight hours per teletype in section 7.2.  Spill bay 18 remains on sill.  Daily average spill was 0.6 kcfs.

  - Unit Operation.  Powerhouse One became the priority powerhouse at 0700 hours on 11 October per teletype in section 7.4.

    - Unit 9 went out of service at 0728 on 07 September for bearing replacement.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 15 went out of service at 0719 on 05 September due to exciter failure.

    - Unit 16 went out of service at 0452 on 04 September due to exciter failure.

    - Unit 17 was out of service from 0700 until 1350 on 12 October to remove air gap monitoring.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 went out of service at 0003 on 14 October for WA shore maintenance.

    - F2 went out of service at 0005 on 14 October for WA shore maintenance.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS closures occurred this reporting week.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 24 October |

  - Dewatering and Fish Salvage. The following activities occurred this reporting week.

    - On 14 October Project Fisheries personnel with assistance from District Fisheries salvaged 5 White Sturgeon and 1 adult salmonid from the AFF whilst dewatering Washington shore from the bulkhead at the “Y” junction (UMT confluence) down to weir 50. Fish were released at the Hamilton Island boat launch in good condition.

- RESEARCH

  - Battelle - survival study.  Monitoring University of Idaho acoustic tagged adult lamprey.

  - Columbia River Fish Health Lab. 65 juvenile shad were collected this week.

  - CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF concluded on 11 October.

  - CRITFC - Sea lion.  Acoustic receivers for sea lion tags remain anchored in the BRZ.

  - NOAA/PSMFC - lamprey.  LPS count network improvements moving forward. Cascades Island LPS trap and haul concluded on 15 September.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Fallbacks recorded crossing the separator bars: 12 salmon, and 3 unknown salmonids. Gas Bubble Trauma (GBT) examinations concluded with the end of spill on 31 August.

  - University of Washington – JMF. No fish were collected this week.

  - WDFW - Fish Counting.  Visual counting continues.

  - WDFW – Tule Counting. Tule counting concluded 03 October.

  - WDFW – AFF. Sampling for alternative gear study control group concluded on 11 October.

- FISHWAYS

  - Project biologists inspected 09 through 15 October.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| All week | BI staff gauge | Water level < 0.9’ | B-branch OOS | N/A |  |
| All week | B-branch dewatered |  | Emergency repairs | N/A |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 10/14-15 | Ladder dewatered from UMT confluence “Y” junction to AFF |  | Repairs | N/A |  |

    - PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

    - The SA-24 board that controls the Powerhouse Two adult ladder components is in need of replacement. Work replacing the PLC to allow for fully automated operation of the PH2 adult fishway is complete but some testing and adjustments are ongoing.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 15 October.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance.

    - SLEDs were removed from all entrance gates as of 05 July.

    - B-branch was dewatered on 17 September. It remains out of service. See section 7.1 and MFR 110919 BON B-branch emergency dewater for more information.

    - The AFF was dewatered on 14 October for WA shore repairs.

    - WA shore ladder was dewatered from the “Y” junction (UMT confluence) to weir 50 at the AFF exit on 14 October to facilitate diffuser grating and AWS repairs. From weir 50 to tailwater the ladder will remain at orifice flow until 17 October.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon was returned to service on 06 July.

    - The B2CC was closed 01 September.  All B2CC and Cascades Island avian lines were removed to facilitate barge access for B-branch emergency repairs. They will be reinstalled once repairs are complete.

    - Spill ended on 01 September.

    - Truck releases of summer out-migrants originating from McNary began on 03 August. Truck releases from Lower Granite, Little Goose and Lower Monumental dams commenced on 17 August.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - The CI LPS was taken out of service for the season on 15 September.

    - The NDE ramp is out of service for the season.

    - WA Shore and Bradford Island LPSs remain watered up.

- CONSTRUCTION

  - PH1 Gantry 4. Testing of the new crane continues.

  - B2CC Hoist. Construction scheduled to begin fall of 2011.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent 16 September states that emergency repair at the B branch Bradford Island fish ladder entrance is required immediately due to structural integrity issues and a high risk of failure related to erosion. The B branch needs to be dewatered and repaired. This Saturday in the morning, both A and B branch will be out of service in order to install concrete orifice blocks in B branch just below the junction pool. Once installed, the A branch will be reopened. Effective immediately and until the fish biologists notify the control room operator operate Powerhouse 2 as priority and then use Powerhouse 1 if additional generation is requested by BPA. Continue with the same unit priority as listed in the 2011 Fish Passage Plan. It is recognized that due to the current high number of adult returning fish the project has been in a split flow unit priority, following the 2011 Fish Passage Plan guidance. Once notified by the biologists, return to this priority unless instructed differently by RCC. Effective immediately stop all attraction flow spill from spillbay 18 and add the same volume on to spill at spillbay 1. Continue this operation until further notice, even after switching back to the split flow gen pattern. The purpose of this operation is to ensure better attraction to the Cascades Island entrance.

  - A teletype sent on 27 September states that, effective immediately operate attraction flow consistent with the 2011 FPP. Specifically, operate spillbay 1 spill gate with an opening of 6 inches. Keep spill gate 18 closed since B-branch is out of service. Continue to operate Powerhouse 1 at 1%. In the event hydraulic capacity would cause BON to spill then pick up generation at PH1 to best geometry (1.3 to 1.5 kcfs above the current 1% range) prior to spilling at BON. The intent of operation PH1 at best geometry would be in an effort to minimize spilling at BON once all units have been fully loaded. Once spilling at BON is no longer a concern then revert back to the 1% operation range. Should spill occur at BON then spill in accordance with the spill pattern titled “Special Spill Pattern for North End Spill Bays 1-9”.

  - A teletype sent on 05 October requests that from 0630 hours on 06 October to 1800 hours 31 October 2011, BPA notify the BON operator and RCC at least 30 minutes, or more, prior to any spill at BON. When notified, BON will take appropriate steps to evacuate workers from the spillway tailrace. When workers are not present, notice may allow the contractor to better secure and monitor equipment in the tailrace before and during spill.

  - A teletype sent on 07 October states that effective at 0700 hours on Tuesday, 11 October, until further notice, change powerhouse priority to PH1. Operate according to the 2011 FPP table BON-14 for “Second Powerhouse Adult Fish Ladder Out of Service”. Concurrent with the above operation, operate PH1 turbine units to Best Geometry, expanding the upper limit of the 1% operating range up to 11.7 kcfs per unit. These operations are necessary to reduce flow to PH2 to minimize attraction flow at Washington Shore fish ladder.

  - A teletype sent on 07 October requests that best effort be made as a soft constraint to maintain BON tailwater below elevation 9’ from 0900 to 1500 hours on the following days: Monday, 10 October 2011, and Tuesday, 11 October 2011. The low tailwater elevations assist in cement pours on Tuesday in connection with the repair of erosion below the B-branch Bradford Island ladder.

  - A teletype sent on 11 October requests that best effort be made as a soft constraint to maintain BON tailwater below elevation 12’ from 1200 to 1600 hours on 12 October 2011. Low tailwater elevation assists the stripping of forms used in cement pours at the B-branch ladder.

  - A teletype sent on 13 October requests that at 0700 hours on 26 October, continuing to 0700 on 29 October, operate the BON forebay to the minimum normal operating elevation of 71.5’ with a 1.5’ range up to 73’. To the extent practicable, operate within the bottom 1’ of the normal operating range as a soft constraint. This operation minimizes the potential for sediment released during the Condit dam breach to accumulate at the tribal in-lieu fishing site on the White Salmon river.

JERRY CARROLL

Operations Project Manager