### CENWP-OD-B																						18 December 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 50 of 2011, which covers the period from 11 through 17 December 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 132.2 to 151.4 kcfs.  Daily average project tailwater ranged from 11.8’ to 13.2’ msl.  Daily average powerhouse forebay elevation ranged from 73.7’ to 74.7’ msl.  Daily average river temperature ranged from 43 to 45ºF. Secchi disk measurements remained invariable at 7.0’ all week.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. As of 12 December, spill bay 1 remains 6” open during daylight hours.  Spill bay 18 remains on sill.  Daily average spill ranged from 0.5 to 1.0 kcfs.

  - Unit Operation.  Powerhouse One remains the priority powerhouse.

    - Unit 5 went out of service at 0903 on 05 December 2011 for a 5 year overhaul.

    - Unit 9 was out of service from 0325 until 1110 on 13 December for replacement of the shaft indexing sensor.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 16 went out of service at 0452 on 04 September 2011 due to exciter failure.

    - Unit 18 went out of service at 1030 on 23 November 2011 for a 4 year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 went out of service at 0003 on 14 October 2011 for WA shore repairs and headgate maintenance.

    - F2 was placed in standby for raking trash from 1257 until 1344 on 13 December.

    - F2 was placed in standby for troubleshooting from 1553 until 1601 on 13 December.

    - F2 was placed in standby for testing from 1051 until 1058 on14 December.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 1-1, 3-7, and 3-9 were closed on 12 December for Bradford Island dewatering. FV 4-3 and 4-4 remain closed.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STSs removed. |

  - Dewatering and Fish Salvage. On 14 December Project Biologists recovered 9 shad, 4 rainbow trout, 1 peamouth, 2 adult steelhead, and 14 juvenile salmonids from the junction pool in the Bradford Island ladder. 7 steelhead and 1 shad were recovered from the A-branch ladder. The fish were all released in good condition upstream of the Navlock.

- RESEARCH

  - WDFW - Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 12, 14, and 15 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) – Dewatered. |  |  |  |  |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 12/11-15 | DSM2 dewatered |  | Emergency WA shore repairs | N/A |  |

    - PH1CC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

    - The replacement of the Powerhouse Two adult ladder PLC on the SA-24 board is complete and the system is fully functional.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 15 December.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance. Stab plates were installed on 07 and 08 December in accordance with the FPP for 1 Fish Unit operations.

    - SLEDs were installed at WA shore and Cascades Island entrance gates on 09 December.

    - B-branch remains dewatered. Bradford Island serpentine section and A-branch were dewatered on 13 and 14 December, respectively.

    - The AFF remains dewatered.

    - WA shore ladder was taken out of service below the “Y” junction (UMT confluence) in facilitation of diffuser grating and AWS repairs. The ladder was fully returned to service on 08 December with 1 Fish Unit operations as outlined in table BON-13 in the FPP.

  - Juvenile Fishways.

    - DSM2 remains dewatered. STSs are removed.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains winterized.

    - The B2CC remains closed.  All B2CC and Cascades Island avian lines were removed to facilitate barge access for B-branch emergency repairs. They will be reinstalled during winter maintenance 2011/12.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPS systems remain out of service for the season.

- CONSTRUCTION

  - B2CC Hoist. Construction continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 23 Nov. states in order to keep the latest established Bon tailwater flow range for chum, total project flow may be decreased to as low as 58 kcfs as long as the estimated flow at the confluence of the Columbia and Willamette rivers is equal to or exceeds 77 kcfs.  The 58 kcfs and 77 kcfs is a hard constraint navigation requirement.

  - A teletype sent on 09 December updated and replaced previous teletypes concerning forebay operating range, PH unit priority, and operation of the spillway. Effective on 12 December:

    - Operate BON forebay with a maximum hard constraint of 75.5’ until 14 December when Bradford Island is completely dewatered. Then operate within full normal range.

    - Operate available turbine units according to priority order and operating ranges as defined in the 2011 FPP.

    - Operate the spillway in accordance with the 2011 FPP, i.e. open spill bay 1 6” during daytime hours. Keep spill bay 18 on sill.

  - A teletype sent on 28 November replaces a previous teletype for operation of BON tailwater to provide adequate spawning conditions for Chum salmon. Hard constraint 1: Maintain tailwater at a minimum of 11.7’ all hours. Hard constraint 2: Maintain tailwater within 0.8’ band of 11.7’-12.5’ daily from 0630-1700 hours.

  - A teletype sent on 09 December states that effective at 0700 hours on 12 December, close Powerhouse One Ice and Trash Sluiceway so that contractors can replace hydrophones at units 6 and 7. The expected duration of the work is 1-3 hours. If necessary, this operation may be repeated on 13 December.

  - A teletype sent on 12 December requests that on 15 December from 1500-1600, close spill bay 1 so that contractors can perform a survey of hydrophone trolley pipes in the spillway forebay.

JERRY CARROLL

Operations Project Manager