### CENWP-OD-B																						25 December 2011

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 51 of 2011, which covers the period from 18 through 24 December 2011.

- OPERATION SUMMARY

  - Daily average river flows ranged from 129.2 to 134.0 kcfs.  Daily average project tailwater ranged from 11.9’ to 12.2’ msl.  Daily average powerhouse forebay elevation ranged from 72.9’ to 74.1’ msl.  Daily average river temperature ranged from 42 to 44ºF. Secchi disk measurements remained invariable at 7.0’ all week.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. As of 12 December, spill bay 1 remains 6” open during daylight hours.  Spill bay 1 was closed from 1148 until 1306 on 21 December for an ROV inspection. Spill bay 18 remains on sill due to B-branch dewatering.  Daily average spill ranged from 0.4 to 0.5 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 5 went out of service at 0903 on 05 December 2011 for a 5 year overhaul.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 16 went out of service at 0452 on 04 September 2011 due to exciter failure.

    - Unit 18 went out of service at 1030 on 23 November 2011 for a 4 year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - F1 went out of service at 0003 on 14 October 2011 for WA shore repairs and headgate maintenance.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 1-1, 3-7, and 3-9 were closed on 12 December for Bradford Island dewatering. FV 4-3 and 4-4 remain closed.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STSs removed. |

  - Dewatering and Fish Salvage. On 22 December Project Biologists recovered 200-300 white sturgeon from the Unit 18 scroll case. These fish were released into the forebay in fair condition. There were 33 sturgeon mortalities resulting from this scroll case entrapment. A memorandum for the record is forthcoming.

- RESEARCH

  - WDFW - Fish Counting.  Video counting continues.

- FISHWAYS

  - Project biologists inspected 19, 20, and 21 December.

  - Fish Passage Plan Observations.

Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) – Dewatered. |  |  |  |  |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Nothing to report. |  |  |  |  |  |

    - PH1CC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 15 December.

    - 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance. Stab plates were installed on 07 and 08 December in accordance with the FPP for 1 Fish Unit operations.

    - SLEDs were installed at WA shore and Cascades Island entrance gates on 09 December.

    - B-branch remains dewatered. Bradford Island serpentine section and A-branch were dewatered on 13 and 14 December, respectively.

    - The AFF remains dewatered.

    - WA shore ladder returned to service on 08 December with 1 Fish Unit operations as outlined in table BON-13 in the FPP.

  - Juvenile Fishways.

    - DSM2 remains dewatered. STSs are removed.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains winterized.

    - The B2CC remains closed.  All B2CC and Cascades Island avian lines were removed to facilitate barge access for B-branch emergency repairs. They will be reinstalled during winter maintenance 2011/12.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPS systems remain out of service for the season.

- CONSTRUCTION

  - B2CC Hoist. Construction continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 23 November states in order to keep the latest established Bon tailwater flow range for chum, total project flow may be decreased to as low as 58 kcfs as long as the estimated flow at the confluence of the Columbia and Willamette rivers is equal to or exceeds 77 kcfs.  The 58 kcfs and 77 kcfs is a hard constraint navigation requirement.

  - A teletype sent on 21 December replaces a previous teletype for operation of BON tailwater to provide adequate spawning conditions for Chum salmon. Hard constraint 1: Maintain tailwater at a minimum of 11.7’ all hours. Hard constraint 2: Target a tailwater elevation of 12’, and maintain tailwater within 0.8’ band of 11.7’-12.5’ daily from 0630-1700 hours.

  - A teletype sent on 20 December states that on 21 December close spill bay 1 so that contractors can perform a survey of hydrophone trolley pipes in the spillway forebay. The expected duration of the work is no longer than 1 hour.

JERRY CARROLL

Operations Project Manager