### CENWP-OD-B																						04 March 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 09 of 2012, which covers the period from 26 February through 03 March 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 145.7 to 185.5 kcfs.  Daily average project tailwater ranged from 12.8’ to 15.9’ msl.  Daily average powerhouse forebay elevation ranged from 74.0’ to 75.0’ msl.  Daily average river temperature ranged from at 38 to 40ºF. Secchi disk measurements ranged from 5.0’ to 7.0’.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Spill bays remain 6” open during daylight hours. Daily average spill ranged from 1.1 to 2.3 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 8 was out of service from 0700 until 1034 on 29 February for a trolley pipe repair dive.

    - Unit 9 was out of service from 0700 until 1036 on 29 February for a trolley pipe repair dive.

    - Unit 10 was out of service from 0700 until 1036 on 29 February for a trolley pipe repair dive.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 12 was out of service from 1249 until 1500 on 28 February for a trolley pipe repair dive. The unit was also out of service from 1012 until 1022 on 02 March for the Unit 11 spider move.

    - Unit 13 was out of service from 1255 until 1513 on 28 February for a trolley pipe repair dive. The unit was also out of service from 1007 until 1020 on 02 March for the Unit 11 spider move.

    - Unit 14 was out of service from 1311 until 1510 on 28 February for a trolley pipe repair dive. The unit was also out of service from 1001 until 1013 on 02 March for the Unit 11 spider move.

    - Unit 15 was out of service from 0954 until 1006 on 02 March for the Unit 11 spider move.

    - Unit 16 went out of service at 0452 on 04 September 2011 due to exciter failure. It was returned to service at 1142 on 02 March 2012.

    - Unit 17 was out of service from 0709 until 1233 on 28 February for a trolley pipe repair dive. The unit was also out of service from 0944 until 0956 on 02 March for the Unit 11 spider move.

    - Unit 18 went out of service at 1030 on 23 November 2011 for a 4 year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - Fish Unit 1 was out of service from 0026 on 27 February until 1227 on 29 February for dive operations.

    - Fish Unit 2 was out of service from 0026 on 27 February until 1448 on 28 February for dive operations.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 4-4/3 were closed from 0721 until 1033 on 01 March for a trolley pipe repair dive.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 01 April |

  - Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - Battelle - survival study.  Surgeon/tagging training and fish condition research continues at the JMF. Fish for training currently held in raceways at the SMF.

  - FFU - Sea lion.  Observations began on 06 January. Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2012/sea_lion_hazing2012.html

  - ODFW/WDFW - Sea lion trapping. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Acoustic tagging and branding began on 06 February.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring began on 01 March. No fallbacks were recorded crossing the separator bars this week.

  - USDA - Sea lion hazing. Hazing from the deck began on 01 March.

  - WDFW - Fish Counting.  Video counting continues. Visual counting scheduled to begin on 01 April.

- FISHWAYS

  - Project biologists inspected 27 February, 01, 02, and 03 March. NOAA inspected 02 March.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 03/01 | B-branch | Entrance diff < 1.0’ | Dive in spillway FB | N/A | N/A |
| 03/01 | Bay 18 | No attraction flow | Dive in spillway FB | N/A | N/A |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 02/27 | Ladder entrances | Entrance diff < 1.0’ | Fish unit dive operations | N/A | N/A |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 03 March.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Stab plates were installed on 07 and 08 December in accordance with the FPP for 1 Fish Unit operations.

    - All SLEDs are installed.

    - All ladders remain in service.

    - The AFF remains dewatered.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

      - The ITS was closed from 0640 until 1024 on 29 February for a trolley pipe repair dive at PH1.

    - The hydrocannon was returned to service the week of 26 February.

    - The B2CC remains closed.

    - All B2CC and Cascades Island avian lines were removed to facilitate barge access for B-branch emergency repairs. They will be reinstalled as soon as maintenance schedules allow.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPS systems remain out of service for the season.

- CONSTRUCTION

  - B2CC Hoist. Construction is complete. Final testing and modifications continue.

  - Spillway Debris Removal. Rock removal began on 02 March.

  - Seawater Recirculation System Removal (JMF). Scheduled to commence on 05 March.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 23 November states in order to keep the latest established Bon tailwater flow range for chum, total project flow may be decreased to as low as 58 kcfs as long as the estimated flow at the confluence of the Columbia and Willamette rivers is equal to or exceeds 77 kcfs.  The 58 kcfs and 77 kcfs is a hard constraint navigation requirement.

  - A teletype sent on 28 December replaces a previous teletype for operation of BON tailwater to provide adequate spawning conditions for Chum salmon. As a hard constraint, maintain tailwater at a minimum of 12.0’ all hours.

  - A teletype sent on 19 January states that, effective immediately and until further notice, operate BON to minimize spill until the project removes boulders that have accumulated in the spillway stilling basin. Operate turbine units above the 1% of the best efficiency range as necessary to pass flows. The project is authorized to operate up to the maximum load within the cavitation limit. If flows exceed hydraulic capacity, operate the spillway as necessary according to the spill pattern defined in “BON Special Spill Pattern – Jan. 19, 2012”.

  - A teletype sent on 24 February states that, effective at 1000 hours on 27 February, through 1800 hours on 31 March, BPA is requested to provide at least one-hour notification to BON control room prior to spill. A day-ahead advisory of possible spill is also requested, when possible, in order to inform the contractor at the site of possible events and evacuations of the stilling basin work area.

  - A teletype sent on 28 February requests to operate BON with a depressed forebay commencing 1000 hours on 29 February, continuing through 1800 hours on 31 March. A depressed forebay elevation gives buffer space, allowing for sufficient time needed to evacuate workers from the stilling basin in the event of an emergency.

  - A teletype sent on 01 March outlines the spill priority list order for the Columbia basin projects with associated total dissolved gas spill caps.

  - A teletype sent on 02 March states that, effective immediately, switch to PH1 priority and operate available units in priority order as defined in the 2012 FPP. Operate the B2CC according to the following criteria: Open the B2CC within 1 hour of notification that the JMF has counted 2 kelts per day for 2 consecutive days with a cumulative total of 20 kelts.

JERRY CARROLL

Operations Project Manager