### CENWP-OD-B																						11 March 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 10 of 2012, which covers the period from 04 through 10 March 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 148.5 to 178.3 kcfs.  Daily average project tailwater ranged from 13.9’ to 15.8’ msl.  Daily average powerhouse forebay elevation ranged from 73.8’ to 74.9’ msl.  Daily average river temperature ranged from at 40 to 42ºF. Secchi disk measurements ranged from 5.0’ to 7.0’.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Spill bays remain 6” open during daylight hours. Daily average spill ranged from 1.1 to 2.3 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 1 was out of service from 0701 until 0728 on 07 March for raking trash.

    - Unit 2 was out of service from 1353 until 1638 on 06 March for raking trash.

    - Unit 3 was out of service from 1136 until 1400 on 06 March for raking trash.

    - Unit 4 was out of service from 0844 until 1153 on 06 March for raking trash.

    - Unit 5 was out of service from 0844 until 0847 on 06 March for raking trash.

    - Unit 6 was out of service from 1531 until 1638 on 05 March, and from 0700 until 0726 on 06 March for raking trash.

    - Unit 7 was out of service from 1312 until 1540 on 05 March for raking trash.

    - Unit 8 was out of service from 1129 until 1315 on 05 March for raking trash.

    - Unit 9 was out of service from 1023 until 1133 on 05 March for raking trash.

    - Unit 10 was out of service from 0452 until 1153 on 05 March for slip ring maintenance.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 18 went out of service at 1030 on 23 November 2011 for a 4 year overhaul.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 4-4/3 were closed from 1344 until 1528 on 09 March to support a hydro acoustic survey near B-branch entrance.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 01 April |

  - Dewatering and Fish Salvage. The following dewatering or fish salvage activities occurred this week:

    - On 7 March, Project Biologists recovered 12 yearling chinook, 20 sculpin, 7 small mouth bass, and 25 crayfish during the Navlock dewatering. All fish were released at the Hamilton Island boat ramp in good condition.

- RESEARCH

  - Battelle - survival study.  Surgeon/tagging training and fish condition research continue at the JMF. Fish for training currently held in raceways at the SMF.

  - CRITFC - Sea lion hazing.  Hazing from boats began on 05 March.

  - FFU - Sea lion.  Observations began on 06 January. Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2012/sea_lion_hazing2012.html

  - ODFW/WDFW - Sea lion trapping. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Acoustic tagging and branding began on 06 February.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring began on 01 March. Fallbacks recorded crossing the separator bars this week: 3 steelhead. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm

  - USDA - Sea lion hazing. Hazing from the deck began on 01 March.

  - WDFW - Fish Counting.  Video counting continues. Visual counting scheduled to begin on 01 April.

- FISHWAYS

  - Project biologists inspected 04 through 06 and 08 through 10 March.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE | ##### TR# |
| --- | --- | --- | --- | --- | --- |
| 03/07 | BON | Missed inspection | Short staffed | N/A | N/A |
| BRADFORD ISLAND (BI) |  |  |  |  |  |
| 03/04 | B-branch | Entrance diff > 2.0’ | Unknown | N/A | N/A |
| 03/09 | B-branch | Entrance diff < 1.0’ | Hydro survey | N/A | N/A |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) – Nothing to report. |  |  |  |  |  |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November. Temperatures are unavailable this week due to technical difficulties.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 10 March.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Stab plates were installed on 07 and 08 December in accordance with the FPP for 1 Fish Unit operations.

    - All SLEDs are installed.

    - All ladders remain in service.

    - The AFF remains dewatered.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon was returned to service the week of 26 February.

    - The B2CC remains closed.

    - All B2CC and Spillway avian lines were removed to facilitate barge access for B-branch emergency repairs. They will be reinstalled as soon as maintenance schedules allow.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPS systems remain out of service for the season.

- CONSTRUCTION

  - B2CC Hoist. Construction is complete. Final testing and modifications continue.

  - Spillway Debris Removal. Rock removal began on 02 March and finished on 09 March.

  - Seawater Recirculation System Removal (JMF). Commenced on 05 March.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 23 November states in order to keep the latest established Bon tailwater flow range for chum, total project flow may be decreased to as low as 58 kcfs as long as the estimated flow at the confluence of the Columbia and Willamette rivers is equal to or exceeds 77 kcfs.  The 58 kcfs and 77 kcfs is a hard constraint navigation requirement.

  - A teletype sent on 28 December replaces a previous teletype for operation of BON tailwater to provide adequate spawning conditions for Chum salmon. As a hard constraint, maintain tailwater at a minimum of 12.0’ all hours.

  - A teletype sent on 19 January states that, effective immediately and until further notice, operate BON to minimize spill until the project removes boulders that have accumulated in the spillway stilling basin. Operate turbine units above the 1% of the best efficiency range as necessary to pass flows. The project is authorized to operate up to the maximum load within the cavitation limit. If flows exceed hydraulic capacity, operate the spillway as necessary according to the spill pattern defined in “BON Special Spill Pattern – Jan. 19, 2012”.

  - A teletype sent on 01 March outlines the spill priority list order for the Columbia basin projects with associated total dissolved gas spill caps.

  - A teletype sent on 06 March states that, effective immediately, switch to PH2 priority and operate available units in priority order as defined in the 2012 FPP. Operate the B2CC according to the following criteria: Open the B2CC within 1 hour of notification that the JMF has counted 2 kelts per day for 2 consecutive days with a cumulative total of 20 kelts.

JERRY CARROLL

Operations Project Manager