### CENWP-OD-B																						22 April 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 16 of 2012, which covers the period from 15 through 21 April 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 290.7 to 323.3 kcfs.  Daily average project tailwater ranged from 22.3’ to 24.1’ msl.  Daily average powerhouse forebay elevation ranged from 73.0’ to 74.1’ msl.  Daily average river temperature ranged from 47 to 48ºF. Secchi disk measurements ranged from 3.0’ to 4.0’.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively, and remain at least 6” open during daylight hours. Daily average spill ranged from 93.0 to 95.4 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 0 was out of service from 1100 on 09 April until 1124 on 18 April for a semi-annual overhaul.

    - Unit 5 was out of service from 1251 until 1336 on 16 April due to a line trip.

    - Unit 6 was out of service from 1251 until 1337 on 16 April due to a line trip.

    - Unit 7 was out of service from 1251 until 1341 on 16 April due to a line trip.

    - Unit 8 was out of service from 1251 until 1342 on 16 April due to a line trip.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 1-1 was closed to float trash from 2050 on 19 April until 0430 on 20 April, from 2151 on 20 April until 0350 on 21 April, and from 2300 until 0404 on 22 April.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 08 April |

  - Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - Battelle – Surgeon Training/Assessment. Surgeon/tagging training at the JMF scheduled to conclude 27 April. Fish for training are currently held in raceways at the JMF. Surgeon assessment and Sort-By-Code activities will continue through July.

  - Battelle - Survival Study. Hydrophone installation completed on 21 March.

  - CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF scheduled to begin 23 April.

  - CRITFC - Sea Lion Hazing.  Hazing continues.

  - FFU - Sea Lion.  Observations continue. Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2012/sea_lion_hazing2012.html

  - NOAA – Juvenile Steelhead. Barged and in-river migrant sampling scheduled to begin in May.

  - NOAA - Lamprey. Personnel moved the lamprey trap davit at the South Monolith SDE closer to the entrance allowing the trap to sit adjacent to attraction flow.

  - ODFW – NPMP. Sampling in the BRZ scheduled for 25 and 26 April.

  - ODFW/WDFW - Sea lion. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Acoustic tagging and branding began on 06 February. NOAA Fisheries authorized permanent removal of specific California sea lions effective 20 March.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring began on 01 March. Sort by Code is active. Fallbacks recorded crossing the separator bars this week: 10 steelhead and 3 unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. The second Spring Creek release of approximately 5 million Tule fall Chinook occurred on 13 April and began arriving at the JMF at 0100 on 14 April, approximately 15 hours after release. Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - USDA - Sea Lion/Avian hazing. Hazing continues.

  - WDFW - Fish Counting.  Visual counting continues.

- FISHWAYS

  - Project biologists inspected 15 through 21 April.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| SPILLWAY |  |  |  |  |
| 04/15 | Bay 2 | Incorrect spill pattern | Failed hoists | Under repair |
| BRADFORD ISLAND (BI) – Nothing to report. |  |  |  |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |
| All week | PH2 Entrances | Gates < 13’ submergence | Unknown | Investigation |
| 04/16,18 | Weir 38 | Water level < 0.9’ | Unknown | Adjusted Valve #2 |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November. Temperatures are unavailable this week due to technical difficulties.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 21 April.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Sea lion barriers are in place on all installed FOGs.

    - All SLEDs are installed.

    - All ladders remain in service.

    - The AFF remains in bypass.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains in service.

    - The B2CC remains open.

    - The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPS systems remain out of service for the season.

- CONSTRUCTION

  - Seawater Recirculation System Removal (JMF). Removal continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES AND OTHER NOTEWORTHY ITEMS

  - A teletype sent on 23 November states in order to keep the latest established Bon tailwater flow range for chum, total project flow may be decreased to as low as 58 kcfs as long as the estimated flow at the confluence of the Columbia and Willamette rivers is equal to or exceeds 77 kcfs.  The 58 kcfs and 77 kcfs is a hard constraint navigation requirement.

  - A teletype sent on 28 December replaces a previous teletype for operation of BON tailwater to provide adequate spawning conditions for Chum salmon. As a hard constraint, maintain tailwater at a minimum of 12.0’ all hours.

  - A teletype sent on 09 April states that, effective from 0001 hours on 10 April until 2400 hours on 15 June, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the spring season at BON, as follows: 100 kcfs for 24 hours per day following the spill pattern defined in the 2012 Fish Passage Plan. This teletype was updated on 20 April to define minimum generation requirements.

  - A teletype sent on 13 April states that, effective immediately and extending through 1700 hours on 18 April, operate PH2 units at 25% of the 1% operating range while maintaining spring spill rates. This operation is intended to improve passage conditions through the PH2 gatewells during the downstream migration of approximately 8 million juvenile chinook released from Spring Creek Nation Fish Hatchery on 11 and 13 April.

  - A teletype sent on 17 April updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.

JERRY CARROLL

Operations Project Manager