### CENWP-OD-B																						13 May 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 19 of 2012, which covers the period from 06 through 12 May 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 336.7 to 394.9 kcfs.  Daily average project tailwater ranged from 25.1’ to 27.9’ msl.  Daily average powerhouse forebay elevation ranged from 73.4’ to 74.7’ msl.  Daily average river temperature ranged from 51 to 52ºF. Secchi disk measurements ranged from 2.8’ to 3.0’.

  - Spring Spill for fish passage is in effect until 15 June. Daily average spill ranged from 114.0 to 173.4 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 8 was out of service from 0121 on 07 May until 1129 on 10 May for an annual overhaul.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

    - Unit 18 went out of service at 0049 on 12 May for an oil leak in governor piping.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - Fish Units 1 and 2 were out of service from 1914 until 2054 on 07 May to float trash.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS closures occurred this reporting week.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | Inspection scheduled for week of 21 May |

  - Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - Battelle/PNNL – Surgeon Training/Assessment. Surgeon assessment and Sort-By-Code activities will continue through July.

  - Battelle/PNNL - Survival Study. Hydrophones remain installed.

  - CRITFC – AFF.  Annual Chinook, Coho, Sockeye and Steelhead sampling in the AFF began 23 April.

  - CRITFC - Sea Lion Hazing.  Hazing continues.

  - FFU - Sea Lion.  Observations continue. Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2012/sea_lion_hazing2012.html

  - NOAA – Juvenile Steelhead. Barged and in-river migrant sampling began 03 May.

  - NOAA - Lamprey. Personnel moved the lamprey trap davit at the South Monolith SDE closer to the entrance allowing the trap to sit adjacent to attraction flow. LPS count network installation continues.

  - NOAA – Pathology. 25 yearling Chinook and 25 Coho were collected at the JMF for research this week.

  - ODFW/WDFW - Sea lion. Sea lion traps remain downstream of the old Navlock. Two traps were moved to the forebay, upstream of the old Navlock in an effort to capture the sea lions above Bonneville.  Acoustic tagging and branding began on 06 February. NOAA Fisheries authorized permanent removal of specific California sea lions effective 20 March.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Sort by Code is active for PNNL and NOAA. Fallbacks recorded crossing the separator bars this week: 50 steelhead, 16 salmon, and 17 unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - USDA - Sea Lion/Avian hazing. Hazing continues.

  - WDFW - Fish Counting.  Visual counting continues.

- FISHWAYS

  - Project biologists inspected 06 through 12 May.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE | ##### RESPONSE |
| --- | --- | --- | --- | --- |
| SPILLWAY |  |  |  |  |
| All week | Bay 17 | Incorrect height for spill pattern | Failed hoists | Under repair |
| BRADFORD ISLAND (BI) – Nothing to report. |  |  |  |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |
| All week | PH2 Entrances | Gates < 13’ submergence | Necessary to maintain differential | N/A |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November. Temperatures are unavailable this week due to technical difficulties.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 12 May.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Sea lion barriers are in place on all installed FOGs.

    - All SLEDs are installed.

    - All ladders remain in service.

    - The AFF remains in service.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains in service.

    - The B2CC remains open.

    - The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPS systems remain out of service. They are scheduled to return to service in mid May.

- CONSTRUCTION

  - Seawater Recirculation System Removal (JMF). Removal continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent on 09 April states that, effective from 0001 hours on 10 April until 2400 hours on 15 June, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the spring season at BON, as follows: 100 kcfs for 24 hours per day following the spill pattern defined in the 2012 Fish Passage Plan. This teletype was updated on 20 April to define minimum generation requirements.

  - A teletype sent on 09 May updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.

  - A teletype sent on 11 May states that, effective through 18 May, for the purposes of system flood control; make best efforts to keep BON max instantaneous flow no higher than 420 kcfs.

JERRY CARROLL

Operations Project Manager