### CENWP-OD-B																						24 June 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 25 of 2012, which covers the period from 17 through 23 June 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 325.2 to 367.2 kcfs.  Daily average project tailwater ranged from 23.9 to 26.0’ msl.  Daily average powerhouse forebay elevation ranged from 73.1’ to 74.2’ msl.  Daily average river temperature ranged from 59 to 62ºF. Secchi disk measurements ranged from 4.0’ to 5.0’.

  - Summer Spill for fish passage is in effect. Daily average spill ranged from 116.6 to 170.9 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 3 went out of service at 0000 on 18 June for Bank 3/4 outage and annual maintenance.

    - Unit 4 went out of service at 0007 on 18 June for Bank 3/4 outage.

    - Unit 11 has remained out of service since 12 June 2008 due to rotor damage. The unit returned to service briefly on 21 June for load rejection testing, with a heat run scheduled for the week of 25 June

    - Unit 12 was out of service from 1215 until 1334 on 18 June due to a loss of preferred AC.

    - Unit 13 was out of service from 0733 until 0809 on 20 June for raking trash.

    - Unit 14 went out of service at 0710 on 11 June for a 4 year overhaul.

    - Unit 15 was out of service from 0819 until 0856 on 20 June for raking trash.

    - Unit 16 was out of service from 0900 until 0931 on 20 June for raking trash.

    - Unit 17 was out of service from 1001 until 1040 on 20 June for raking trash.

    - Unit 18 was out of service from 1043 until 1148 on 20 June for raking trash.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 1-1 was closed for cleaning from 0700 until 0710 on 21 June.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | NA | OOS | U11 remains out of service. |
| 12-18 | NA | NA | STS inspection truck is currently OOS. |

  - Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - Battelle/PNNL – Surgeon Training/Assessment. Surgeon assessment and Sort-By-Code activities will continue through July.

  - Battelle/PNNL - Survival Study. Hydrophones remain installed.

  - CRITFC – AFF.  Annual Chinook, Coho, Sockeye and Steelhead sampling in the AFF continues.

  - NOAA - Lamprey. LPS count network installed. Trapping at the CI LCS, WA shore NDE, and the AFF continues.

  - NOAA – Pathology. No fish were collected at the JMF for research this week.

  - ODFW/WDFW - Sea lion. Sea lion traps remain downstream of the old Navlock, and in the forebay, upstream of the old Navlock.  Trapping has concluded for the season.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Sort by Code is active for PNNL and NOAA. Fallbacks recorded crossing the separator bars this week: 5 steelhead, 8 salmon, and 5 unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - USDA - Avian hazing. Avian hazing continues.

  - University of Idaho. Cameras installed behind the Bradford Island picket leads to monitor lamprey and salmonid behavior. Lamprey trapping in conjunction with NOAA.

  - WDFW - Fish Counting.  Visual counting continues. Nighttime video lamprey counting began 15 June.

- FISHWAYS

  - Project biologists inspected 17 through 23 June.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| DATE | ##### LOCATion | ##### FPP itEMS | ##### CAUSE |  | ##### RESPONSE |
| --- | --- | --- | --- | --- | --- |
| SPILLWAY |  |  |  |  |  |
| All week | Bay 17 | Incorrect height | Hoist OOS | None |  |
| BRADFORD ISLAND (BI) – Nothing to report. |  |  |  |  |  |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |  |
| 6/20 | UMT fishway | Staff gauge < 1.2’ | N/A |  | Auto adjust |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November. Temperatures are unavailable this week due to technical difficulties.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 22 June.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Sea lion barriers are in place on all installed FOGs.

    - All SLEDs are installed.

    - All ladders remain in service.

      - All ladders were switched to Shad Mode on 04 June, congruous with FPP 2.5.1.2a

    - The AFF remains in service.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains in service.

    - The B2CC remains open.

    - The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPSs and associated traps remain in service.

- CONSTRUCTION

  - Seawater Recirculation System Removal (JMF). Removal continues.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent on 14 June states that, effective from 0001 hours on 16 June until 2400 hours on 31 August, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the summer season at BON, as follows: 16 June – 21 July: Alternate between 2-day treatments of 95 kcfs 24 hours per day, and 85 kcfs day/121 kcfs night. 21 July – 31 August: 75 kcfs day/Gas cap night.

  - A teletype sent on 15 June requests that the BON pool be operated within a 1.5’ band, soft constraint, from 0600 hours 18 June to 1800 hours 21 June, and from 0600 hours 25 June to 1800 hours 28 June, during treaty fishing.

  - A teletype sent on 21 June updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.

JERRY CARROLL

Operations Project Manager