### CENWP-OD-B																						05 August 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 31 of 2012, which covers the period from 29 July through 04 August 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 232.2 to 264.6 kcfs.  Daily average project tailwater ranged from 19.2’ to 20.6’ msl.  Daily average powerhouse forebay elevation ranged from 73.4’ to 74.4’ msl.  Daily average river temperature ranged from 67 to 69ºF. Secchi disk measurements ranged from 4.0’ to 6.0’.

  - Summer Spill for fish passage is in effect. Daily average spill ranged from 89.5 to 138.4 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 4 was out of service from 0400 on 1 August until 1622 on 3 August for BPA power generation restrictions.

    - Unit 10 went out of service at 0928 on 09 July for a 5 year overhaul.

    - Unit 11 went out of service at 0005 on 06 July 2012 for governor trouble.

    - Unit 12 was out of service from 1008 on 30 July to 0244 on 31 July and from 1005 on 31 July to 0336 on 1 August for BPA line work.

    - Unit 13 was out of service from 1950 to 1957 on 28 July for reserve shutdown and 1003 on 30 July to 31 July and 0958 on 31 July to 0339 on 1 August for BPA line work.

    - Unit 14 went out of service at 0710 on 11 June for a 4 year overhaul.

    - Unit 15 was out of service from 0954 on 15 July to 0239 on 31 July and from 0954 on 31 July to 0339 on 1 August for BPA line work.

    - Unit 16 was out of service from 0959 on 30 July to 0238 on 31 July and from 0955 on 31 July to 0338 on 1 August for BPA line work.

    - Unit 17 was out of service from 1003 on 30 July to 0236 on 31 July and from 0959 on 31 July to 0338 on 1 August for BPA line work; and at 0735 on 1 August for air gap monitor installation.

    - Unit 18 was out of service from 0639 to 0944 on 28 July for STS repair; from 1008 on 30 July to 0246 on 31 July and from 1010 on 31 July to 0338 on 1 August for BPA line work.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - Fish Unit 1 was out of service from 0909 on 30 July to 0314 on 31 July and from 0803 on 31 July to 0408 on 1 August for BPA line work.

    - Fish Unit 2 was out of service from 2158 on 28 July to 0429 on 29 July for lamprey operations; from 2159 on 29 July until 0407 on 30 July to float trash; from 1005 on 30 July to 0501 on 1 August and from 0422 on 1 August to 0441 on 2 August for BPA line work; from 2151 on 2 August to 0450 on 3 August and at 2147 on 3 August for lamprey operations.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS closures occurred this reporting week.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11,12,18 | 7/25-27 | NA | STSs inspected and repaired; remain installed |
| 13-17 | 7/25-27 | NA | STSs removed |

  - Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - Battelle/PNNL – Surgeon Training/Assessment. Surgeon assessment and Sort-By-Code activities will continue through August.

  - Battelle/PNNL - Survival Study. Hydrophones remain installed.

  - CRITFC – AFF.  Annual Chinook, Coho, Sockeye and Steelhead sampling in the AFF continues.

  - NOAA - Lamprey. LPS count network installed. Trapping at the CI LCS and the AFF continues.

  - NOAA – Pathology. 25 subyearling Chinook were collected at the JMF for research this week.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Sort by Code is active for PNNL. Fallbacks recorded crossing the separator bars this week: 5 steelhead, 1 salmon, and 3 unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - USDA - Avian hazing. Avian hazing concluded on 30 July.

  - University of Idaho. Camera monitoring behind the Bradford Island picket leads has concluded. Lamprey trapping in conjunction with NOAA continues.

  - WDFW - Fish Counting.  Visual counting continues. Nighttime video lamprey counting continues.

- FISHWAYS

  - Project biologists inspected 29 July through 04 August. NOAA inspected on 03 August.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| Date | Location | FPP Items | Cause | Response |
| --- | --- | --- | --- | --- |
| SPILLWAY |  |  |  |  |
| 7/30, 31 | Bay 17 | Incorrect height | Hoist OOS | None |
| BRADFORD ISLAND (BI) |  |  |  |  |
| 7/29, 31; 8/1 | B-branch staff gauge | Water level < 0.9’ | FV 3-9 needs calibration | Electricians calibrated FV 3-9 |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |
| 7/30-31 | PH2 MU 11-18, FU 1 & 2 | OOS | BPA substation maintenance | N/A |
| 7/30-31 | PH2 Entrances | No differential | BPA substation maintenance | N/A |
| 8/2, 3 | Weir 37 | Water level < 0.9’ | Unknown | Ops adjusted |
| 8/4 | NDE | Submergence < 13’ | Gate will not lower | ROV inspection scheduled |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November. BON site specific temperatures are unavailable this week due to technical difficulties. The temperatures below are provided via this website and show the averages for BON forebay.

| Date | Temperature (F) |
| --- | --- |
| 7/29 | 66.9 |
| 7/30 | 66.8 |
| 7/31 | 66.6 |
| 8/01 | 66.7 |
| 8/02 | 66.8 |
| 8/03 | 67.3 |
| 8/04 | 67.8 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 03 August.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance.

    - SLEDs are removed from all entrances.

    - All ladders remain in service.

    - The AFF remains in service.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains in service.

    - The B2CC remains open.

    - The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPSs and associated traps remain in service.

- CONSTRUCTION

  - Seawater Recirculation System Removal (JMF). Removal continues.

  - Cascades Island LPS Continuation. Construction on the exit section began on 30 July. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.

  - Gantry 7 Rehabilitation. The Powerhouse 2 intake gantry crane went out of service on 01 August for rehabilitation. It is scheduled to return to service in February 2013.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent on 14 June states that, effective from 0001 hours on 16 June until 2400 hours on 31 August, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the summer season at BON, as follows: 16 June – 21 July: Alternate between 2-day treatments of 95 kcfs 24 hours per day, and 85 kcfs day/121 kcfs night. 21 July – 31 August: 75 kcfs day/Gas cap night.

  - A teletype sent on 25 July requests BON forebay be maintained between 75.0–76.5’ as a hard constraint and 75.5-76.5’ soft constraint from 1400 to 2000 on 11 August for a sailboat race near The Dalles.

  - A teletype sent on 01 August updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.

  - A teletype sent on 30 July states that BON project may operate PH2 unscreened units (Units 13-17) as necessary to pass flows. This operation is necessary due to the rehabilitation of Gantry 7. During the Gantry 7 outage, the project will not be able to remove or install screens. Additionally, limit flow through PH2 screened units (Units 11, 12, and 18) to a maximum of 15 kcfs per unit. The intent of this limitation is to minimize the change of damaging an operating screen during the Gantry 7 outage, which would force the unit out of service until February 2013.

JERRY CARROLL

Operations Project Manager