### CENWP-OD-B																						19 August 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 33 of 2012, which covers the period from 12 through 18 August 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 181.0 to 225.9 kcfs.  Daily average project tailwater ranged from 15.6’ to 18.5’ msl.  Daily average powerhouse forebay elevation ranged from 73.5’ to 74.8’ msl.  Daily average water temperature ranged from 71 to 73ºF. Secchi disk measurements remained at, approximately, 6.0’ all week.

  - Summer Spill for fish passage is in effect. Daily average spill ranged from 87.4 to 89.9 kcfs.

  - Unit Operation.  Powerhouse Two is the priority powerhouse.

    - Unit 10 went out of service at 0928 on 09 July for a 5 year overhaul.

    - Unit 11 went out of service at 0005 on 06 July 2012 for governor trouble.

    - Unit 12 went out of service at 2150 on 05 August for annual maintenance and T12 outage. The unit returned to service at 1249 on 16 August.

    - Unit 13 went out of service at 2151 on 05 August for T11 outage. The unit returned to service at 1247 on 16 August.

    - Unit 14 went out of service at 0710 on 11 June for a 4 year overhaul.

    - Unit 15 was out of service from 1002 until 1352 on 13 August, and from 1000 until 1254 on 16 August for 2-SQ2 BUS outage.

    - Unit 17 went out of service at 0735 on 01 August for air gap monitor installation.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - Fish Unit 1 was out of service from 1155 until 1234 on 16 August for 2-SQ2 BUS outage.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV 1-1 was closed from 0721 until 0750 on 13 August, and 0843 until 1025 on 16 August for cleaning.

    - FV 3-9 was closed from 0706 until 0709 on 13 August, and from 0716 until 0731 on 16 August for cleaning.

    - FV 4-3 was out of service from 1000 on 13 August until 1600 on 14 August to inspect FV 4-4.

    - FV 4-4 was out of service from 1000 on 13 August until 1600 on 14 August for inspection.

    - FV 5-9 was closed from 0812 until 0820 on 16 August for cleaning.

    - FV 6-9 was closed from 0812 until 0820 on 16 August for cleaning.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11,12,18 | N/A | N/A | STSs installed |
| 13-17 | N/A | N/A | STSs removed |

  - Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - Battelle/PNNL – JMF Juvenile Assessment. Sort-By-Code activities concluded on 04 August. Juvenile monitoring may continue through September.

  - Battelle/PNNL – Survival Study. Hydrophones remain installed.

  - CRITFC – AFF.  Annual Chinook, Coho, Sockeye and Steelhead sampling in the AFF continues. Sampling was suspended on 13 August due to temperatures reaching 70F. Sampling resumed on 16 August with modified procedures for sampling above 70F.

  - NOAA – Lamprey. LPS count network installed. Trapping at the CI LCS and the AFF concluded on 08 August in anticipation of temperature restraints. Trapping is scheduled to resume 21 August.

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Modified sampling procedures for temperatures above 70F were enacted on 14 August. Truck releases of summer out-migrants originating from McNary, Lower Granite, Little Goose, and Lower Monumental dams are scheduled to begin 18 August. Fallbacks recorded crossing the separator bars this week: 1 steelhead, 1 salmon, and 1 unknown salmonid. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.

  - University of Idaho. Lamprey trapping in conjunction with NOAA was halted on 08 August in anticipation of temperature restraints. Trapping is scheduled to resume 22 August.

  - WDFW – Fish Counting.  Visual counting continues. Nighttime video lamprey counting continues.

  - WDFW – Tule Counting. Began on 15 August.

- FISHWAYS

  - Project biologists inspected 12 through 18 August.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| Date | Location | FPP Items | Cause | Response |
| --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |
| 8/16 | A-branch | Staff gauge > 1.1’ | Unknown | None |
| 8/17 | A-branch | FG3-5 open w/<16.7 TW | Burnt motor | TR in place |
| 8/13, 17, 18 | B-branch | Ent. differential > 2.0’ | FV 4-4 damaged | Investigation |
| 8/14 | B-branch | No ent. differential | FV 4-4 OOS | Investigation |
| 8/13 | B-branch | Incorrect diffusers open | FV 4-4 damaged | Investigation |
| 8/13 | B-branch | South entrance gate open | FV 4-4 damaged | Investigation |
| CASCADES ISLAND (CI) – Nothing to report. |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |
| 8/12, 14, 15, 18 | Weir 37 | Water level < 0.9’ | W37 bleed-off incorrect position | Ops adjusted |
| 8/14-18 | NDE | Submergence < 13’ | Gate will not lower | N/A |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures.  Temperatures are taken 01 March through 30 November. BON site specific temperatures are unavailable this week due to technical difficulties. The temperatures below are provided via this website and show the averages for BON forebay.

| Date | Temperature (F) |
| --- | --- |
| 8/12 | 69.8 |
| 8/13 | 70.3 |
| 8/14 | 70.5 |
| 8/15 | 70.7 |
| 8/16 | 70.9 |
| 8/17 | 71.3 |
| 8/18 | 71.3 |

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 17 August.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance.

    - SLEDs are removed from all entrances.

    - All ladders remain in service.

    - The AFF remains in service.

  - Juvenile Fishways.

    - DSM2 remains in service.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The hydrocannon remains in service.

    - The B2CC remains open.

    - The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - All LPSs and associated traps remain in service except for the WA shore NDE trap.

    - NOAA and U of I suspended tagging efforts on 08 August, but continue the trap and haul operation for the Cascades Island LPS.

    - On 16 August, NOAA personnel and fisheries biologists observed lamprey in the FV 5-9 AWS. A portable trap was installed in this area to trap lamprey and transport them, with the fish collected in the CI LPS, to the Stevenson release site.

- CONSTRUCTION

  - Seawater Recirculation System Removal (JMF). Removal continues.

  - Cascades Island LPS Continuation. Construction continues. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.

  - Gantry 7 Rehabilitation. The Powerhouse 2 intake gantry crane went out of service on 01 August for rehabilitation. It is scheduled to return to service in February 2013.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent on 14 June states that, effective from 0001 hours on 16 June until 2400 hours on 31 August, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the summer season at BON, as follows: 16 June – 21 July: Alternate between 2-day treatments of 95 kcfs 24 hours per day, and 85 kcfs day/121 kcfs night. 21 July – 31 August: 75 kcfs day/Gas cap night.

  - A teletype sent on 30 July states that BON project may operate PH2 unscreened units (Units 13-17) as necessary to pass flows. This operation is necessary due to the rehabilitation of Gantry 7. During the Gantry 7 outage, the project will not be able to remove or install screens. Additionally, limit flow through PH2 screened units (Units 11, 12, and 18) to a maximum of 15 kcfs per unit. The intent of this limitation is to minimize the change of damaging an operating screen during the Gantry 7 outage, which would force the unit out of service until February 2013.

  - A teletype sent on 16 August updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.

  - A teletype sent on 17 August requests that the BON pool be operated within a 1.5’ band, hard constraint, for these periods: 0600 on 21 August to 1800 on 23 August, from 0600 on 27 August to 1800 on 30 August, from 0600 on 04 September to 1800 on 08 September. The goal of this operation is to limit pool fluctuation for treaty fishing.

  - A teletype sent on 17 August requests that BON forebay be maintained above elevation 74.0’ as a soft constraint from 1100 to 1800 hours on 19 August. This operation is necessary for an amateur kite board event at Stevenson, WA.

JERRY CARROLL

Operations Project Manager