### CENWP-OD-B																						18 November 2012

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 46 of 2012, which covers the period from 11 through 17 November 2012.

- OPERATION SUMMARY

  - Daily average river flows ranged from 135.1 to 145.6 kcfs.  Daily average project tailwater ranged from 12.4’ to 14.1’ msl.  Daily average powerhouse forebay elevation ranged from 72.7’ to 74.5’ msl.  Daily average water temperature ranged from 54ºF to 55ºF. Secchi disk measurements ranged from 6.0 to 7.0’.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Spill bay 1 is open 6” during daylight hours. Spill bay 18 is closed for B-branch repairs. Daily average spill remained at 1.1 kcfs.

  - Unit Operation.  Powerhouse One is the priority powerhouse.

    - Unit 3 went out of service at 0909 on 01 October for a 5 year overhaul. The unit was returned to service at 1419 on 15 November.

    - Unit 4 was out of service from 0855 until 1001 on 15 November for Bank 3/4 testing.

    - Unit 11 went out of service at 0800 on 25 September for high thrust bearing temperatures.

    - Unit 13 went out of service at 1100 on 16 November for oil leakage.

    - Unit 14 was out of service from 2309 on 10 November until 1026 on 13 November for oil leakage.

  - Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV5-9 was closed for cleaning from 0958 to 1004 on 16 November.

  - STS/VBS inspections.  No STS/VBS inspection occurred this reporting week.

    - Table 1.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | N/A | N/A | Unit 11 OOS |
| 12,18 | N/A | N/A | STSs installed, VBSs removed on 11/05 |
| 13-17 | N/A | N/A | STSs removed |

  - Dewatering and Fish Salvage.

    - On 13 November, 9 Chinook, 4 steelhead, 6 lamprey, 2 small-mouth bass, 12 suckers, and 10 peamouth were recovered from the WA shore fish ladder down to tailwater.  All fishes were released to the forebay in good condition.

    - On 15 November, 3 sturgeon, 1 Chinook, 1 trout, 4 small-mouth bass, 9 shad, and 10 suckers were recovered from the WA shore entrance pool. All fishes were returned to the tailrace in good condition.

- RESEARCH

  - Battelle/PNNL – Survival Study. Hydrophones remain installed.

  - NOAA – Lamprey. LPS count network installed.

  - WDFW – Fish Counting.  Visual counting concluded on 31 October.  Video counting began 01 November.

- FISHWAYS

  - Project biologists inspected 11 through 17 November.

  - Fish Passage Plan Observations.

    - Table 2.  FPP Out of Criteria items.

| Date | Location |  | FPP Items | Cause |  |  | Response |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |  |  |  |
| All week | A-branch |  | FG3-3 half open | Mechanically bound |  |  | TR in place |
| All week | BI weir |  | Water level < 0.9’ | B-branch dewatered |  |  | N/A |
| All week | A-branch |  | Water level > 1.1’ | B-branch dewatered |  |  | N/A |
| All week | B-branch ladder |  | Out of service | Dewatered to repair FV 4-4 |  |  | N/A |
| 11/12,14-17 | PH1CC |  | South differential < 1.0 | N/A |  |  | None |
| CASCADES ISLAND (CI) |  |  |  |  |  |  |  |
| 11/17 | UMT | Water level < 0.9 |  |  | N/A | None |  |
| WASHINGTON SHORE (WS) – OOS |  |  |  |  |  |  |  |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures. Temperatures are taken 01 March through 30 November. BON site specific temperatures are unavailable this week due to technical difficulties.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 16 November.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance.

    - SLEDs are removed from all entrances.

    - The WA Shore ladder was dewatered to tailwater on 13 November, and below tailwater on 15 November.

    - The Cascades Island fish ladder exit was opened on 08 November.

    - On 09 August, FV 4-4 was discovered to be broken in the full open position. Upon ROV inspection, the gate was found to be sheared from its attachment point on the wall. B-branch was dewatered from the junction pool down to the entrance during 25-27 September to facilitate repairs on FV 4-4. A-branch and the upper Bradford Island ladder remain in service and are currently being operated as close to FPP criteria as possible.

    - The AFF was dewatered on 07 November.

  - Juvenile Fishways.

    - The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The JMF was dewatered and winterized on 05 November.

    - The hydrocannon was winterized on 05 November.

    - The DSM2 was dewatered on 11 November.

    - The B2CC closed on 16 September.

    - The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2012/13.

    - The spillway avian array was removed on 11 October to facilitate barge access for spillway erosion repair. The array will be reinstalled during the winter of 2012/13.

  - Lamprey Fishways.

    - The Cascades Is. entrance weir remains installed.

    - The Cascades Is. LPS was taken out of service on 20 September.

    - The WA shore LPS was taken out of service on 11 November.

    - The Bradford Island LPS remains in service.

- CONSTRUCTION

  - Seawater Recirculation System Removal (JMF). Removal continues.

  - Cascades Island LPS Continuation. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.

  - Gantry 7 Rehabilitation. The Powerhouse 2 intake gantry crane went out of service on 01 August for rehabilitation. It is scheduled to return to service in February 2013.

  - Spillway Erosion Repair. Rock removal and erosion cavity repair continues.

  - WA Shore LPS/LFS Installation. Installation of an entrance and flume has begun. The contractor will also repair expansion joints in the north monolith AWS.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent on 23 August updates the operation of BON project PH2 unscreened units (Units 13-17) to pass flow. This operation is necessary due to the rehabilitation of Gantry 7. During the Gantry 7 outage, the project will not be able to remove or install screens. Additionally, limit flow through PH2 screened units (Units 11, 12, and 18) to a maximum of 15 kcfs per unit. The intent of this limitation is to minimize the chance of damaging an operating screen during the Gantry 7 outage, which would force the unit out of service until February 2013.

  - A teletype sent on 23 August states that effective 0001 hours on September 1, the PH2 Corner Collector (B2CC) will be operated any time one or more unscreened units are operating at PH2, with the exception of September 4 (0700-1700), when it will be closed for repairs. The B2CC will be closed on September 16 through November 11 for repairs. Additionally, on 16 September PH1 will become the priority powerhouse.  In November, MU18 will be taken out of service and the VBS will be removed from MU11 and MU12, when the downstream migrant channel (DSM2) is dewatered for installation of a lamprey passage system. At that time, those units will be operated last in the priority order.

  - A teletype sent on 30 October states that effective immediately until further notice operate the BON forebay 24 hours a day from 71.5 - 76.0’ as a hard constraint. The goal of this operation is to facilitate repair work of the Bradford Island B-branch fish ladder and to minimize the duration A-branch ladder is outside of FPP criteria, while providing BPA with some flexibility for other concurrent operational constraints.

  - A teletype sent on 25 October states that effective 25 October until further notice, BPA is requested to provide a minimum of one hour notice prior to spill, with a day-ahead notice preferred if possible due to ongoing spillway repair work and related safety concerns.

  - A teletype sent on 31 October states the flow operations necessary to maintain a daytime tailwater appropriate for daytime chum spawning during abbreviated periods of high flow and high tide, while maintaining navigational requirements. This teletype will remain in effect until chum spawning is declared complete by salmon managers.

JERRY CARROLL

Operations Project Manager