### CENWP-OD-B

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 12 of 2013, which covers the period from 17 March through 23 March 2013.

- OPERATION SUMMARY

  - Daily average river flows ranged from 128.1 to 142.0 kcfs.  Daily average project tailwater ranged from 12.0’ to 12.7’ msl.  Daily average powerhouse forebay elevation ranged from 73.3’ to 75.1’ msl.  Daily average water temperatures ranged from 43ºF to 45ºF. Secchi disk measurements ranged from 6.0’ to 7.0’ or greater throughout the week.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Daily average spill ranged from 1.2 to 1.3 kcfs.

  - Unit Operation.  Powerhouse Two became the priority powerhouse on 14 March.

    - Unit 0 went out of service at 0757 on 18 March for pipe repairs.

    - Unit 1 was out of service from 1307 to 1338 on 18 March for auto sync checks.

    - Unit 2 was out of service from 1107 to 1300 on 18 March for auto sync checks.

    - Unit 4 was out of service from 1035 to 1102 on 18 March for auto sync checks.

    - Unit 5 was out of service from 0955 to 1033 on 18 March for auto sync checks.

    - Unit 7 was out of service from 0848 to 0912 on 18 March for auto sync checks.

    - Unit 8 was out of service from 0813 to 0847 on 18 March for auto sync checks.

    - Unit 9 was out of service from 0731 to 0811 on 18 March for auto sync checks.

    - Unit 11 went out of service at 0900 on 09 September 2012 for high thrust bearing temperatures.

    - Unit 14 went out of service at 0000 on 04 March for TRD testing.

    - Unit 15 was out of service from 0003 to 1332 on 19 March to repair the gate limit.

  - Fish Units.  Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

    - Unit F2 went on standby from 1534 to 1601 on 19 March due to a water spill within the powerhouse.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  No AWS closures occurred this reporting week.

  - STS/VBS inspections.

    - Table 3.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | N/A | N/A | Unit 11 OOS |
| 14 | N/A | N/A | Unit 14 OOS for TRD testing |
| 12,18 | 03/12 | N/A | STS installed |
| 13,15 | 03/13 | N/A | STS installed |
| 16,17 | 03/14 | N/A | STS installed |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - CRITFC – Sea Lion Hazing. Hazing from boats commenced on 04 March.

  - Fisheries Field Unit – Sea Lion Observations.  Observations commenced on 04 January and will continue until the end of May.  Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2013/sea_lion_hazing2013.html

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring began on 12 March.  Fallbacks recorded crossing the separator bars this week: three steelhead and two unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm.

  - USDA - Sea Lion Hazing. Land based hazing began on 01 March and will continue until the end of May.

  - Normandeau – Fish Counting.  Video counting continues. Visual counting is scheduled to begin on 01 April. Normandeau assumed fish counting responsibilities from WDFW as of 01 March.

- FISHWAYS

  - Project biologists inspected 17 March through 23 March.

  - Fish Passage Plan Observations.

    - Table 4.  FPP Out of Criteria items.

| Date | Location | FPP Items | Cause | Response |
| --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |
| 3/20 | PH1CC | S Entrance Diff < 1.0’ | N/A | None |
| 3/22-23 | PH1CC | N Entrance Diff > 2.0’ | N/A | None |
| All week | A-branch | FG3-3 half open | Mechanically bound | TR in place |
| All week | A-branch | Water level > 1.1’ | N/A | None |
| CASCADES ISLAND (CI) – Nothing to Report. |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |
| 3/21 | PH2 S-Monolith | Entrance Diff < 1.0’ | N/A | None |
| 3/18-23 | DSM2 | Channel elev < 64.2’ | N/A | None |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.

  - Fishway Temperatures. Temperatures are normally taken from 01 March through 30 November. New temperature probes have been ordered and will be installed upon delivery.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 16 March.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Sea lion barriers were installed on all gates as of 12 March.

    - SLEDs were installed on the following dates.  Both entrances of PH1CC: 31 January.   Cascades Island: 11 February.  PH2 south monolith entrances: 26 February.  PH2 north monolith entrances: 13 March.  B-branch entrance: 18 March.

    - The WA Shore ladder from the Cascades Island picket leads to WA shore exit was open on 12 February.  The lower WA Shore ladder from the south monolith entrances to the UMT was opened on 26 February. The north monolith entrances were opened on 13 March.

    - The AFF was watered up on 19 March and remains in bypass mode.

  - Juvenile Fishways.

    - Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - The JMF was watered up on 12 March and switched to sample mode at 0900.  The hydro-cannon came online on 07 March.

    - The DSM2 was brought online at 1440 on 06 March.  At 1605 on 07 March the DSM2 was taken offline due to issues that arose from the WA shore LPS construction.  It was brought back online at 2005 on 09 March.

    - The B2CC was opened at 0001 on 01 March.  It was closed at 1336 on 13 March and reopened at 1220 on 22 March upon reaching the kelt trigger.

    - The B2CC avian array was reinstalled on 20 March.

    - The spillway avian array was reinstalled on 19 March.

    - The PH2 avian array was reinstalled on 18 March.

  - Lamprey Fishways.  All LPSs remain dewatered.

- CONSTRUCTION

  - Cascades Island LPS Continuation. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.

  - Gantry 7 Rehabilitation. The Powerhouse 2 intake gantry crane went out of service on 01 August for rehabilitation. It returned to service on 2 March.

  - WA Shore LPS/LFS Installation. Dive work is complete. Construction continues.

  - AFF.  Modifications and improvements began on 19 February and were completed on 15 March.  The AFF was watered up on 19 March and remains in bypass mode for testing.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent 28 February updates the winter spill priority list and wintertime spill caps.

  - A teletype sent on 20 February modifies the previous teletype for chum spawning, in order to gradually reduce the BON tailwater elevation for chum redd protection in response to the continued reduction in the water supply forecast.  The four hard constraints are as follows.  #1: Effective 1000 on 20 February, maintain a minimum tailwater elevation of 13.0’.  #2: Effective 1000 on 22 February, maintain a minimum tailwater of 12.5’.  #3: Effective 1000 on 24 February, maintain a minimum tailwater of 12.0’.  #4: Effective 1000 on 26 February, maintain a tailwater minimum of 11.8’.  For all four hard constraints, for one hour each day during non-dive hours (e.g. between 2000 and 2400) rewet exposed redds by achieving a tailwater elevation of 13.5’ when both powerhouses are in service in order to maintain a tailwater of 13.5’ for 1 hour in every 24 hour period.

  - A teletype sent on 28 February states that once the DSM2 is operating, the project has switched to PH2 priority, and screens are installed, the B2CC will be closed.  The B2CC will be reopened within one hour of notification from project fisheries that the kelt trigger has been reached.  The trigger coordinated for this year is 2 kelts per day for 2 consecutive days as counted at the JMF.  JMF personnel will notify project fisheries when the trigger has been reached.  This special operation is required due to ongoing construction at PH2, delaying the start of the DSM2 and screen installation.

  - A teletype sent on 01 March updates a previous teletype (see teletype 7.3) regarding the operations of the B2CC.  This update states that the B2CC may be closed temporarily to ensure safe conditions of a floating work platform as it exits the PH2 tailrace and for reattachment of PH2 avian float lines below the B2CC.  This closure will be minimized to the extent possible.

JERRY CARROLL

Operations Project Manager