### CENWP-OD-B

MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 13 of 2013, which covers the period from 24 March through 30 March 2013.

- OPERATION SUMMARY

  - Daily average river flows ranged from 135.9 to 179.1 kcfs.  Daily average project tailwater ranged from 12.5’ to 15.0’ msl.  Daily average powerhouse forebay elevation ranged from 73.9’ to 74.6’ msl.  Daily average water temperatures ranged from 44ºF to 46ºF. Secchi disk measurements ranged from 5.0’ to 7.0’ or greater throughout the week.

  - Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Daily average spill ranged from 1.2 to 1.3 kcfs.

  - Unit Operation.  Powerhouse Two became the priority powerhouse on 14 March.

    - Unit 0 went out of service at 0757 on 18 March for pipe repairs.  It returned to service at 1252 on 26 March.

    - Unit 4 was out of service from 0001 on 25 March until 1027 on 28 March for annual maintenance.

    - Unit 11 went out of service at 0900 on 09 September 2012 for high thrust bearing temperatures.

    - Unit 14 went out of service at 0000 on 04 March for TRD testing.  It returned to service at 1014 on 28 March.

  - Fish Units.  Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.

  - Gatewell drawdowns are measured on Tuesdays and more frequently as needed.

- ACTIVITIES

  - Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.

    - FV1-1 was closed for cleaning from 0701 to 0707 on 25 March and from 0655 to 0703 on 28 March.

  - STS/VBS inspections.

    - Table 3.  STS and VBS inspection.

| #### Unit | #### Date | #### Run Hours | #### Comments |
| --- | --- | --- | --- |
| 11 | N/A | N/A | Unit 11 OOS |
| 14 | 03/26 | N/A | STS installed |
| 12,18 | 03/12 | N/A | STS installed |
| 13,15 | 03/13 | N/A | STS installed |
| 16,17 | 03/14 | N/A | STS installed |

  - Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.

- RESEARCH

  - CRITFC – Sea Lion Hazing. Hazing from boats commenced on 04 March.

  - Fisheries Field Unit – Sea Lion Observations.  Observations commenced on 04 January and will continue until the end of May.  Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2013/sea_lion_hazing2013.html

  - PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring began on 12 March.  Fallbacks recorded crossing the separator bars this week: four steelhead and one unknown salmonid. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm.

  - USDA - Sea Lion Hazing. Land based hazing began on 01 March and will continue until the end of May.

  - Normandeau – Fish Counting.  Video counting continues. Visual counting is scheduled to begin on 01 April. Normandeau assumed fish counting responsibilities from WDFW as of 01 March.

- FISHWAYS

  - Project biologists inspected 24 March through 30 March.

  - Fish Passage Plan Observations.

    - Table 4.  FPP Out of Criteria items.

| Date | Location | FPP Items | Cause | Response |
| --- | --- | --- | --- | --- |
| BRADFORD ISLAND (BI) |  |  |  |  |
| 3/24,27-28 | PH1CC | S Entrance Diff < 1.0’ | N/A | None |
| 3/27-28,30 | WG-64 | Submergence > 8.4’ | N/A | None |
| All week | A-branch | FG3-3 half open | Mechanically bound | TR in place |
| All week | A-branch | Water level > 1.1’ | N/A | None |
| CASCADES ISLAND (CI) – Nothing to Report. |  |  |  |  |
| WASHINGTON SHORE (WS) |  |  |  |  |
| 3/29 | DSM2 | Channel elev < 64.2’ | N/A | None |

    - PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.

    - A new mechanical velocity meter was installed in the PH2CC on 26 March.

  - Fishway Temperatures. Temperatures are normally taken from 01 March through 30 November. New temperature probes have been ordered and will be installed upon delivery.

  - Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.

  - Adult Fishways.

    - Calibration was checked on 28 March.

    - 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Sea lion barriers are in place on all installed FOGs.

    - All SLEDs are installed.

    - All ladders remain in service.

    - The AFF was watered up on 19 March and remains in bypass mode.

  - Juvenile Fishways.

    - Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.

    - DSM2 remains in service.

    - The hydrocannon remains in service.

    - The B2CC was opened at 0001 on 01 March.  It was closed at 1336 on 13 March and reopened at 1220 on 22 March upon reaching the kelt trigger.

    - The B2CC avian array was reinstalled on 20 March.

    - The spillway avian array was reinstalled on 19 March.

    - The PH2 avian array was reinstalled on 18 March.

  - Lamprey Fishways.  All LPSs remain dewatered.

- CONSTRUCTION

  - Cascades Island LPS Continuation. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.

  - WA Shore LPS/LFS Installation. Dive work is complete. Construction continues.

  - AFF.  Modifications and improvements were completed on 15 March.  The AFF was watered up on 19 March and remains in bypass mode for testing.

- HAZMAT, SPILLS AND CLEANUP. Nothing to report.

- TELETYPES CURRENTLY IN EFFECT

  - A teletype sent 28 February updates the winter spill priority list and wintertime spill caps.

  - A teletype sent on 20 February modifies the previous teletype for chum spawning, in order to gradually reduce the BON tailwater elevation for chum redd protection in response to the continued reduction in the water supply forecast.  The four hard constraints are as follows.  #1: Effective 1000 on 20 February, maintain a minimum tailwater elevation of 13.0’.  #2: Effective 1000 on 22 February, maintain a minimum tailwater of 12.5’.  #3: Effective 1000 on 24 February, maintain a minimum tailwater of 12.0’.  #4: Effective 1000 on 26 February, maintain a tailwater minimum of 11.8’.  For all four hard constraints, for one hour each day during non-dive hours (e.g. between 2000 and 2400) rewet exposed redds by achieving a tailwater elevation of 13.5’ when both powerhouses are in service in order to maintain a tailwater of 13.5’ for 1 hour in every 24 hour period.

  - A teletype sent on 28 March updates the spill priority list and spill caps for 1 April through 2359 on 15 May, 2013.

  - A teletype sent on 27 March states the most recent forebay restrictions and turbine flow testing criteria for velocity data collection at PH2.  This data is being collected to evaluate the turbine reduction device (TRD).  On 01 and 02 April from 0700 to 1900, maintain the forebay between 71.5’ – 73.0’ as a hard constraint and 71.5’ – 72.5’ as a soft constraint, with a minimum target turbine flow of 17 kcfs.  These flows will be tested in gatewell slot 14A on 01 April and 15A on 02 April.  An updated daily schedule and progress report will be provided to the control room by the velocity test contractor.  A modified unit priority for this testing is provided in Appendix A of the FPP.  Additionally, as a soft constrain operate the unit adjacent to the test unit at the same flow rate.  Outside the test dates and times, operate the forebay at the normal elevation of 71.5’ – 76.5’.

JERRY CARROLL

Operations Project Manager