### CENWP-OP-B                                                                                                 9 November 1999

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 44 of 1999, which covers the period 31 October through 6 November 1999.

- OPERATION SUMMARY.

1.1. Daily Average River flow ranged from 124.3 to 133.0 kcfs.  Daily average tailwater elevation for the project ranged from 11.2’ to 12.3’ msl.  River temperature ranged from 55 to 56° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island and B-branch fish ladders respectively, remain open 4”.

- First powerhouse unit 4 remained out of service for rehab, unit 5 for post release pipe installation preparation, unit 6 for turbine release pipe installation, unit 10 for stator replacement, and unit 16 for stator re-wedge.  Unit 2 was out of service from 0530-1630 on 2 November for semi-annual maintenance.  Unit 9 was out of service from 0612 on 3 November through 1042 on 6 November to replace the current transformer wiring.  Unit 6 was watered-up on 4 November and will begin post release pipe installation commissioning testing on 8 November.

1.3. The first powerhouse ice and trash sluiceway remained open and the second powerhouse sluiceway remained closed this reporting week.

1.4. Second powerhouse fish units remained in service this reporting week.  The units are placed on standby from approximately 0000-0300 each night to float trash if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1. STS Inspections.  There were no STS inspections this reporting week.

2.2. VBS Inspections.  There were no VBS inspections this reporting week.

2.3. AWS Closures.  FV1-1 was closed for cleaning from 1458-1519 and FV3-9 from 1525-1534 on 4 November.

2.4. Fish Salvage.  The Adult Fish Collection and Monitoring Facility (AFC&MF) was dewatered on 2 November this reporting week for winter maintenance.  Project biologists recovered approximately 15 adult shad, 5 peamouth chubs, 4 chinook jacks, 2 lamprey, and one sculpin.  The PH1 DSM was turned south on 4 November this reporting week due to a broken trash sweep.  Project biologists recovered 10 juvenile shad, 2 hatchery steelhead, one wild adult steelhead, and one juvenile steelhead.

- RESEARCH.

3.1. WDFW Fish Counting.  WDFW began 24-h/d video counting at Bradford Island and Washington shore on 1 November.

- FISHWAYS.  Project biologists inspected on 1, 4, and 5 November this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP VIOLATION | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week | PH1 collection channel gate 21. |  |  | See 6.2 below. |
| All Week | A-branch diffuser FG3-4. |  |  | See 6.3 below. |
| All Week | A-branch diffuser FG3-6. |  |  | See 6.3 below. |
| All Week | PH1 and PH2 velocity meters. |  |  | See 6.4 below. |
| All Week | PH1 chain gate 10C. | Chain gate was closed and should be open on sill. | Maintenance crews were cleaning in front of trashracks and chain gates were open accordingly. | Cleaning was completed on 4 November and chain gates were set back to FPP criteria. |
| 11/1 | Weir 37, Washington shore ladder. | Staff gauge registered >1.0’+/-. 1’. | Bleed-off valve was out of calibration. | Valve was calibrated on 2 November of the subsequent reporting week. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration at PH1, B-branch and Cascades Island on 4 November this reporting week.  All sensors were within the 0.3’ allowed variance with the following exceptions.  PH1 WG-1 registered 1.5’ lower than by direct measurement and PH1 WG-65 registered 1.1’ lower than by direct measurement, and Cascades Island tailwater transducer registered 0.6’ higher than by direct measurement.

4.3. Avian Predation Status.  There are currently three avian lines down at PH1.

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- CONSTRUCTION UPDATE.

5.1. The second powerhouse DSM channel and flume continue to operate in automatic control.  Orifice 11A-S remained closed in manual control due to the poor flow characteristics associated with the orifice.  The add-in water supply remains closed and the outfall remains in low outfall bypass mode.  Screen cleaners continue to operate and are being cycled approximately every 3 hours.

- OTHER NOTABLE ITEMS.

6.1. The PH1 DSM was turned south on 4 November due to a broken trash sweep.  The channel is scheduled to return to service on 10 November.

6.2. PH1 collection channel orifice gate 21 has lost a section of its gate.  Consequently, a portion of the gate remains open and should be closed.  The sluice gate should be open according to current tailwater conditions, however the gate has been closed due to the orifice gate opening.  The gate will not be repaired until the winter maintenance season.

6.3. A-branch diffusers FG3-4 and FG3-6 remain out of service due to the diffuser gates becoming separated from the shafts.  They will not be repaired until the winter maintenance season.

6.4. PH1 and PH2 collection channel electronic velocity meters continue to work intermittently.  Meters should register 1.5-4.0 fps however; frequently during fishway inspections the meters are not working or register <1.5 fps.  However, the PH1 south collection channel mechanical meter continues to operate.

DEBORAH L. CHENOWETH

# Operations Project Manager